3GPP TSG RAN WG4 Meeting #115	R4-2507018

Saint Julian's, MT, 19 May 2025 – 23 May 2025

Agenda Item:	7.29.3.2

Source: 	Ericsson

Title:	Discussion on CSI-RS based L1 measurement requirements

Document for:	Discussion

# Introduction

In this contribution we discuss CSI-RS based L1 measurement requirements.

# Discussion

## CSI-RS based L1 measurement requirements

### Intra-frequency and Inter-frequency definition

Intra-frequency and inter-frequency measurement is pending for many meetings and the following candidate options are shortlisted after last meeting.

- Agreement: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

We think both options work from defining RAN4 requirements. However, we have a slight preference for Option 2, as it eliminates the need to further define requirements for intra-frequency measurements with gaps and requirements for inter-frequency measurements without gaps. Option 1 is driven by the definition of CSI-RS-based L3 measurements, and as some companies have noted, we are uncertain whether it has been implemented in the field. Therefore, we think it should not be a limiting factor.

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

### Measurement procedure

The following is concluded in RAN4#114bis meeting.

- Whether step 2 can be skipped for neighboring cell or not based on new UE capability reporting.

- Further discuss whether step 2 can be skipped for serving cell or not.

- For a UE not supporting the capability, Step 2 is necessary.

  - The CSI-RS resources in Step 3 should be at least Type-D QCL’ed with the SSB in Step 2.

  - UE Rx beam sweeping factor is not included in the CSI-RS based L1-RSRP measurement period assuming the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

- For a UE supporting the capability, Step 2 can be skipped, and if the Step 2 is skipped:

  - The CSI-RS resources in Step 3 should be Type-D QCL’ed with the SSB in Step 1.

  - The CSI-RS resources should be periodic or semi-persistent, and the CSI-RS resources are in the CSI-RS resource set with ‘repetition = off.’

  - UE Rx beam sweeping factor needs to be included in the CSI-RS based L1-RSRP measurement period, and further discuss the detail of beam sweeping factor.

In last meeting RX beam sweeping based on CSI-RS is agreed as an optional UE capability. We call it as csi-rsBasedRxbeamSweeping for rest of the discussion.

One of the open issues is the above agreed capability applies to when SCell is configured as a candidate cell. In legacy, for serving cell measurements, the UE would perform SSB-based Rx beam sweeping for CSI-RS-based L1-RSRP measurements. However, when the same cell is configured as a candidate cell, the UE is expected to use CSI-RS-based Rx beam sweeping for the candidate cell measurements. This dual approach could introduce ambiguity and confusion in handling measurements for SCells. To address this potential issue and maintain clarity, we are supportive of excluding this requirement for SCells.

- csi-rsBasedRxbeamSweeping is not applied to serving cells.

Another issue in the aforementioned agreement pertains to determining the beam sweeping factor for csi-rsBasedRxbeamSweeping. The rationale for setting the legacy beam sweeping factor at 8 for SSB-based RX beam sweeping was based on the assumption that up to 128 beams could be transmitted by the network, requiring the UE to perform sweeping over a larger area to capture finer beams. However, in practice, the deployment does not involve as many transmit beams as initially anticipated, and not many fine beams are utilized by the gNB. Therefore, we believe that the Rx beam sweeping factor can be reduced to align more closely with actual network configurations and operational practices.

- For a UE supporting CSI-RS based RX beam sweeping, beam sweeping factor is equal to 4.

### Restriction on CSI-RS resource configuration

In the last meeting, restriction on CSI-RS resource configuration was discussed and following WF was agreed.

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration. - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction: - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

CSI-RS based L3-RSRP uses the CSI-RS type, CSI-RS-Resource-Mobility. The periodicity and offset of CSI-RS-Resource-Mobility are {4ms, 5ms, 10ms, 20ms, 40ms}, whereas for CSI-RS for L1-RSRP, they are {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots.

The maximum periodicity for L3-RSRP is 40 ms, during which one intra-frequency layer can have two separate 5 ms windows where the CSI-RS is allowed. Given that the CSI-RS for L1-RSRP can have a periodicity of up to 640 slots, we would like to clarify the configuration restriction. Therefore, we propose that all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms

### Requirements applicability

Following options were discussed in the last meeting.

Candidate options:

  - Option 1: Qualcomm

RAN4 requirements apply only when all CSI-RS resources from candidate cells on the same layer are configured with the same SCS and CP type.

  - Option 2: MediaTek

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR1, legacy measurement period, measurement restriction and scheduling restriction requirements are all applicable.

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, define measurement restriction when two CSI-RS resources from difference cells overlap in time domain.

For CSI-RS based L1-RSRP measurement on neighbor cell (without gap when RTD<CP) in FR2, legacy measurement period and scheduling restriction requirements are both applicable.

  - Option 2: vivo

RRM requirements for CSI-RS based LTM are only applicable if the CSI-RS of candidate cell is configured with QCL-D to one of the SSBs for L1 measurements.

Before discussing the applicability of the requirements, it is essential to reach a consensus on the definition of intra-frequency. With regard to the options presented above, most of the aspects are already covered within the defintion of the intra-frequency. Therefore, we propose revisiting this discussion once there is an agreement on the intra-frequency definition.

- Revisit the requirements applicability once there is an agreement on the intra-frequency definition

### Early CSI acquisition on a candidate cell

Following options were discussed in the last meeting.

- Option 1: Qualcomm

  - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.

  - For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded:

    - Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc.

- Option 2: Nokia

  - Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

For the baseline early CSI acquisition, we believe that the UE should be prepared to send the CSI within the LTM cell switch delay or on the first occasion of the CSI reporting request. Although the RRM specification typically does not include detailed requirements for CSI, there are certain clauses that specify the timing for UE to send CSI reports. We have referenced the relevant sections of TS 38.133 where some aspects of CSI reporting are mentioned.

TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource. - TCSI_processing is the UE processing time for CSI reporting. - TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

To enhance clarity and ensure compliance, it would be beneficial to add a sentence in the RRM specification stating that the UE should be ready to send a valid CSI report immediately upon the LTM cell switch delay completion.

One example of how we can define the early CSI reporting delay is:

TCSI_reporting_LTM is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

- For the baseline early CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

### Others

The following WF is agreed upon in the last meeting. We provide our understanding on the same.

  - After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

  - If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

  - Discuss how many measurements CSI-RS measurements UE is capable of performing during each measurement period.

In FR2, we prioritized SSB-based measurements due to the RX beam sweeping factor of 8 for SSB-based L1-RSRP measurements. However, for CSI-RS, the factor N is 1. Furthermore, we agreed to define CSI-RS-based L1-RSRP measurements without a measurement gap. We believe the delay in acquiring CSI-RS is minimal, and the UE should be able to obtain these measurements without significant difficulty. Hence, we do not see the need for such prioritization.

Regarding the second bullet point in the proposal, our understanding is that the UE will not accept a configuration exceeding its reported capabilities, which would lead to an RRC configuration failure. Therefore, we do not perceive this as an issue, and we believe there is no need to specify it further.

Regarding the third bullet point in the proposal, we believe RAN4 should define the number of intra-frequency and inter-frequency layers that the UE must be capable of measuring.

- RAN4 should define the number of intra-frequency and inter-frequency layers that the UE must be capable of measuring for the CSI-RS based L1-RSRP on the neighbouring cells.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:
- the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

- the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

- the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP.

- at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

- csi-rsBasedRxbeamSweeping is not applied to serving cells.

- For a UE supporting CSI-RS based RX beam sweeping, beam sweeping factor is equal to 4.

- In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

  - all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms.

- Revisit the requirements applicability once there is an agreement on the intra-frequency definition

- For the baseline CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), DLTM).

- RAN4 should define the number of intra-frequency and inter-frequency layers that the UE must be capable of measuring for the CSI-RS based L1-RSRP on the neighbouring cells.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4