3GPP TSG-RAN WG4 Meeting #115	R4-2507026

# Malta , MT, 19 - 23 May, 2025

Title: 	Discussion on less than 5MHz for NR NTN demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.10.4.6

Document for:	Discussion

Background

During RAN4#114bis meeting, WF [1] for NR IoT NTN less than 5MHz demod was approved. In this contribution, we share our view on less than 5MHz for NR NTN.

Discussion

UE demodulation requirements

PDCCH

#### Issue 2-2: PDCCH – Scope of the UE demodulation requirements Way forward: - Option 1: No new PDCCH requirements - Option 2: Discuss the feasibility of introducing PDCCH requirements

In Rel-18 NR FR1 less than 5MHz WI, the punctured PDCCH cases are defined for TN UE in TS 38.101-4. However, only additional PDSCH requirements are defined for NTN UE in TS 38.101-5 with assumption that NTN UE also need to pass TN test cases with the following applicability rule, i.e. the NTN UE supporting less than 5MHz should also need to pass the punctured PDCCH cases defined in TS 38.101-4. We think such requirements are enough to verify whether an NTN UE support less than 5MHz. Therefore, we prefer not to define new punctured PDCCH requirements for NTN.

Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1

Do not define new punctured PDCCH requirements for NTN.

SAN demodulation requirements

PUSCH

Issue 1-1: PUSCH – Scope of the SAN demodulation requirements Way forward: - Option 1: No new PUSCH requirements - Option 2: FFS Inputs from NTN operators can be considered

In Rel-18 NTN, only one typical and small channel bandwidth per SCS, 5MHz/15kHz SCS and 10MHz/30kHz SCS, is selected for demodulation performance requirements definition in TS 38.108. All SAN only need to perform the demodulation test with one small channel bandwidth. It is RAN4 assumption that there is no SAN that only support the channel bandwidth smaller than the defined value. We think such test is enough to ensure NTN SAN performance. We prefer not to define new PUSCH requirements by assuming that there is no SAN that only support 3MHz channel bandwidth.

Do not define new PUSCH requirements by assuming that there is no SAN that only support 3MHz channel bandwidth.

PUCCH

Issue 1-2: PUCCH – Scope of the SAN demodulation requirements Way forward: - Option 1: FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition - Option 2: Define PUCCH format 2 cases for SAN Inputs from NTN operators can be considered

As per the simulation assumptions for PUCCH [1], we provide the initial simulation results for different PUCCH formats with channel bandwidth 3MHz and 5MHz.

PUCCH format 0

Table 2.2.2-1 Ideal simulation results for PUCCH format 0

| Case Number | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- |
|  |  | 5MHz | 3MHz |
| 1 | 1x1 | 6.92 | 8.29 |
| 2 | 1x2 | 0.70 | 1.63 |

PUCCH format 1

Table 2.2.2-2 Ideal simulation results for PUCCH format 1

| Case Number | SCS (kHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5MHz |  | 3MHz |  |
|  |  |  | ACK missed detection | NACK to ACK | ACK missed detection | NACK to ACK |
| 1 | 15 | 1x1 | -1.19 | -0.28 | -0.32 | 0.77 |
| 2 | 15 | 1x2 | -7.19 | -5.77 | -6.44 | -5.25 |

PUCCH format 2

Table 2.2.2-3 Ideal simulation results for PUCCH format 2 for UCI BLER

| Case Number | SCS (kHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5MHz | 3MHz |
| 1 | 15 | 1x1 | 4.27 | 6.99 |
| 2 | 15 | 1x2 | -1.85 | -0.13 |

PUCCH format 3

Table 2.2.2-4 Ideal simulation results for PUCCH format 3

| Case Number | SCS (kHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5MHz |  | 3MHz |  |
|  |  |  | DMRS1+0 | DMRS1+1 | DMRS1+0 | DMRS1+1 |
| 1 | 15 | 1x1 | 4.27 | 4.11 | 5.31 | 4.89 |
| 2 | 15 | 1x2 | -2.11 | -2.45 | -1.35 | -1.74 |

PUCCH format 4

Table 2.2.2-5 Ideal simulation results for PUCCH format 4

| Case Number | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5MHz |  | 3MHz |  |
|  |  | DMRS1+0 | DMRS1+1 | DMRS1+0 | DMRS1+1 |
| 1 | 1x1 | 7.27 | 7.19 | 8.23 | 7.99 |
| 2 | 1x2 | 0.57 | 0.39 | 1.26 | 1.04 |

PUCCH format 1 (multi-slot)

Table 2.2.2-6 Ideal simulation results for PUCCH format 1 (multi-slot)

| Case Number | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5MHz |  | 3MHz |  |
|  |  | ACK missed detection | NACK to ACK | ACK missed detection | NACK to ACK |
| 1 | 1x1 | -4.16 | -2.27 | -4.05 | -2.11 |
| 2 | 1x2 | -10.26 | -9.08 | -10.15 | -9.18 |

Based on our simulation results, the target SNR point is almost less than 1dB degradation for 3MHz cases comparing to 5MHz cases for all PUCCH formats except PUCCH format 2 that has around 2dB degradation. Also in Rel-18 NR FR1 less than 5MHz WI, only PUCCH format 2 cases with applicability rules are defined for TN UE in TS 38.104. For NTN SAN, same methodology can be used: i.e. only define PUCCH format 2 case with the same test applicability rules to reduce the test efforts:

For a SAN declaring to support less than 5MHz (see D.XX in table 4.6-1) a UCI BLER performance test with PUCCH Format 2 shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 can be skipped if applicable.

Only define PUCCH format 2 cases for NR SAN.

Introduce the similar test applicability rule in TS 38.181 as for NR TN less than 5MHz:

- For a SAN declaring to support less than 5MHz (see D.XX in table 4.6-1) a UCI BLER performance test with PUCCH Format 2 shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 can be skipped if applicable.

PRACH

Issue 1-3: PRACH – Scope of the SAN demodulation requirements Agreements: - No new PRACH requirements - Discuss further whether to introduce the test applicability rule

In current TS 38.141, the test applicability rule for PRACH is defined as following:

Unless otherwise stated, for BS supporting less than 5MHz carrier bandwidth (see D.14 in table 4.6-1), only test requirements relating to RACH preamble formats with 15kHz SCS with sequence length LRA=139, and PRACH formats with 1.25kHz SCS with sequence length LRA=839 shall apply.

Since there are no PRACH formats with LRA=1151 and LRA=571 for 15kHz SCS and 30kHz SCS respectively which are only applicable for channel bandwidth not less than 20MHz in TS 38.108, it is not necessary to introduce such test applicability rule for NR NTN.

Not introduce the test applicability rule for NR NTN PRACH.

Proposals

In this contribution, we share our view on less than 5MHz for NR NTN. Our observations and proposals are:

- Do not define new punctured PDCCH requirements for NTN.

Not define new PUSCH requirements by assuming that there is no SAN that only support 3MHz channel bandwidth.

Only define PUCCH format 2 case for NR SAN.

| Parameter | Values |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna | 1x1, 1x2 |
| Test metric | BLER |

Introduce the following test applicability rule in TS 38.181.

-	For a SAN declaring to support less than 5MHz (see D.XX in table 4.6-1) a UCI BLER performance test with PUCCH Format 2 shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 can be skipped if applicable.

Not introduce the test applicability rule for NR NTN PRACH.

Reference

R4-2504753, WF for NR IoT NTN less than 5MHz demod, RAN4#114bis, Ericsson