3GPP TSG-RAN WG4 Meeting #115	R4-2507028

# Malta, MT, 19 - 23 May, 2025

Title: 	Discussion on NR NTN Phase 3 demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.31.7

Document for:	Discussion

Background

During RAN#106 meeting, revised WID [1] for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved. In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements.

Discussion

Based on the WID, the mainly RAN4 demodulation related enhancement is listed as following:

Downlink coverage enhancements for both GSO and NGSO in FR1-NTN or FR2-NTN

Based on RAN1 agreements, PDCCH repetition for CSS (except for Type-3) and PDSCH repetition for SIB1/Msg4 are supported. However, the enhancements are not target on RRC connected mode, it is difficult to perform the test. So we propose to not define requirements for above enhancements.

Do not define requirements for downlink coverage enhancements.

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN

Based on RAN1 agreements, inter-slot OCC length 2 and inter-slot OCC length 4 is supported for DFT-s-OFDM PUSCH repetition Type A is supported. Considering that there is baseband processing impact, we propose to define one requirement with inter-slot OCC length 2 to verify the performance of this feature.

Define one performance requirement with inter-slot OCC length 2.

Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands

Based on existing RAN agreements, NTN UE also need to pass TN test cases by the following applicability rule:

Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1

For RedCap, the only thing that need to consider is to define additional PDSCH requirements in TS 38.101-5 for RedCap UEs with 1Rx, because there are only 2Rx requirements defined in current TS 38.101-5. A similar set of requirements should be defined by reusing the existing PDSCH cases configuration but changing the number of Rx from 2 to 1. In addition, the new reference channel should be defined for HD-FDD (e)RedCap requirements but with the same requirements as for FDD (e)RedCap as did for TN (e)RedCap.

For eRedCap, new performance requirements for reduced baseband need to be introduced. Whether the existing 2Rx NTN performance requirements can be reused for eRedCap without baseband reduced, further checking is needed with consideration the supported max data rate.

Table 2-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1 1x2, ULA Low | 70 | 0.3 |
| 1-2 | TBD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x1 1x2, ULA Low | 70 | 7.6 |
| 1-3 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x1 1x2, ULA Low | 70 | -0.4 |
| 1-4 | TBD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x1 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

- Define a set of requirements for (e)RedCap with 1Rx with similar test configurations as the existing PDSCH cases

- Define new reference channel for HD-FDD (e)RedCap with same performance requirements as for FDD

- Define new performance requirements for eRedCap with reduced baseband

- Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

Proposals

In this contribution, we share our view on NR NTN Phase 3 demodulation performance requirements. Our observations and proposals are:

- Do not define requirements for downlink coverage enhancements.

Define one performance requirement with inter-slot OCC length 2.

- Define a set of requirements for (e)RedCap with 1Rx with similar test configurations as the existing PDSCH cases

- Define new reference channel for HD-FDD (e)RedCap with same performance requirements as for FDD

- Define new performance requirements for eRedCap with reduced baseband

- Check if the existing PDSCH requirements for 2Rx can be reused for eRedCap without reduced baseband

Reference

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, Thales, CATT