3GPP TSG-RAN WG4 Meeting #115	R4-2507031

Malta, MT, 19th-23rd May, 2025

Title: 	Overviews on UE and BS performance requirements for 7MHz bandwidth

Source: 	Huawei, HiSilicon

Agenda item:	7.8.6

Document for:	Discussion

Background

According to the TU assignment, it’s time to start discussing demodulation performance requirements for 7MHz bandwidth in this meeting, this contribution provides our related overviews.

- Discussions

UE performance

Only 10MHz bandwidth is introduced for FDD PDSCH single carrier requirements currently, it’s common understanding that 10MHz could be a common bandwidth which is high likely supported by most UEs. From baseband processing point view, there is no difference between 7MHz bandwidth and 10MHz bandwidth. Therefore, we propose not to define PDSCH requirements for single carrier.

Proposal 1: RAN4 not to define single carrier PDSCH requirements for 7MHz

According to [2], 7MHz bandwidth has been added to n5 and n26 band, and according to NR CA configurations and bandwidth combinations sets defined in Table 5.5A.3.1-1d, take CA_n5A_n7A CA configuration as an example, all channel bandwidth defined in n5 could be configured for CC(s), which means it’s not excluded that 7MHz carrier could be used for CA operation.

Observation 1: From RF spec, it’s not excluded that 7MHz bandwidth could be used for CC(s) for CA scenario

Capability rules specify that CA configuration with supported maximum aggregation bandwidth is prioritized to be selected. Considering minimum 5MHz bandwidth has been introduced for CA requirements, 7MHz could be likely to be selected, so it is necessary to introduce CA requirements with 7MHz bandwidth. Considering n5 and n26 are target band for 7MHz, it’s proposed to only introduce FDD requirements for 7MHz.

Proposal 2: RAN4 to introduce CA requirements for 7MHz bandwidth.

We propose to reuse R15 CA test setup:

Duplex mode: Only FDD

Bandwidth/SCS: 7MHz/15kHz

Number of RB: 35PRBs

Rank: Rank2

MCS:16QAM,0.48

Propagation condition: TDLA30-10

Antenna configuration: 2T2R, ULA Low

Other parameters: Reuse Table 5.2.1

Proposal 3: RAN4 to use following test setup for CA requirements for 7MHz CC:

Duplex mode: Only FDD

Bandwidth/SCS: 7MHz/15kHz

Number of RB: 35PRBs

Rank: Rank2

MCS: 16QAM,0.48

Propagation conditions: TDLA30-10

Antenna configuration: 2T2R, 2T4R ULA Low

Other parameters: Reuse Table 5.2.1

SDR requirements

5MHz bandwidth has been defined for SDR test. The capability rules for SDR test specifies that the first step is to select one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate. This implies one CA configuration including 7MHz bandwidth could provide largest data rate. Hence it is necessary to introduce SDR requirements for 7MHz bandwidth. Considering the current test setup for SDR test is generally band agnostic, there is no simulation work and spec changes except for PDCCH configuration. Regarding the PDCCH, we propose to configure 2/AL2.

Proposal 4: RAN4 to define SDR requirements for 7MHz bandwidth with no simulation work and spec changes except for configuring 2/AL2 for PDCCH configuration for 7MHz bandwidth.

CSI requirements

For single carrier requirements, similar views as PDSCH single carrier requirements, current CSI tests for FDD are based on 10MHz and there is no baseband processing difference between 7MHz and 10MHz. We don’t see the need to introduce additional requirements for 7MHz.

Proposal 5: RAN4 not to introduce CSI requirements for 7MHz bandwidth for single carrier.

However, for CA requirements, we share similar views as SDR test, 7MHz bandwidth requirement should be introduced. There is no simulation and more CR work except for adding sub-band Size configuration for 7MHz, it’s proposed to set 8.

Proposal 6: RAN4 to define CQI CA requirements for 7MHz bandwidth.

Proposal 7: RAN4 to set subband Size to 8 for 7MHz

BS performance

Only 5MHz, 10MHz and 20MHz among all defined bandwidths are introduced for PUSCH and PUCCH requirements, that implies no big performance and baseband processing difference between different bandwidth. The current applicability rules defined in TS 38.141-1 guarantees that all supported maximum bandwidth is testable. For example, if a BS supports maximum bandwidth is 7MHz, 5MHz requirements is applied and all PRBs corresponding to 5MHz is placed on the centre of 7MHz carrier. Therefore, it’s proposed not to introduce BS requirements for BS performance.

Proposal 8: RAN4 not to introduce BS requirements

Release independent

Release independent is on discussion in RF session, following agreement was made.

Agreement: - Support of 7 MHz channel BW could be Release independent from Rel-15 - FFS on if the maximum initial BWP size shall then be restricted to 5 MHz. - Support of 7MHz is optional in Rel-19 and in the previous releases

According to [1], following action is recommended:

ACTION: RAN4 respectfully requests that RAN2 consider the signalling aspects related to adding the 7 MHz channel bandwidth, and to inform RAN4 if RAN2 sees any reason from a signalling perspective why the 7 MHz channel bandwidth cannot be release independent from Release 15.

From above it can be observed that RAN4 has reached consensus on introduction of release independent but need the confirmation from RAN2. Hence, it’s proposed to follow this discussion that once the release independent is agreed, it should also apply to demodulation requirements.

Proposal 9: RAN4 to follow the discussion on release independent, once it is agreed, it should also apply to demodulation requirements (if introduced).

- Conclusion

In this contribution we provide our overviews on UE and BS performance requirements for 7MHz bandwidth. The proposals are:

Proposal 1: RAN4 not to define single carrier PDSCH requirements for 7MHz

Observation 1: From RF spec, it’s not excluded that 7MHz bandwidth could be used for CC(s) for CA scenario

Proposal 2: RAN4 to introduce CA requirements for 7MHz bandwidth.

Proposal 3: RAN4 to use following test setup for CA requirements for 7MHz CC:

Duplex mode: Only FDD

Bandwidth/SCS: 7MHz/15kHz

Number of RB: 35PRBs

Rank: Rank2

MCS: 16QAM,0.48

Propagation conditions: TDLA30-10

Antenna configuration: 2T2R, 2T4R ULA Low

Other parameters: Reuse Table 5.2.1

Proposal 4: RAN4 to define SDR requirements for 7MHz bandwidth with no simulation work and spec changes except for configuring 2/AL2 for PDCCH configuration for 7MHz bandwidth.

Proposal 5: RAN4 not to introduce CSI requirements for 7MHz bandwidth for single carrier.

RAN4 to define CQI CA requirements for 7MHz bandwidth.

Proposal 7: RAN4 to set subband Size to 8 for 7MHz

Proposal 8: RAN4 not to introduce BS requirements

Proposal 9: RAN4 to follow the discussion on release independent, once it is agreed, it should also apply to demodulation requirements (if introduced).

- Reference

[1]  R4-2503017 LS on Signaling for 7 MHz Channel Bandwidth

[2]  R4-2505136 Introduction of 7 MHz channel bandwidth. Nokia