3GPP TSG-RAN WG4 Meeting #115	R4-2507042

Malta, MT, 19th-23rd May, 2025

Title: 	On demodulation and CSI requirements for R19 MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	7.24.6

Document for:	Discussion

Background

According to the TU assign, it’s time to start the performance part for R19 MIMO evolution in this meeting. This contribution provides our related general views.

- Discussions

There are four main features introduced by RAN1: Enhancement for UE-initiated/event-driven beam management, CSI enhancement, support for 3-antenna-port codebook-based transmission and Enhancement for asymmetric DL sTRP/UL mTRP scenarios. Following table summarizes our views on whether the features have demod and CSI impact.

| Feature | Demod and CSI impact? |
| --- | --- |
| Enhancement for UE-initiated/event-driven beam management | No |
| CSI enhancements | Yes |
| Support for 3-antenna-port codebook-based transmission | No |
| Enhancement for asymmetric DL sTRP/UL mTRP scenarios | No |

Following proposal is made:

Proposal 1: Not to introduce requirements for Enhancement for UE-initiated/even-driven beam management, Support for 3-antenna-port codebook-based transmission and Enhancement for asymmetric DL sTRP/UL mTRP scenarios

We will discuss features related to CSI enhancement one by one in next sub-clauses.

Support up to 128 ports

Four objectives are included in this feature, Type 1 SP codebook refinement, Type II codebook refinement, extension of CRI(s)-based CSI reporting and SRS port grouping. There are two schemes introduced for Type 1 SP codebook refinement, following is agreements from RAN1:

Agreement For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following: - Scheme-A (based on Scheme1 in RAN1#116 agreement): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook mode-1 (L=1) where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and for rank-3/4, follow legacy mechanisms for <16 ports - Scheme-B (based on Scheme2 in RAN1#116 agreement): Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and - W1 structure: - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates - For 1<RI≤4, L=1 SD basis vector is independently selected for different layers - The SD basis selection indication includes layer-common (q1,q2) and log2N1N2 bits for each layer - Note: This implies that each of the SD basis vectors is selected from a group of N1N2 orthogonal basis vectors W2 structure: Layer-specific inter-polarization co-phasing with the alphabet {+1, +j, -1, -j} Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes: - The same O1=O2 value(s) as RI=1-4 are supported - Scheme-A (based on Scheme3 described in RAN1#116bis): - W1 structure: - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). - The v layers are mapped to the selected SD basis vectors following legacy Rel-15 Type-I for RI=5-8. - W2 structure: - Following legacy Rel-15 Type-I RI=5-8 - Scheme-B (based on Scheme2 described in RAN1#116bis): - W1 structure: - Independent selection of different ceil(v/2) SD basis vectors for RI = v, where each SD basis vector is applied to two respective layers following legacy Rel-15 Type-I for RI=5-8, except that, if v is odd, the last SD basis vector is applied to the orphan layer. - FFS: mapping between the orphan layer and its selected SD basis vector and, if needed, UE reporting of the selection - FFS: support of 4 selected SD basis vectors for RI=5-6 - The SD basis vectors are freely selected from a group of N1N2 orthogonal SD DFT basis vectors via combinatorial indication, as well as a layer-common (q1,q2) - W2 structure: - For the orphan layer, the inter-polarization co-phasing is selected from {1, j, -1, -j} - For two layers sharing a same SD basis vector, the inter-polarization co-phasing between two layers is selected from the following pairs {(1, -1), (j, -j)} to achieve inter-layer orthogonality. - Only Scheme-A (RI=1-4+RI=5-8) and Scheme-B (RI=1-4+RI=5-8) are supported in Rel-19

Scheme-A

For scheme-A, for rank 1~4, compared to the legacy SP type 1 codebook, the main refinement is adding new configurations for (N1,N2) value for L=1, the codebook design is reused from Rel-15 Type I codebook. Hence, except for larger number of CSI-RS ports, there is no fundamental changes for codebook.

Observation 1: For scheme-A and rank 1~4, there is no fundamental changes for codebook except for large number of CSI-RS ports.

For rank 5-8, the main change is that for R15 Type1 SP codebook, the SD basis interval is fixed, which means the beam selection for all layers should be performed jointly and for R19 codebook, the subsequent 2 to 4 beams could be selected freely. Take 5 layers as an example, the indexes for three SD basis are (i1,1, i1,2), (i1,1+O2, i1,2), (i1,1+ O1, i1,2+O2) respectively. UE should perform PMI selection for three layers jointly with traversing i1,1 and i1,2

Table 5.2.2.2.1-9: Codebook for 5-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

New codebook allows UE to perform PMI selection freely for 3 SD basis with the restrictions that  they are orthogonal in at least one dimension (horizontal or vertical).

Observation 2: For scheme-A and rank 5~8, there is fundamental changes for codebook.

Scheme-B

For scheme-B, the legacy codebook specifies the restrictions on the index interval of SD basis (indicated by i1,3 or be fixed), which means the SD basis selection should be performed jointly. Take legacy 4-layer codebook design as an example:

| codebookMode = 1-2, |  |  |  |
| --- | --- | --- | --- |
| where . and the mapping from to and is given in Table 5.2.2.2.1-4. |  |  |  |

There are two SD basis, the SD basis for 1st layer and 3rd layer are same and the SD basis index interval between 1st SD basis and 2nd basis indicated by i1,3 is limited to (0, O1, O2) in n1 direction and (0, O2) in n2 direction according to Table 5.2.2.2.1-3 in TS 38.214. Such kind of design only allows UE to perform PMI selection for all layers dependently. The new codebook adds new (N1, N2) values and independent selection of SD basis for each layer is supported.

Observation 3: For scheme-B, there are fundamental changes for codebook.

Based on the above, we see protentional PMI selection impacted due to the new codebook design, hence it’s proposed to define PMI requirements for Type 1 single panel codebook for both scheme-A and scheme-B.

Even there is new design for both rank 1~4 and rank5~8. However, it’s proposed to only focus on rank 1~4 since rank5~8 is only applicable for 6Rx/8Rx UE, which is not a common UE type. We slightly prefer rank2, which is testable for both 2Rx and 4Rx UE.

According to RAN1’s design, 48,64 and 128 ports are introduced with 64 ports are supported as a part of respective basic feature and while those for P=48 and P=128 are supported as two separate UE capabilities.

Agreement For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities.

That means UE supporting R19 Type-1 codebook refinement should support at least 64 ports. Hence it’s enough to only consider 64 ports.

RAN1 has specified following (N1,N2) values:

Agreement (further modified on Thursday as shown in red) For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values: Total # CSI-RS ports across aggregated resources (=P); (N1, N2). 48; (8,3). (6,4). 64; (16,2). (8,4). 128; (16,4). (8,8).

It’s proposed to use (16,2) as starting point.

RAN1 has specified following number of aggregations of NZP CSI-RS resources (denoted as K) to attain 32 < P (or PCSI-RS) ≤ 128:

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding aggregation of K NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128, support only the following combinations of K and P (or PCSI-RS): - For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports) - For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports) - For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports)

It’s proposed to use K=2 as starting point.

RAN1 has specified that all aggregation NZP-CSI-RS resources can be located in 1 slot or 2 consecutive slots but “within 1 slot” is basic feature, therefore, it’s proposed to configure all aggregation CSI-RS resources within one slot.

Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, all the K>1 NZP CSI-RS resources also share the same QCL, PCoffset, and PCoffsetSS. In addition: - ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be UE capability.

RAN1 has specified following two capabilities for UE processing time:

Agreement For the Rel-19 Type-I SP and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports via aggregating K>1 CSI-RS resources, regarding timeline, introduce two UE capabilities: - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources Agreement For the Rel-19 Type-I SP codebook refinement for P=128 CSI-RS ports, with Capability 2 timeline, when periodic or semi-persistent CSI reporting is configured, nCSI_ref is the smallest value greater than or equal to 10⋅2μ, such that it corresponds to a valid downlink slot.

It can be observed that capability 1 is stricter, for capability 2, the timeline for CSI calculation is relaxed and scaled by ceil(P/32). The existing CSI-RS scheduling and CSI reporting pattern should be re-designed to match the capability 2.

Considering large number of Tx ports may lead to quite low SNR, it’s proposed to select MCS carefully to make SNR in reasonable range, MCS20 is used for R15 32Tx case, it’s proposed to use higher MCS such as MCS22 as starting point.

Proposal 2: RAN4 to introduce PMI requirements for Type 1 Single panel I codebook supporting up to 128 ports with following test setup:

Scheme A and B

Wideband granularity

Rank 2

64 Ports with K=2

(N1, N2) = (16,2)

Configure all aggregation CSI-RS resources within one slot.

MCS22

Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

- RAN1 also introduced refinement for Type-1 multi-panel codebook, the agreements are copied as follows:

Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following (compromise between Scheme1 and Scheme2 described in RAN1#116bis): - W1 structure: Independent SD basis selection across all the Ng=K NZP CSI-RS resources, reusing legacy Rel-15 Type-I Pcsi-rs<16 SP SD basis selection rules with L=1 for RI=1-4 - Ng = K = {2, 4} denotes the number of NZP CSI-RS resources associated with the Ng panels - W2 structure: - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource, - Layer-common wideband inter-resource QPSK co-phasing independently reported for resource n = 2,…,K with respect to the first resource. Rel-19 Type-I MP does not support RI=5-8. Reuse Rel-15 Type-I MP legacy designs for UCI omission, and CBSR. For CSI calculation, reuse Rel-18 Type II CJT CSI-RS port ordering for UE assumption on the transmitted PDSCH symbols across antenna ports.

The main change is that the new codebook supports independent SD basis selection across different panels while legacy codebook only supports single SD basis with phase adjustment applied for different panels. Such changes impact the performance and PMI selection algorithm, but RAN4 has never defined PMI performance for R15 Type-1 multi-panel codebook before, it’s proposed not to define PMI requirements for Type-1 multi-panel codebook.

Proposal 3: Not to define PMI requirements for Rel-19 Type-I multi-panel (MP) codebook supporting up to 128 CSI-RS ports

- For Type-II refine for up to 128 CSI-RS ports, the summary of changes is shown as follows:

Agreement For the Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports, in accordance to the WID, the following enhancement areas are supported: - Adding new (N1, N2) values for the Rel-16 eType-II regular and Rel-18 Type-II Doppler regular codebooks where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and O1=O2=4 - FFS: How to configure the aggregated NZP CSI-RS resources when AP-CSI-RS resources are configured as CMR for Rel-18 Type-II Doppler codebooks - Adding new PCSI-RS values for Rel-17 FeType-II Port Selection (PS) codebook where PCSI-RS (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources There will be separate UE feature groups for each of the enhanced codebooks. Note: Per WID objective 2b, - No other legacy codebook design aspects (such as SD/FD/DD basis design including O1/O2, W2/combining coefficient design, codebook parameter definitions and respective values) can be modified. - Only RI=1-4 is supported

It can be seen that for Rel-16 eType-II regular codebook and Rel-18 Type-II Doppler codebook, there is no codebook design changes except for introducing new (N1, N2) values to support up to 128 CSI-RS ports. For Rel-18 Type-II Doppler codebooks, RAN1 has specified the new aperiodic CMR configuration to aggregate NZP CSI-RS resources. For Rel-17 FeType-II Port Selection (PS) codebook, new PCSI-RS values is introduced. However, except for that UE need to do measurement for more CSI-RS ports, we don’t see the any impact on PMI selection algorithm. However, the capability of support more CSI-RS ports has been verified in Type 1 SP test. Hence, it’s proposed not to introduce PMI requirements for Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports.

Proposal 4: Not to introduce PMI requirements for Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports.

RAN1 also introduced multi-beam CRI reporting, supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource. Given the configured Ks CSI-RS resources, UE can report M CRIs, For Rel-15 Type-I Single Panel codebook, M is NW-configured via higher-layer (RRC) signaling with candidate value(s) of {1, …, min(4,KS)}. For Rel-16 eType-II, M=1 is supported. Considering RAN4 has never introduced CRI requirements before and CRI requirements is one of candidate topic for R20 demodulation performance enhancement, it’s proposed to not introduce new requirements for this feature.

Proposal 5: Not to introduce requirements for CRI-based CSI refinement

SRS port grouping for TDD low-complexity 6R/8Rx receiver

SRS port grouping is introduced to facilitate low-complexity 6R/8Rx receiver to support 2 CWs. The structure of low-complexity 6R/8Rx receiver is similar as simplified 8Rx receiver introduced in R18 8Rx requirements definition, the receiver is the combination of two separate 3Rx/4Rx MIMO detectors. BS can perform beamforming to map two CWs to two groups of receiving antennas corresponding to two groups of SRS port respectively and each group of receiving antennas is implemented in one MIMO detector. Here are RAN1’s agreements:

Agreement For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource): - SRS port group 0, corresponding to CW0, comprises the even PSRS/2 out of PSRS ports; and - SRS port group 1, corresponding to CW1, comprises the odd PSRS/2 out of PSRS ports The above feature is applicable only for reportQuantity = ‘cri-RI-CQI’. No other spec enhancement is introduced on new CW-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

The 8Rx UE supporting SRS port grouping has different UE baseband processing behaviour compared to the legacy R18 baseline 8Rx receiver and R18 simplified 8Rx receiver. The difference is that 8Rx UE supporting SRS port grouping uses two separate groups of 4Rx to decode two CWs separately, where each group of 4Rx matches the specific CW and the mapping between CW and receiving antennas/SRS groups are specified. The R18 simplified 8Rx receiver combines the post-symbol from two MIMO detectors and apply the decoding with one decoder.

Observation 4: R19 8Rx receiver supporting SRS port grouping has different baseband processing behaviour compared to R18 simplified 8Rx receiver since the former applies two decoders for two MIMO detectors separately and the latter combines post-symbol from two MIMO detectors and applies one decoder.

Based on the observation4, we don’t think R18 simplified receiver requirements can be applied for R19 UE supporting SRS port grouping and propose to define the new requirements for this new UE. The test purpose is to verify whether the UE can identify the mapping between antenna groups/MIMO detector and Codeword correctly. It’s proposed to further discuss the test setup considering the test may require UE transmit SRS and BS perform beamforming to separate two CWs, which has never been done before for the demod and CSI test.

Proposal 6: RAN4 to define the performance requirements for R19 8Rx UE supporting SRS port grouping, FFS for the test setup.

Aperiodic standalone CJT calibration reporting

RAN1 also introduced UE CSI reporting enhancement for CJT deployment under non-ideal synchronization and backhaul including frequency, phase offset and delay offset reporting. RAN1 specified the reporting accuracy, granularity, dynamic range and reporting mechanism. RAN1 also specified the mechanism supporting to indicate UE to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset. Our understanding is that this feature is highly related to measurement rather than demodulation or PMI calculation and measurement related requirements are always introduced in RRM part rather than demod part. RRM is responsible for defining accuracy requirements for measurement. Therefore, it’s proposed not to introduce the requirements.

Proposal 7: Not to introduce requirements for aperiodic standalone CJT calibration reporting.

- Conclusion

This contribution provides our views on demodulation and CSI requirements for R19 MIMO evolution, the proposals and observations are:

Proposal 1: Not to introduce requirements for Enhancement for UE-initiated/even-driven beam management, Support for 3-antenna-port codebook-based transmission and Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Observation 1: For scheme-A and rank 1~4, there is no fundamental changes for codebook except for large number of CSI-RS ports.

Observation 2: For scheme-A and rank 5~8, there is fundamental changes for codebook.

Observation 3: For scheme-B, there are fundamental changes for codebook.

Proposal 2: RAN4 to introduce PMI requirements for Type 1 Single panel I codebook supporting up to 128 ports with following test setup:

Scheme A and B

Wideband granularity

Rank 2

64 Ports with K=2

(N1, N2) = (16,2)

Configure all aggregation CSI-RS resources within one slot.

MCS22

Re-discuss the CSI-RS scheduling and CSI reporting pattern to match the capability 2 of timeline to support up to 128 CSI-RS ports

Proposal 3: Not to define PMI requirements for Rel-19 Type-I multi-panel (MP) codebook supporting up to 128 CSI-RS ports

Proposal 4: Not to introduce PMI requirements for Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports.

Proposal 5: Not to introduce requirements for CRI-based CSI refinement

Observation 4: R19 8Rx receiver supporting SRS port grouping has different baseband processing behaviour compared to R18 simplified 8Rx receiver since the former applies two decoders for two MIMO detectors separately and the latter combines post-symbol from two MIMO detectors and applies one decoder.

Proposal 6: RAN4 to define the performance requirements for R19 8Rx UE supporting SRS port grouping, FFS for the test setup.

Proposal 7: Not to introduce requirements for aperiodic standalone CJT calibration reporting.