3GPP TSG-RAN WG4 Meeting #115	                                                                     R4-2507044

Malta, MT, 19th-23rd May, 2025

Title: 	Discussions on SCM

Source: 	Huawei, HiSilicon

Agenda item:	7.16.3

Document for:	Discussion

Background

This contribution provides our views on open issues for SCM.

- Enhanced TDL channel

The detailed introduction for enhanced TDL channel is captured in [2]. The general multi-cluster MIMO channel impulse response matrix with dimensions Nrx× Ntx at some time instant is

Hτ=n=1Nm=1Mnpn,m∙Dn(rx)Hn,mDn(tx)∙δτ-τn,m

where

- n is the cluster index, m is the delay tap index, and τ is the delay variable

- N is the number of clusters, and Mn is the number of delay taps in cluster n

- pn,m is the power of delay tap m of cluster n so that nmpn,m=1

- τn,m is the propagation delay of tap m of cluster n

- Hn,m is the spatially correlated fading MIMO channel matrix of delay tap m of cluster n with unit average power response and dimensions Nrx× Ntx

- Dn(rx) is the diagonal RX steering matrix of cluster n with dimensions Nrx× Nrx

- Dn(tx) is the diagonal TX steering matrix of cluster n with dimensions Ntx× Ntx

δτ is the Dirac delta function

Cluster model 1

For cluster model 1, each cluster is associated with a TDL channel model with full delay profile, such as RAN4 TDL-A, TDL-B, or TDL-C so that Mn=12 for each n. Thus, the complexity of the channel model grows with the number of clusters N. Cluster powers and tap powers have the relation pn,m=pn∙pmTDL, where pmTDL represents the power of tap m in the underlying TDL model. Furthermore, the tap delays τn,m=τmTDL are drawn directly from the underlying TDL model.

Comparison with legacy channel

We did a simulation to verify the performance for Rank4 and Rank8 and compare the performance with legacy channel model. Simulation assumptions are captured in Table 2.1-1:

Table 2.1-1: Simulation assumptions for comparison of cluster model 1 with legacy channel

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1) for 4Tx, (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Random precoder |
| Rank | 4,8 |
| Antenna configuration | 4T4R for Rank4, 8T8R for Rank8 |
| Channel | Legacy TDLC300-100 XPL medium Legacy TDLC300-100 XPL high Cluster model 1 with 2 clusters for Rank4 Cluster model 1 with 4 clusters for Rank8 |
| MCS | 13 |
| Cluster powers | For 2 clusters: p1, p2=[0dB, 0dB] For 4 clusters: p1, p2 , p3 p4 =[0dB, 0dB, 0dB, 0dB] |
| Cluster TX steering in 1st array dim | For 2 clusters: θtx1(1)=0, θtx1(2)=π For 4 clusters: θtx1(1)=0, θtx1(2)=π/2, θtx1(3)=π, θtx1(4)=3π/2 |
| Cluster RX steering in 1st array dim | For 2 clusters: θrx1(1)=0, θrx1(2)=π For 4 clusters: θrx1(1)=0, θrx1(2)=π/2, θrx1(3)=π, θrx1(4)=3π/2 |

Simulation results are captured in Figure 2.1-1

Figure 2.1-1: Simulation results for comparison of cluster model 1 with legacy channel model

It can be observed that legacy TDL channel has rank deficiency and cluster model 1 can improve the spatial freedom degree, from the simulation it can be observed that 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 1: Cluster model 1 can highly improve the spatial diversity compared to legacy TDL channel, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Configurability

The power ratio of four clusters is a variable which can adjust per CW/layer SINR to the desired level. For example, fixed precoder i1,1=i1,2=i2=0 can gather the signal of each rank on special direction, which matches the direction of pre-configured clusters with pre-configured power, in the case, equal power for all clusters lead to same performance for each layer, unequal power leads to difference performance. We did a simulation to verify this characteristic with the simulation assumptions in Table 2.1-2. In the simulation, two cases are simulated, case1: p1,p2,p3,p4=[0dB, 0dB, 0dB, 0dB] and case2: p1,p2,p3,p4=[0dB, 0dB, -9dB, -9dB] with fixed precoder i1,1=i1,2=i1,3=0. It can be observed that for case1, two codeword has same performance and for case2, two codeword have quite different performance, the reason is that special precoder i1,1=i1,2=i1,3=0 maps the signal of second codeword to last two clusters with low power.

Table 2.1-2: Simulation assumptions for verification on configurability

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Fixed precoder i1,1=i1,2=i1,3=0 |
| Rank | 8 |
| Antenna configuration | 8T8R |
| Channel | Cluster model 1 with 4 clusters |
| MCS | 13 |
| Cluster powers | Case 1: p1,p2,p3,p4=[0dB, 0dB, 0dB, 0dB] Case 2: p1,p2,p3,p4=[0dB, 0dB, -9dB, -9dB] |
| Cluster TX steering in 1st array dim | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

The simulation results are captured in Figure 2.1-2

Figure 2.1-2 Simulation results for verification on configurability

Observation 2: The configurability of cluster model 1 is reflected in the fact that power ratio of clusters is a variable to control the SINR imbalance of layers to the desired level.

Cluster model 2

In the second clustering option, the delay profile of a single TDL channel model is split into multiple clusters. As a result, the implementation complexity of the model remains almost equal to the underlying TDL model as the total number of delay taps n=1NMn=12, regardless of the number of clusters. Here, the channel tap delays and powers are drawn directly from the underlying TDL model.

Comparison with legacy channel

To check the performance of cluster model 2 and compare it with legacy channel model, a simulation was done with simulation assumptions captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for comparison of cluster model 2 with legacy channel

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1) for 4Tx, (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Random precoder |
| Rank | 4,8 |
| Antenna configuration | 4T4R for Rank4, 8T8R for Rank8 |
| Channel | Legacy TDLC300-100 XPL medium Legacy TDLC300-100 XPL high Cluster model 2 in Table 2.2-3 for Rank4,TDLC300-100, XPL high for each cluster Cluster model 2 in Table 2.2-4 for Rank8, TDLC300-100, XPL high for each cluster |
| MCS | 13 |

Table 2.2-2: Cluster model 2 with 2 TX-RX clusters

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θk1=0 θk2=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 |  |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 | Cluster 2 θk1=π θk2=π |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

Table 2.2-3: Cluster model 2 with 4 TX-RX clusters-type1

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θtx2(1)=0 θrx1(1)=0 θrx2(1)=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θtx1(2)=-π/2 θtx2(2)=-π/2 θrx1(2)=-π/2 θrx2(2)=-π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 | Cluster 3 θtx1(3)=-π θtx2(3)=-π θrx1(3)=-π θrx2(3)=-π |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 | Cluster 4 θtx1(4)=-3π/2 θtx2(4)=-3π/2 θrx1(4)=-3π/2 θrx2(4)=-3π/2 |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

It can be observed that legacy TDL channel has rank deficiency and cluster model 2 can improve the spatial diversity, similar observation as cluster model 1, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 3: Cluster model 2 can highly improve the spatial diversity compared to legacy TDL channel, 2 and 4 clusters can support Rank4 and Rank8 respectively.

The simulation results are shown in Figure 2.2-1:

Figure 2.2-1: Simulation results for comparison of cluster model 1 with legacy channel model

Configurability

The mapping between clusters and taps can be a variable to control the per CW/layer SINR to the desired level. For example, mapping one cluster to a group taps with low power and large delay leads to poor performance for the layer corresponding to this cluster and mapping one cluster to a group of taps with high power and small delay can lead to good performance for the layer corresponding to this cluster. Same time, fixed precoder is used to project the signal of each layer to specific cluster. A simulation was done to verify this characteristic, two types of cluster-tap mapping were assumed. Type 1 was captured in Table 2.2-4, type 2 was captured in Table 2.2-6. From the simulation results it can be observed that for type 1, the power of each cluster varies slightly, leading to not large performance difference between two codewords. For type 2, 2nd cluster and 3rd cluster are mapped to last four taps with low power and large delay, leading to big performance difference between two codewords. Noted that fixed precoder i1,1=i1,2=i1,3=0 project signal corresponding to first codeword to first 2 clusters and second codeword to last 2 clusters.

Table 2.2-4: Simulation assumptions for verification on configurability

| Parameters | Values |
| --- | --- |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1) for 8Tx. (1 dimension antenna array) |
| Precoder | Single panel I, Fixed precoder i1,1=i1,2=i1,3=0 |
| Rank | 8 |
| Antenna configuration | 8T8R |
| Channel | Cluster model 1 with 4 clusters, XPL high for each cluster, TDLC300-100 |
| MCS | 13 |
| Cluster TX steering in 1st array dim | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Table 2.2-5: Cluster model 2 with 4 TX-RX clusters-type2

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θk1=0 θk2=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θk1=-π/2 θk2=π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θk1=-π θk2=-π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θk1=-3π/2 θk2=-3π/2 |
| 12 | 2595 | -16.0 |  |

The simulation results are shown in Figure 2.2-2:

Figure 2.2-2 Simulation results for verification on configurability

Observation 4: The configurability of cluster model 2 is reflected in the fact that mapping between clusters and taps is a variable to control the SINR imbalance of layers to the desired level

Cluster model 3

Then next a new extended TDL channel model is proposed harmonizing the cluster model 1 and cluster 2. The key idea is that instead of implementing each cluster in one TDLC channel or all clusters in one TDL channel, all clusters are split into fixed two TDL channels and specific PMI projects two CWs to clusters corresponding to two TDL channels respectively, which means signal of two CWs will pass two TDL channels respectively and power ratio of two channels is introduced to adjust the performance of two codewords to the exact desired level. Take following table as example:

Table 2.3-1: Cluster configuration for each TDL channel

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam for TDL channel 1 p1 | Tx-Rx Beam for TDL channel 2 p2 |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx11=0 θtx21=0 θrx11=0 θrx21=0 | Cluster 3 θtx13=-π θtx23=-π θrx13=-π θrx23=-π |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Cluster 2 θtx12=-π/2 θtx22=-π/2 θrx12=-π/2 θrx22=-π/2 | Cluster 4 θtx14=-3π/2 θtx24=-3π/2 θrx14=-3π/2 θrx24=-3π/2 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Since at most 8 layers will be tested so two TDL channels with 4 clusters is enough to cover all potential tests. The advantage of this new approach is that it keeps the balance of complexity and configurability since no additional complexity is introduced compared to existing dual cluster model in 38.101-4 and is more configurable compared to cluster model 2. Please see the simulation verifying the characteristic of cluster model 3:

Conclusion

The pros and cons of three cluster model are summarized as following:

|  | Cluster model 1 | Cluster model 2 | Cluster model 3 |
| --- | --- | --- | --- |
| Pros | More configurable, performance gap between two codewords is predictable can be adjusted the any desired level. | No complexity introduced compared to legacy TDL channel | Similar configurability as cluster model 1, the complexity is between cluster model 1 and 2. |
| Cons | More complicate, the complexity increases linearly as number of clusters increase | Less configurable, the trend but not exact of gap between two codewords is predictable and configurable. |  |

Based on above analysis, our concern for cluster model 1 is high implementation complexity and long simulation time. Therefore our preference is to take cluster model 2 and cluster 3 as first priority to study.

Proposal 1: RAN4 to prioritize cluster model 2 and 3.

Current alignment cases for cluster model 1, 2 and 3 are based on 4 clusters, however our analysis above shows that 2 clusters are enough to support rank4. Based on our PMI simulations in [3], the gain of follow PMI over random PMI is limited compared to CDL channel, the reason is that 4 clusters provide good spatial diversity already, which leads to limited beamform gain compared to 2 clusters. Therefore, we propose to consider 2 clusters for rank<=4 PDSCH and PMI cases and 4 clusters for rank>4 PDSCH and PMI cases for all cluster models (Noted that cluster model 3 is only applicable for Rank>4 cases).

Proposal 2: RAN4 to consider 2 clusters for rank<=4 PDSCH and PMI cases and 4 clusters for rank>4 PDSCH cases for all cluster models.

- CDL channel

Importance of introducing beam steering

We have strong concern that current CDL channel defined in TS 38.901 cannot verify UE PMI selection algorithm correctly. The reason is that the spatial characteristic is fixed if the angular information for each cluster is fixed and UE can identify the best PMI to match the pre-defined CDL channel in advanced which leads to the risk that UE cheating to use the report the pre-identified PMI all the time instead of performing PMI selection algorithm can also pass the test.

We did a simulation to compare fixed PMI performance with follow PMI performance. We first perform the measurement to exploit the most suitable PMI matching the A2 channel with ray splitting then outcome is that the most suitable PMI is (i1,1,i1,2,i1,3,i2)=(1,0,0,0), which is used for fixed PMI case.

Table 3-1: Simulation assumptions for fixed PMI vs follow PMI case

| Parameter | Value |
| --- | --- |
| Duplex mode | TDD |
| TDD Slot Configuration Pattern | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS | 40MHz/30kHz |
| Channel model | CDL channel A2 with ray splitting |
| Antenna configuration | Rank2: 8T4R |
| MCS | 13 (64 QAM table) |
| PMI | Single panel I - Random - Fixed PMI (Fixed PMI (i1,1,i1,2,i1,3,i2)=(1,0,0,0)) - follow PMI |

The simulation results are shown in Figure 3-1.

Figure 3-1: Simulation results for fixed PMI vs follow PMI case.

It is observed that fixed PMI outperforms follow PMI at high SNR, even at middle and low SNR, fixed PMI has quite same performance as follow PMI, which means UE cheating to report pre-identified PMI all the time can also pass the test.

Observation 5: For A2 channel with ray splitting, UE cheating to report pre-identified PMI (i1,1,i1,2,i1,3,i2)=(1,0,0,0))outperforms reporting follow PMI with PMI selection algorithm.

Beam steering has been implemented for TDL channel for all LTE and NR PMI tests, we don’t see any reason to discard it for CDL channel.

Observation 6: Beam steering has been implemented for TDL channel for all LTE and NR PMI tests, there is no reason to discard for CDL channel

Also, the same concern has also been raised in clause 7.7.5.1 in TS 38.901, see the following text copied from 38.901:

#### 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

To address the concern, TS 38.901 introduces scaling of angles to enable angle values variation, which has same effect of beam steering in RAN4 PMI test.  Therefore, we strongly propose to introduce beam steering for both CDL channel and extension TDL channel. For extension TDL channel, current beam steering approach defined in B.2.3.2.3 in TS 38.101-4 can be applied. For CDL channel, one potential approach is to make AOD/ZOD of each cluster time varying.

Proposal 3: RAN4 shall introduce beam steering for both CDL channel and extension TDL channel. For extension TDL channel, current beam steering approach defined in B.2.3.2.3 in TS 38.101-4 can be applied. For CDL channel, make AOD/ZOD of each cluster time varying.

One issue for introducing beam steering for CDL channel is that truncated channel may not no longer be valid. According to the truncation step defined in [1], the 12 strongest clusters in original channel considering AAV will be retained, however, if beam steering approach is applied, the clusters with angular changes in truncated channel may no longer be the 12 strongest clusters in original channel. Therefore, RAN4 shall discuss how to address this problem if beam steering is agreed.

Proposal 4: If beam steering is agreed, RAN4 shall discuss how to address the problem that the clusters with angular changes in truncated channel may no longer be the 12 strongest clusters in original channel.

Generic of CDL

The target frequency for A3 CDL channel is 6GHz, which is only applicable for TDD band. However, CDL channel parameters such as desired angular spread and doppler highly depends on frequency but demodulation requirements are defined as band agnostic.

Observation 7: The key parameters of CDL channel such as doppler and desired angular spread highly depends on frequency but demodulation requirements are defined as band agnostic.

Our proposal is to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

Proposal 5: RAN4 to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

AAV assumptions

Two AAV options are listed in [2]: without AAV and fixed subarray.

#### AAV Configuration for comparison Way Forward Options: - Option 1 (previously 1Y): with Antenna Array Virtualisation (AAV) using subarray configuration - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports - For interested companies: (M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only - Option 3: with AAV with the following configuration - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

The way forward says the “AAV configuration for comparison”. However, until now we don’t have a clear consensus on intention and purpose of comparison and what is the test metric of comparison. What’s the criteria to judge two AAV approaches? Based on our simulation results shown in the previous meeting, there is no performance gain of AAV option 1Y compared to no AAV. Therefore, we don’t see any benefit of evaluating AAV 1Y.

Proposal 6: Proponents of option 1 Y are welcome to clarify the benefit of this AAV approach and clarify the intention and purpose of comparison for two AAV approaches.

Ray splitting

There was a debate on whether to consider ray splitting for non-truncation channel. We simulated A2 channel with and without ray splitting and simulation results are shown in [3]. Following observations are made:

Observation 8: There is 2dB performance degradation for Rank4 case and 3.3dB performance degradation for Rank8 case due to the lack of ray splitting.

Observation 9: There is 1.5dB performance degradation for Type1 follow PMI Rank4 case and 0.8dB performance degradation for eType2 follow PMI Rank4 case due to the lack of ray splitting.

Considering the big impact on performance, it’s proposed to consider ray splitting for non-truncation channel.

Proposal 7: RAN4 to consider ray splitting for truncation channel (If agreed)

Issues on truncated channel

Our simulation results in [3] show that truncated channel (A4) performs better than A2 without ray splitting ray but worse than A2 with ray splitting channel. The SNR gap between A4 and A2 without ray splitting is 0.1dB for rank4 and 2.4dB(CW1) for rank8. The SNR gap between A4 and A2 with ray splitting is 1.9dB for rank4 and 1.6dB(CW0)

Observation 10: Truncated channel (A4) performs better than A2 without ray splitting ray but worse than A2 with ray splitting channel. The performance gap is noneligible.

The other problem is that the angular spread of truncation channel (A4) is not same as the desired value for UMa scenario. The intention of angular scaling is to scale the angular of each cluster to desired value matching the desired scenario, e,g, UMa.  It has been agreed to do angular scaling first and do truncation secondly. However, this leads to the issue that angular spread of truncated channel doesn’t match the desired value.

Following table captures the angular spread of AOD/AOD/ZOD/ZOA for A4 channel and desired value for UMa scenario. It can be observed that there is still some difference.

|  | ASdesired, ASD | ASdesired, ASA | ASdesired, ZSD | ASdesired, ZSA |
| --- | --- | --- | --- | --- |
| Desired value for UMa scenario | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| A4 | 23.1476 | 68.5220 | 3.7329 | 14.6701 |

Observation 11: Doing angular scaling first and truncation secondly leads to the issue that angular spread of truncated channel doesn’t match the desired value. There is some difference between angular of A4 and desired value.

Therefore, it’s proposed to further discuss how to improve the truncation procedures to maintain the performance and angular spread.

Proposal 8: RAN4 to further discuss how to improve the truncation procedures to maintain the performance and angular spread.

- Conclusion

This contribution provides our views on SCM for demodulation requirements. The proposals and observations are:

Observation 1: Cluster model 1 can highly improve the spatial diversity compared to legacy TDL channel, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 2: The configurability of cluster model 1 is reflected in the fact that power ratio of clusters is a variable to control the SINR imbalance of layers to the desired level.

Observation 3: Cluster model 2 can highly improve the spatial diversity compared to legacy TDL channel, 2 and 4 clusters can support Rank4 and Rank8 respectively.

Observation 4: The configurability of cluster model 2 is reflected in the fact that mapping between clusters and taps is a variable to control the SINR imbalance of layers to the desired level

Proposal 1: RAN4 to prioritize cluster model 2 and 3.

Proposal 2: RAN4 to consider 2 clusters for rank<=4 PDSCH and PMI cases and 4 clusters for rank>4 PDSCH cases for all cluster models.

Observation 5: For A2 channel with ray splitting, UE cheating to report pre-identified PMI (i1,1,i1,2,i1,3,i2)=(1,0,0,0))outperforms reporting follow PMI with PMI selection algorithm.

Observation 6: Beam steering has been implemented for TDL channel for all LTE and NR PMI tests, there is no reason to discard for CDL channel

Proposal 3: RAN4 shall introduce beam steering for both CDL channel and extension TDL channel. For extension TDL channel, current beam steering approach defined in B.2.3.2.3 in TS 38.101-4 can be applied. For CDL channel, make AOD/ZOD of each cluster time varying.

Proposal 4: If beam steering is agreed, RAN4 shall discuss how to address the problem that the clusters with angular changes in truncated channel may no longer be the 12 strongest clusters in original channel.

Observation 7: The key parameters of CDL channel such as doppler and desired angular spread highly depends on frequency but demodulation requirements are defined as band agnostic.

Proposal 5: RAN4 to select one typical band for FDD and TDD respectively and utilize the centre frequency of such two typical FDD band TDD band as target frequency to derive desired angular spread and doppler. The two scaled CDL models can represent TDD and FDD scenario respectively for requirements definition.

Proposal 6: Proponents of option 1 Y are welcome to clarify the benefit of this AAV approach and clarify the intention and purpose of comparison for two AAV approaches.

Observation 8: There is 2dB performance degradation for Rank4 case and 3.3dB performance degradation for Rank8 case due to the lack of ray splitting.

Observation 9: There is 1.5dB performance degradation for Type1 follow PMI Rank4 case and 0.8dB performance degradation for eType2 follow PMI Rank4 case due to the lack of ray splitting.

Proposal 7: RAN4 to consider ray splitting for truncation channel (If agreed)

Observation 10: Truncated channel (A4) performs better than A2 without ray splitting ray but worse than A2 with ray splitting channel. The performance gap is noneligible.

Observation 11: Doing angular scaling first and truncation secondly leads to the issue that angular spread of truncated channel doesn’t match the desired value. There is some difference between angular of A4 and desired value.

Proposal 8: RAN4 to further discuss how to improve the truncation procedures to maintain the performance and angular spread.

- Reference

[1] R4-2504756 Way Forward for [114bis][320] NR_SCM

[2] R4-2504759 TP to TR38.753: TDL approaches and related Annex

[3] R4-25xxxxx Simulation results for SCM. Huawei, HiSilicon.