3GPP TSG-RAN WG4 Meeting #115	                                                                     R4-2507046

Malta, MT, 19th-23rd May, 2025

Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO cases

Source: 	Huawei, HiSilicon

Agenda item:       7.16.2

Document for:	Approval

Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

- Draft TP

# 6  Comparison of spatial channel Models

- 6.1  PDSCH performance under SU-MIMO scenario

This section provides comparison for different spatial channel models with common simulation assumptions captured in Table 6.1-1.

Table 6.1-1: Common Simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Channel model |  |  | Cluster model 1 Cluster model 2 Cluster model 3 CDL channel model |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | For CDL channel: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination For TDL channel: Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | Rank 4: SNR@70% of maximum throughput Rank8: SNR(dB) @ 30% and 70% of max throughput for each codeword |

Simulation assumptions for cluster model 1 are captured in Table 6.1-2:

Table 6.1-2: Simulation assumptions for Cluster model 1

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Number of clusters | 4 | 4 |
| Cluster powers p1, p2, p3, p4 | [0dB, 0dB, -9dB, -9dB] | [0dB, 0dB, -9dB, -9dB] |
| Per cluster configuration | XPL-High, TDLC300-10 | XPL-High, TDLC300-10 |
| Cluster TX steering in 1st array dim θtx1(1),θtx1(2),θtx1(3), θtx1(4) | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 | θtx1(1)=0, θtx1(2)=-π/2, θtx1(3)=-π, θtx1(4)=-3π/2 |
| Cluster RX steering in 1st array dim θrx1(1),θrx1(2),θrx1(3), θtx1(4) | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 | θrx1(1)=0, θrx1(2)=-π/2, θrx1(3)=-π, θrx1(4)=-3π/2 |

Simulation assumptions for cluster model 2 are captured in Table 6.1-3:

Table 6.1-3: Simulation assumptions for Cluster model 2

| Parameters | Values |  |
| --- | --- | --- |
|  | Rank4 | Rank8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (2,1,2,1) | (4,1,4,1) |
| Precoder | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10 | XPL-High, TDLC300-10 |
| Cluster-tap mapping | Table 6.1-4 | Table 6.1-4 |

Table 6.1-4: Cluster model 2 with 4 TX-RX clusters

| Tap | Delay(ns) | Power | Beam steering |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 | Cluster 2 θtx1(2)=-π/2 θrx1(2)=-π/2 |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 |  |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 | Cluster 3 θtx1(3)=-π θrx1(3)=-π |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 | Cluster 4 θtx1(4)=-3π/2 θrx1(4)=-3π/2 |
| 12 | 2595 | -16.0 |  |

Simulation assumptions for cluster model 3 are captured in Table 6.1-5:

Table 6.1-5: Simulation assumptions for comparison of cluster model 3

| Parameters | Values |
| --- | --- |
| Rank | 8 |
| Antenna setup: Ntx1, Ntx2, Nrx1, Nrx2 | (4,1,4,1) |
| Precoder for PDSCH and DMRS | Single panel I, Fixed PMI with i1,1=i1,2=i2=0 |
| Per cluster configuration | XPL-High, TDLC300-10 |
| Channel powers p1, p2, | p1, p2=[0dB, -9dB] |
| Cluster-tap mapping | Table 6.1-6 |

Table 6.1-6: Cluster model 3 with 4 clusters

| Tap | Delay(ns) | Power(dB) | Tx-Rx Beam for TDL channel 1 p1 | Tx-Rx Beam for TDL channel 2 p2 |
| --- | --- | --- | --- | --- |
| 1 | 0 | -6.9 | Cluster 1 θtx1(1)=0 θrx1(1)=0 | Cluster 3 θtx1(3)=-π θrx1(3)=-π |
| 2 | 65 | 0 |  |  |
| 3 | 70 | -7.7 |  |  |
| 4 | 190 | -2.5 |  |  |
| 5 | 195 | -2.4 | Cluster 2 θtx1(2)=-π/2 θrx1(2)=-π/2 | Cluster 4 θtx1(4)=-3π/2 θrx1(4)=-3π/2 |
| 6 | 200 | -9.9 |  |  |
| 7 | 240 | -8.0 |  |  |
| 8 | 325 | -6.6 |  |  |
| 9 | 520 | -7.1 |  |  |
| 10 | 1045 | -13.0 |  |  |
| 11 | 1510 | -14.2 |  |  |
| 12 | 2595 | -16.0 |  |  |

Simulation assumptions for CDL channel specifically are captured in Table 6.1-7:

Table 6.1-7: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

The summary of simulation results for Rank 4 are captured in Table 6.1-8

Table 6.1-8: Summary of simulation results for Rank4

| Channel model | SNR (dB)@70%@ of max throughput |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Company 7 |
| Cluster mode 1 |  |  |  |  |  |  |  |
| Cluster mode 2 |  |  |  |  |  |  |  |
| Cluster mode 3 |  |  |  |  |  |  |  |
| CDL channel |  |  |  |  |  |  |  |

The summary of simulation results for Rank 8 are captured in Table 6.1-9

Table 6.1-9: Summary of simulation results for Rank8

| Table 6.1-8: Summary of simulation results for Rank4Channel model | SNR (dB)@70%@ of max throughput |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company 1 | Company 2 | Company 3 | Company 4 | Company 5 | Company 6 | Company 7 |
| Cluster mode 1 |  |  |  |  |  |  |  |
| Cluster mode 2 |  |  |  |  |  |  |  |
| Cluster mode 3 |  |  |  |  |  |  |  |
| CDL channel |  |  |  |  |  |  |  |

Observations: TBD

- Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.