3GPP TSG-RAN WG4 Meeting #115	  R4-2507048

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	7.21.3

Source:	Huawei, HiSilicon

Title:	Discussion on CSI reporting requirement and testing framework for CSI prediction

Document for:	Discussion

# Introduction

According to WF in RAN4 #114bis [1], we discuss the remaining issues related to CSI reporting requirement and testing framework for CSI prediction.

# Testing setup

RAN4 #113bis: Channel model for CSI prediction tests For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. RAN4 #114bis Updated simulation assumptions for AIML CSI prediction [2]: Table 1: Updated simulation assumptions for AIML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Subcarrier spacing; 15kHz for FDD, 30kHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. gNB TX antennas; Option1:32(N1,N2)=(4,4) as defined in Rel-15 type1 Option2:16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements. UE RX antennas; 4. Channel model; TDLA30 with XP medium correlation as baseline. Doppler spread; 20Hz, 50Hz and 100Hz. Channel estimation for CSI prediction; Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). CSI-RS configuration; FFS: following CSI-RS configuration: - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). - For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5).. Rank per UE; Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline. CSI feedback assumption; Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account. Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed. Test setup(timeline); Only for step-3 FFS: following test setup (timeline): For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration. SNR assumption; Only for step-3 [FFS, e.g., Option1: claimed by companies for step-2 first round simulation(TP as KPI) Option2: Example: [-6:2:10] dB Option3: other suggestions ]. CSI feedback overhead; PC7 as baseline. Baseline; FFS: - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-2 or performance requirements). KPI; Step-1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Step-2: - check SGCS after CSI feedback Step-3: - check TPs after CSI feedback. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms. Training data and test data; Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. 100Hz Doppler spreads; 100Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. 100Hz Doppler spreads..

It is observed that in RAN4 legacy, TDL channel models are used for PMI reporting testing. Spatial channel model for demodulation performance requirements is still under study in RAN4. It is suggested that RAN4 will not consider CDL channel model in AI CSI prediction.

Proposal 1: CDL channel model is not used in RAN4 for AI CSI prediction.

For generalization test, there are two options to verify the performance of AI CSI prediction. One is to define several requirements under different test configurations/conditions. If the DUT is able to pass all the tests, then it means that the performance of CSI prediction can be guaranteed in a generalized scenario consisting of these configurations/conditions. This method is what RAN4 legacy uses.

RAN4 #113 WF: Testing conditions Use static conditions as baseline. - FFS on whether to have non-static condition

Another is that the test configurations/conditions vary over time during testing. This method has never been used in RAN4 before. Firstly, the intention is not clear to introduce more complicated testing compared legacy for generalized performance verification. Secondly, when model switching is involved under non-static condition to ensure the performance, LCM procedure testing and generalized performance testing are coupled together in one testing.

Proposal 2: Non-static condition is not used for generalized performance testing.

For LCM procedure, if model/performance monitoring and management are performed by NW, there is no need to introduce non-static condition for UE testing either. When to activate/deactivate AI is up to NW implementation. For UE-side monitoring, two types are on the table in RAN1. One is to report performance metric, another is to report performance monitoring output. The former one needs to define the performance metric and corresponding report mechanism. The latter one needs to define the performance metric, threshold criterion and corresponding report mechanism. The performance metric and threshold definition may be subject to configurations/conditions, which has not been discussed yet in other WGs.

Proposal 3: RAN4 will start to discuss whether/how to define requirements for UE-side monitoring if other WGs achieve sufficient progress.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: CDL channel model is not used in RAN4 for AI CSI prediction.

Proposal 2: For generalization testing, there is no need to use non-static condition.

Proposal 3: RAN4 will start to discuss whether/how to define requirements for UE-side monitoring if other WGs achieve sufficient progress.

# References

- R4-2505107, “WF on AI ML for air interface study”, 3GPP RAN4 #114bis, 7th ‒ 11th April, 2025.

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, 3GPP RAN4 #114bis, 7th ‒ 11th April, 2025.