3GPP TSG-RAN WG4 Meeting #115	  R4-2507053

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	7.22.2

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects on AI/ML for NR air interface phase 2

Document for:	Discussion and Decision

# Introduction

In RAN#106 plenary meeting [1], a new SID on AI/ML for air-interface was approved for the remaining Release 19, where the study objectives on AI/ML general framework are given below:

RAN#106 New SID on AI/ML [1] The following areas will be further studied as part of this project: - CSI feedback enhancement [RAN1]: - For CSI temporal prediction: further update TR 38.843 with additional evaluations. - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study as it may relate to their required work.

According to the new SID, RAN4 is expected to finalize the testing framework and procedure for one-sided models and further analyze the various testing options for two-sided models.

This contribution focuses on general aspects on AI/ML for NR air interface phase 2.

# Discussion

- Relation to legacy requirements

For legacy PMI reporting, relative throughput compared to random PMI is used as test metric. In AI-CSI compression, the testing framework can be reused. Since the motivation of introducing AI-CSI compression is to achieve performance gain compared to Rel-16 eType II codebook-based PMI reporting, there are two options for considering the relation to legacy requirement.

- Option 1: With CSI feedback reduction rate of X%, AI-based throughput gain is no worse than that of Rel-16 eType II.

- Option 2: With no larger CSI feedback payload, AI-based throughput is Y% higher than that of Rel-16 eType II.

Proposal 1: Regarding to relation to legacy requirements in AI-CSI compression, RAN4 to discuss based on the following two options.

- Option 1: With CSI feedback reduction rate of X%, AI-based throughput gain is no worse than that of Rel-16 eType II.

- Option 2: With no larger CSI feedback payload, AI-based throughput is Y% higher than that of Rel-16 eType II.

For AI-BM, relation to legacy requirements can be discussed for UE-side model and NW-side model, respectively. For NW-side model, UE reports L1-RSRP to NW as model input. In this case, the requirement of measurement accuracy can reuse legacy. For UE-side model, UE reports prediction of L1-RSRP as model output to NW. In this case, new reporting is introduced and RAN4 will discuss how to define the requirement accordingly.

Proposal 2: Regarding to relation to legacy requirements in AI-BM, RAN4 to reuse legacy requirement for L1-RSRP for NW-side model. For UE-side model, RAN4 to discuss how to define the requirement of prediction accuracy.

For AI-positioning, there is no need to define requirement for case 1 since it is UE direct positioning. For case 2a and case 2b, timing information and/or LoS/NLoS indicator as model output is reported by UE/gNB to LMF. It is observed that whether and how to distinguish AI-based reporting from legacy method-based reporting is still under RAN1 discussion. If there is no method to distinguish AI from non-AI, RAN4 will reuse legacy requirement of timing information and LoS/NLoS indicator. For case 2b and 3b, timing information, paired timing information and power information are supported while phase information is still under discussion. For timing information, paired timing information and power information, RAN4 can still reuse legacy. If phase information is introduced, RAN4 will discuss how to define the requirement accordingly.

Proposal 3: Regarding to relation to legacy requirements in AI-Pos, for case 2a and 3a, RAN4 to reuse legacy requirement for timing information and/or LoS/NLoS indicator if AI-based reporting and non-AI-based reporting cannot be distinguished from specification perspective.

Proposal 4: Regarding to relation to legacy requirements in AI-Pos, RAN4 to reuse legacy requirement for timing information, paired timing information and power information in case 2b and 3b.

- Post-deployment validation due to model change/drift

For verifying post deployment performance, model monitoring can be employed as one method. This is related to requirement of LCM procedure. Another option is to define reference model for both one-sided model and two-sided model. With this method, minimum performance can be guaranteed no matter how AI model is updated in real deployment. For example, if the performance of newly updated model is worse than that of reference model, then UE can switch from newly updated model to reference model or avoid activating the newly updated model. If the performance of newly updated model is better than that of reference model, UE can directly switch to the newly updated model.

Proposal 5: For post-deployment validation due to model change/drift, RAN4 to discuss the verification method based on the following two options.

- Option 1: Rely on model monitoring in LCM procedure.

- Option 2: Define reference model to switch if newly updated model is worse than the reference model.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Regarding to relation to legacy requirements in AI-CSI compression, RAN4 to discuss based on the following two options.

- Option 1: With CSI feedback reduction rate of X%, AI-based throughput gain is no worse than that of Rel-16 eType II.

- Option 2: With no larger CSI feedback payload, AI-based throughput is Y% higher than that of Rel-16 eType II.

Proposal 2: Regarding to relation to legacy requirements in AI-BM, RAN4 to reuse legacy requirement for L1-RSRP for NW-side model. For UE-side model, RAN4 to discuss how to define the requirement of prediction accuracy.

Proposal 3: Regarding to relation to legacy requirements in AI-Pos, for case 2a and 3a, RAN4 to reuse legacy requirement for timing information and/or LoS/NLoS indicator if AI-based reporting and non-AI-based reporting cannot be distinguished from specification perspective.

Proposal 4: Regarding to relation to legacy requirements in AI-Pos, RAN4 to reuse legacy requirement for timing information, paired timing information and power information in case 2b and 3b.

Proposal 5: For post-deployment validation due to model change/drift, RAN4 to discuss the verification method based on the following two options.

- Option 1: Rely on model monitoring in LCM procedure.

- Option 2: Define reference model to switch if newly updated model is worse than the reference model.

# References

- RP-242393, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.