3GPP TSG-RAN WG4 Meeting #115	  R4-2507054

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	7.22.3

Source:	Huawei, HiSilicon

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #114bis [1], we discuss the remaining issues related to CSI compression two-sided models.

# Feasibility criteria of full model standardization

RAN4 #112 WF [2]: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

According to the previous agreement of RAN4, companies provide simulation results under a common model structure using their own training dataset. It is expected that after a round of parameters refinement, good alignment can be achieved. Currently, the intermediate KPI SGCS is collected and the criteria of performance alignment have not been determined yet. Since the primary goal during SI phase is to reach a consensus on whether we can achieve agreement regarding the complete model, encompassing both its structure and parameters, using the difference span of SGCS as a criterion of procedure feasibility verification should suffice.

Observation 1: Using SGCS as a key metric for verifying the feasibility of procedures related to the standardization of full models should suffice.

If we do not consider eventual throughput alignment, it seems rather challenging to determine the specific value of the SGCS difference span at which performance can be considered convergent, and thus the feasibility of the procedure can be claimed. Maybe we could define multiple degrees of performance convergence based on SGCS difference span. For the SGCS difference span, we propose the following approach: First, calculate the average SGCS across companies, excluding the maximum and minimum values. Subsequently, determine the percentage difference between each company's SGCS and the calculated average. Finally, take the absolute value of the percentage difference with the largest absolute value as the SGCS difference span.

Proposal 1: Use the value of SGCS difference span as the criteria to verify the feasibility related to the standardization of full models. Define the SGCS difference span as follows.

- Calculate the average SGCS across companies, excluding the maximum and minimum values.

- Determine the percentage difference between each company's SGCS and the calculated average.

- Take the absolute value of the percentage difference with the largest absolute value as the SGCS difference span.

Proposal 2: Define multiple degrees of performance convergence based on SGCS difference span:

- SGCS difference span > 10% indicates low - degree convergence.

- SGCS difference span within [5%, 10%] indicates medium - degree convergence.

- SGCS difference span < 5% indicates high - degree of convergence.

# Simulation results of Option 3

RAN4 #114bis WF [1]: ## Reduction of simulation results volume In order to simplify simulations, the following was agreed; - No more results are expected Option 3 track 1 - No more results are expected Unquantized model results - For option 3 track 2, use the Samsung decoder as the frozen decoder. Lower priority for Nokia decoder. - For option 4, use the Samsung encoder as the frozen encoder. Lower priority for Nokia encoder.

Currently, we are at the feasibility verification step 4. Two tracks have been identified for this step. In track 1, each company generates its own training dataset, which means the datasets are not aligned among companies. In track 2, the training dataset is generated by mixing the datasets from different companies, which means the datasets are aligned among companies. According to WF in RAN4#114bis, no more results are expected in Option 3 track 1 and no more results are expected for unquantized models.

It is agreed in R18 38.843 that the assumption of encoder should consider the complexity limitation at UE.

However, the complexity of agreed CNN model structure at the feasibility study stage is much higher than that of R16 eType-II, as shown in Table 1.

Table 1: Complexity of CNN (current assumption in RAN4) and R16 Type-II

| CNN-based encoder (current assumption in RAN4) | 47 MFLOPs |
| --- | --- |
| R16 Type-II | 0.37 MFLOPs |

RAN4 #114bis WF [1]: ## Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

According to RAN4#114bis WF, using a lower complexity CNN or transformer encoder is encouraged and model structures as well complexity should be reported by companies. We use the models as suggested in [3].

RAN4 #114bis Simulation assumptions on AI/ML CSI compression [3]: ## 3. Model structure assumptions for encoder with lower complexity (for information) ## 3.1 Transformer based lower-complexity encoder Figure 1-1. Model structure of CNN-based encoder with 4.14MFLOPs Table 1-2. Description of CNN-based encoder with 4.14MFLOPs Model type; CNN (Resnet like design). FLOPs; 4.14M. Model size; 92Kbyte. Num of Resnet blocks; 2. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,16). Size of message (feature map) passed across layers; (13,32). ## 3.2 CNN based lower-complexity encoder Figure 2-1. Model structure of Transformer-based encoder with 3.63MFLOPs Table 2-2. Description of Transformer-based encoder with 3.63MFLOPs Model type; Transformer. FLOPs; 3.63M. Model size; 1.6Mbyte. d_model; 64. d_feedforward; 256. N_head; 8. d_head; 64. N_TF; 4.

In the following, simulation results of RAN4 previously assumed encoder and low-complexity encoders are both provided in Option 3 track 2.

Proposal 3: SGCS results for Option 3 Track 2 are provided in Table 2.

Table 2. SGCS results for Option 3 Track 2

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own [Note: Not captured in WF] | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Huawei, Hisilicon | 0.71 | - | 0.704 | 0.6619 | - | 0.6517 |
| Encoder-2 (Transformer with 3.63M FLOPs) | Huawei, Hisilicon | 0.6793 | - | 0.6217 | 0.2360 | - | 0.5834 |
| Encoder-3 (CNN with 4.14M FLOPs) | Huawei, Hisilicon | 0.6911 | - | 0.6852 | 0.6481 | - | 0.6243 |

Observation 2: In Option 3 track 2, nearly no SGCS performance loss with fixed selected decoder using mixed dataset to retrain the encoder, assuming the encoder model structure is aligned between training and retraining.

Observation 3: In Option 3 track 2, around 6%~7% (0.6619 vs. 0.704 and 0.6517 vs. 0.704) SGCS performance loss is observed with fixed selected decoder using our own dataset to retrain the encoder.

Observation 4: Under a different backbone from current CNN-based assumption, larger than 10% (0.6217 vs. 0.704) SGCS is observed for encoder retraining with a lower-complexity Transformer-based model structure. However, less than 10% performance loss is observed if low-complexity Transformer-based model structure is used to derive the fixed decoder.

Observation 5: Under the same backbone, 3% (0.6852 vs. 0.704) SGCS performance loss is observed for encoder retraining with a lower-complexity CNN-based model structure. However, less than 1% (0.6852 vs. 0.6911) performance loss is observed if low-complexity CNN-based model structure is used to derive the fixed decoder.

Based on the Observations 4 and 5, if a lower - complexity encoder is not captured at the initial stage of deriving the test decoder, companies will experience an SGCS performance decline when they retrain the encoder with reduced complexity, even when using the same training dataset. In contrast, when a lower - complexity encoder is employed to derive the test decoder, there’s almost no observable performance degradation compared to using a significantly higher - complexity encoder, if the backbone is aligned.

Proposal 4: Use low complexity encoder to derive the test decoder if AI-CSI turns to WI.

If the backbone is not aligned, according to Observation 4, even though better SGCS performance is achieved when the lower - complexity encoder is employed to derive the test decoder, the SGCS performance loss exists compared to the backbone aligned cases. This can be a regarded as a generalization issue over different encoder structures. Therefore, to accommodate more flexible encoder designs for testing decoder, we can consider mixing up multiple encoder structures (with different backbones and different complexities) to derive the test decoder. As shown in Table 3, we train a common decoder under three encoder backbones at first, and then retrain the encoders with this fixed decoder. Compared with the decoder trained under only one encoder backbone (e.g., Encoder-1), it can be observed that the decoder trained under mixed encoder backbones can be compatible with more encoder backbones (e.g., Encoder-2), the differences between performance can be within a smaller range under various encoder structures.

Table 3. SGCS results under mixed encoder backbones for fixed decoder training (Option 3 Track 2)

|  | Training @ mixed, testing @ mixed |  |
| --- | --- | --- |
|  | Training with (Samsung’s) frozen decoder | Training with frozen decoder derived from mix encoder backbones |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | 0.704 | 0.70 |
| Encoder-2 (Transformer with 3.63M FLOPs) | 0.6217 | 0.6711 |
| Encoder-3 (CNN with 4.14M FLOPs) | 0.6852 | 0.6842 |

Proposal 5: Consider multiple encoder backbones to derive the test decoder if AI-CSI turns to WI.

# Simulation results of Option 4

RAN4 #114bis WF [1]: Further simulation activity for option 4 - All interested companies are requested to upload their “own” decoders to the server as soon as possible. - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset. - Definitions - Dataset 1 = Dataset used for training the frozen encoder - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder) - Dataset 3 = Dataset used for training the own encoder (using the own decoder) - First results priority results for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders) - Second priority results for RAN4#115: - Dataset 1 = Dataset 2 = Mixed training dataset - Dataset 3 = Company own dataset - Test Dataset: Mixed Dataset, also consider own test dataset if interested. - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - Results for consideration of lower complexity encoder for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156] - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

In the following, simulation results of RAN4 previously assumed encoder are provided in Option 4 track 2.

Proposal 6: SGCS results for Option 4 Track 2 are provided in Table 2.

Table 3. SGCS results for Option 4 Track 2

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st priority Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own' decoder (retrained with the frozen decoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6892 | - | 0.7064 | 0.6974 | - |
| Encoder-2 (Transformer with 3.63M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6222 | - | 0.6322 | 0.6339 | - |
| Encoder-3 (CNN with 4.14M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6716 | - | 0.6866 | 0.6769 | - |

Observation 6: Only 1% SGCS span among different companies’ decoders is observed when the own encoder is tested with other companies’ decoders.

Proposal 7: RAN4 to specify a reference encoder if AI-CSI turns to WI.

# Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: Using SGCS as a key metric for verifying the feasibility of procedures related to the standardization of full models should suffice.

Proposal 1: Use the value of SGCS difference span as the criteria to verify the feasibility related to the standardization of full models. Define the SGCS difference span as follows.

- Calculate the average SGCS across companies, excluding the maximum and minimum values.

- Determine the percentage difference between each company's SGCS and the calculated average.

- Take the absolute value of the percentage difference with the largest absolute value as the SGCS difference span.

Proposal 2: Define multiple degrees of performance convergence based on SGCS difference span:

- SGCS difference span > 10% indicates low - degree convergence.

- SGCS difference span within [5%, 10%] indicates medium - degree convergence.

- SGCS difference span < 5% indicates high - degree of convergence.

Proposal 3: SGCS results for Option 3 Track 2 are provided in Table 2.

Table 2. SGCS results for Option 3 Track 2

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own [Note: Not captured in WF] | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Huawei, Hisilicon | 0.71 | - | 0.704 | 0.6619 | - | 0.6517 |
| Encoder-2 (Transformer with 3.63M FLOPs) | Huawei, Hisilicon | 0.6793 | - | 0.6217 | 0.2360 | - | 0.5834 |
| Encoder-3 (CNN with 4.14M FLOPs) | Huawei, Hisilicon | 0.6911 | - | 0.6852 | 0.6481 | - | 0.6243 |

Observation 2: In Option 3 track 2, nearly no SGCS performance loss with fixed selected decoder using mixed dataset to retrain the encoder, assuming the encoder model structure is aligned between training and retraining.

Observation 3: In Option 3 track 2, around 6%~7% (0.6619 vs. 0.704 and 0.6517 vs. 0.704) SGCS performance loss is observed with fixed selected decoder using our own dataset to retrain the encoder.

Observation 4: Under a different backbone from current CNN-based assumption, larger than 10% (0.6217 vs. 0.704) SGCS is observed for encoder retraining with a lower-complexity Transformer-based model structure. However, less than 10% performance loss is observed if low-complexity Transformer-based model structure is used to derive the fixed decoder.

Observation 5: Under the same backbone, 3% (0.6852 vs. 0.704) SGCS performance loss is observed for encoder retraining with a lower-complexity CNN-based model structure. However, less than 1% (0.6852 vs. 0.6911) performance loss is observed if low-complexity CNN-based model structure is used to derive the fixed decoder.

Proposal 4: Use low complexity encoder to derive the test decoder if AI-CSI turns to WI.

Proposal 5: Consider multiple encoder backbones to derive the test decoder if AI-CSI turns to WI.

Proposal 6: SGCS results for Option 4 Track 2 are provided in Table 2.

Table 3. SGCS results for Option 4 Track 2

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st priority Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own' decoder (retrained with the frozen decoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6892 | - | 0.7064 | 0.6974 | - |
| Encoder-2 (Transformer with 3.63M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6222 | - | 0.6322 | 0.6339 | - |
| Encoder-3 (CNN with 4.14M FLOPs) | Huawei, Hisilicon | - | - | - | - | - | - | 0.6716 | - | 0.6866 | 0.6769 | - |

Observation 6: Only 1% SGCS span among different companies’ decoders is observed when the own encoder is tested with other companies’ decoders.

Proposal 7: RAN4 to specify a reference encoder if AI-CSI turns to WI.

# References

- R4-2505107, “WF on AI ML for air interface study”, 3GPP RAN4 #114bis, 7th ‒ 11th April, 2025.

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #112, May 20th – 24th, 2024.

- R4-2505156, “Simulation assumptions on AI/ML CSI compression”, 3GPP RAN4 #114bis, 7th ‒ 11th April, 2025.