3GPP TSG-RAN WG4 Meeting # 115	R4-2507055

# Malta, MT, 19th – 23rd May, 2025

Agenda item:	7.22.2

Source: 	National Taiwan University

Title: 	On AI/ML Model LCM and Performance Monitoring Requirement

Document for:	Approval

Introduction

We present our view on LCM and performance monitoring in this contribution

Discussion

Discussion on the Accuracy Requirement of LCM Procedures

In the previous meeting, RAN4 discussed LCM mostly from the procedure execution delay perspective [1]. Although the delay requirements are important for the network control purpose, whether that is important in practice is doubtful, since the major delay component could be the model downloading time, which depends on the link quality and out of 3GPP specification control, or the model switch procedure is indeed a slower parameter adaptation instead of one-time parameter set load.

Therefore, we want to bring to RAN4 attention that the discussion of whether the LCM decisions are precise and accurate can be more important than the delay requirement, especially the decision is taken by the UEs. Therefore, when an activation /switch /fallback action happens, we indeed should be more interested in whether these decisions are correct instead of how long it takes. If we want to study the test procedures for LCM decision accuracy, we need to design the environment and test conditions that can trigger LCM actions/decisions. We can leverage the ideas captured in TR 38.843 on the generalization test:

TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability.

The different conditions considered in the generalization test can be applied in the LCM test, but now we need to change the conditions during the test to trigger LCM actions and decisions.

Therefore, we propose the following:

Proposal 1: In order to discuss the necessity and feasibility of the LCM decision accuracy requirement, RAN4 needs to study the following

- The definition of metrics to evaluate the accuracy of LCM decision: since an explicit definition of a correct LCM decision is complicated, we suggest developing the accuracy requirement based on the following principle

After the LCM decision, the performance metrics (can leverage performance metrics of the legacy procedures, e.g., throughput, BLER) is improved by x%

- The proper environment condition or configuration change to verify the LCM procedure and decision: RAN4 needs to study what types of changes can trigger LCM decisions (and incur performance degradation if no LCM decision is taken) so that we can design a test to verify the LCM decision accuracy.

Conclusion

Proposal 1: In order to discuss the necessity and feasibility of the LCM decision accuracy requirement, RAN4 needs to study the following

- The definition of metrics to evaluate the accuracy of LCM decision: since an explicit definition of a correct LCM decision is complicated, we suggest developing the accuracy requirement based on the following principle

After the LCM decision, the performance metrics (can leverage performance metrics of the legacy procedures, e.g., throughput, BLER) is improved by x%

- The proper environment condition or configuration change to verify the LCM procedure and decision: RAN4 needs to study what types of changes can trigger LCM decisions (and incur performance degradation if no LCM decision is taken) so that we can design a test to verify the LCM decision accuracy.

Reference

[1] R4-2505107