3GPP TSG-RAN WG4 Meeting #115	R4-2507057

St. Julians, Malta, May 19th – 23rd 2025

Agenda Item:	7.22.3

Source:	Ericsson

Title:	Discussion on the 2-sided CSI study

Document for:	Discussion

# 1	Introduction

During RAN4#114bis, a set of further simulations to proceed with the study on 2-sided CSI requirements was agreed. This contribution presents results based on the agreed simulation set. Also, further consideration is given to the other open issues of what would be captured in the specifications and how, and the similarity between RAN4 and RAN1 discussions.

# 2	Option 3 Track 2 simulation results

In this evaluation, Dataset 1 refers to Dataset used to train the reference/test decoder and Dataset 2 refers to Dataset use to train the encoder with frozen reference/test decoder.

## 2.1	Additional result 1 (Dataset 1: Mixed dataset, Dataset 2; Ericsson dataset; Test set: mixed test set)

For the first evaluation, the following steps are taken:

- A joint training is conducted using the mixed dataset to obtain Samsung Track 2 Encoder and Samsung Track 2 Decoder. Note that this training is done by Samsung.

- An encoder (Ericsson own) is then trained with Samsung Track 2 Decoder (frozen) using the Ericsson training dataset.

- The pairs are then tested using the mixed testing dataset.

- The performance for the Ericsson Track 1 for the Ericsson part of the mixed test set is also given for a reference.

The evaluation results can be seen in Figure 1. By comparing the performance to the Ericsson Track 1 model (Ericsson Track 1 Encoder + Ericsson Track 2 Decoder), we could observe that Samsung Track 2 model (Samsung Track 2 Encoder + Samsung Track 2 Decoder) already could handle the Ericsson dataset well. I.e., there is only 0.5% performance drop (0.6882 vs 0.6916) when the model is tested with the Ericsson part of the mixed testing dataset. In this condition, training an encoder with Ericsson dataset does not give noticeable gain (improve from 0.6882 to 0.6685). A larger gain (when tested with Ericsson part of the mixed test set) may be expected if the performance difference of Samsung Track 2 model compared to Ericsson Track 1 model is substantial.

In average, the performance degradation from training Ericsson own encoder from Samsung Track 2 Decoder is 1.2% (0.6932 vs 0.7018). Note that training a new encoder with Ericsson dataset (Ericsson own encoder) causes the encoder to accommodate the Ericsson dataset more compared to the other datasets. This can be seen when we compare the performance of the Ericsson own Encoder – Samsung Track 2 Decoder pair compared to the Samsung Track 2 Encoder – Samsung Track 2 Decoder pair when being tested with the Mediatek, Nokia, Oppo, and Vivo parts of the mixed test set (losses of 5.0%, 0.3%, 0.4%, and 0.4%, respectively). This may suggest that the characteristics of Ericsson dataset has a larger difference toward Mediatek dataset compared to the other datasets

Figure 1	Performance of training an encoder with a reference decoder. Reference decoder: Samsung Track 2; Dataset 1: Mixed; Dataset 2; Ericsson; Test set: Mixed

A similar evaluation has also been done by using the Nokia Track 2 Decoder. The evaluation can be seen in the following figure. In the figure, Ericsson Own N refers to an encoder trained with Nokia’s Track 2 Decoder.

Figure 2	Performance of training an encoder with a reference decoder. Reference decoder: Nokia Track 2; Dataset 1: Mixed; Dataset 2; Ericsson; Test set: Mixed.

From the above figure, we can derive a similar observation as in Figure 1, i.e., there is a performance degradation when the own Encoder is trained with Ericsson dataset and tested with the mixed dataset. Looking in a per company testing dataset, there is variance on the performance gain/loss which depends on the channel characteristics similarity between the dataset used to train the reference decoder and the dataset used to train the own Encoder. Note that compared to the Ericsson Track 1 models, Nokia Track 2 model has a performance loss of about 1.0% (0.6845 vs 0.6916) when being tested with the Ericsson part of the mixed dataset. Training an Encoder using the Ericsson dataset, therefore, could give a better gain (+0.2%) compared to the case of Samsung Track 2 is being used as the reference decoder (+0.0%).

Training an encoder using Dataset 2 with a frozen decoder trained using Dataset 1 may cause a performance gain/loss when being tested with test set corresponds to Dataset 1. The value of the gain/loss depends on the similarity between the Dataset 2 and (part of) Dataset 1.

## 2.2	Additional result 2 (Dataset 1: Mixed dataset, Dataset 2; Ericsson dataset; Test set: Ericsson test set)

In this evaluation, the same model as in Section 2.1.1 is being used but tested with Ericsson test set. Note that the samples for the test set used in this evaluation is different than the Ericsson part of the mixed test set (although the samples are generated under the same simulation assumption). The SGCS performance for several encoder-decoder pairs are listed in Table 1. In the table, the performances for Samsung Track 2 model, Nokia Track 2 model, and Ericsson Track 1 model are also given for comparison.

Table 1	Performance of training an encoder with a reference decoder. Reference decoder: Samsung Track 2 and Nokia Track 2; Dataset 1: Mixed; Dataset 2; Ericsson; Test set: Ericsson

| Tested pairs |  | SGCS |
| --- | --- | --- |
| Encoder | Decoder |  |
| Samsung Track 2 | Samsung Track 2 | 0.6859 |
| Ericsson own | Samsung Track 2 | 0.6863 |
| Nokia Track 2 | Nokia Track 2 | 0.6816 |
| Ericsson own | Nokia Track 2 | 0.6831 |
| Ericsson Track 1 | Ericsson Track 1 | 0.6892 |

From the above table, we can observe that training an Encoder with Ericsson dataset slightly improves the performance when being tested with Ericsson test set. Note that this improvement depends on the similarity between the dataset used to train the decoder and the dataset used to train the own encoder.

Comparing the performance of Samsung Track 2 model (0.6859) to the performance of Ericsson Track 1 model (0.6892), the Samsung Track 2 model may already cover the Ericsson dataset well (only 0.5% performance loss). The performance improvement of training a new (own) encoder with Ericsson dataset + Samsung Track 2 frozen decoder is only limited, i.e., only 0.1% in this case (0.6863 vs 0.6859)

Similarly, comparing the performance of Nokia Track 2 model (0.6816) to the performance of Ericsson Track 1 model (0.6892), there is only a performance loss of 1.1%, i.e., the Nokia Track 2 model may already cover the Ericsson dataset well. The performance improvement of training a new (own) encoder with Ericsson dataset is note much, i.e., only 0.1% gain in this case (0.6831 vs 0.6816).

Training an encoder using Dataset 2 with a frozen decoder trained using Dataset 1 may provide a performance gain when being tested with test set corresponds to Dataset 2. The value of the gain depends on the similarity between the Dataset 2 and Dataset 1.

## 2.3	Additional result 3 (Evaluation with low complexity encoder, Dataset 1 = Dataset 2 = Mixed Dataset; Test Set: Mixed)

In this evaluation, a plurality of encoder sizes is being considered. In particular, some convolution sizes and number of Resnet blocks are being used. The configuration and the complexity of the models can be seen in the following tables. In the table, Config 0 represents the configuration agreed for the RAN4 evaluation with CNN-based model.

Table 2	Complexities and sizes for several Encoder configurations.

| Encoder Config | Backbone | Conv. Size | # Resnet blocks | Params (M) | FLOPs (M) | Storage (MB) |
| --- | --- | --- | --- | --- | --- | --- |
| Encoder Config 0 | CNN | 32 | 6 | 0.13 | 87.44 | 0.50 |
| Encoder Config 1 | CNN | 32 | 4 | 0.09 | 58.50 | 0.35 |
| Encoder Config 2 | CNN | 32 | 2 | 0.05 | 29.55 | 0.21 |
| Encoder Config 3 | CNN | 16 | 2 | 0.02 | 7.67 | 0.09 |

To assess the performance of training with a reference decoder using a lower complexity encoder, a performance baseline shall be determined, e.g., by evaluating the performance of a joint training between the lower complexity encoder and the RAN4 agreed decoder structure. Therefore, in the below table, the performances for several encoder configurations with the agreed decoder obtained via joint training are also listed together with the performance of training of several encoder configurations trained via frozen agreed decoder. The evaluation is done by train a model using a mixed training dataset and test the model using a mixed test set. The loss for each lower complexity encoder in each training mechanism is calculated by comparing its performance to the performance of the agreed encoder using the same training mechanism. Note that for the joint training with Encoder Config 1, Encoder Config 2, and Encoder Config 3 configurations, we use epoch of 110, 70, and 100, respectively as we observe overfitting after that epoch.

Figure 3	Performance of training with a lower complexity encoder using a reference decoder. Reference decoder: Samsung Track 2; Dataset 1: Mixed; Dataset 2; Mixed; Test set: Mixed.

From the above figure, we can observe that the performance degradation of having lower complexity encoder compared to the agreed encoder are similar for both the joint training case and the frozen test decoder training case. This implies that performance degradation of the lower complexity encoder during the frozen decoder training comes from the performance of the lower complexity encoder itself rather than the training incompatibility between the lower complexity encoder and the frozen test (agreed) decoder. Thus, we may conclude that training with a frozen reference decoder is feasible regardless of the encoder sizes/complexities. The overall performance, however, may be limited by the encoder size/complexity.

Note that in this evaluation, we use Samsung Track 2 decoder for the training with frozen decoder. The better performance of the training with frozen decoder compared to joint training may be caused by the better training quality in producing the Samsung Track 2 model and shall not be seen as an indication that training with frozen decoder is superior to joint training.

Training with a frozen reference decoder is generally feasible across encoder sizes/complexities. The overall performance, however, may be limited by the encoder size/complexity.

# 3	Option 4 simulation results

In this evaluation, Dataset 1, Dataset 2, and Dataset 3 are defined as below:

- Dataset 1: Dataset used to train the frozen encoder.

- Dataset 2: Dataset used to train the own decoder with the frozen decoder.

- Dataset 3: Dataset used to train the own encoder with the own decoder.

## 3.1	Additional Result 1 (Dataset 1 = Dataset 2 = Dataset 3 = Mixed dataset)

In this evaluation, Samsung Track 2 Encoder is used as the reference encoder (frozen). A decoder is then trained with that Encoder using a mixed training dataset. The decoder is referred to as own decoder, e.g., Ericsson own decoder. The performance of pairs of Samsung Track 2 Encoder – Companies’ own decoder with mixed test set is listed in the below table. The performance gain/loss is compared to the Samsung Track 2 Encoder – Samsung Track 2 Decoder pair.

Figure 4	Performance of Samsung Track 2 encoder with several companies’ own decoders. Dataset 1 = Dataset 2 = Dataset 3 = Mixed.

From the above figure, we can observe that the companies’ own decoder performance is generally aligned with a gain/loss range of 2.5% (-2.3% to +0.2%) for the mixed test set and with a gain/loss range of 2.8% (-2.4% to +0.4%) for Ericsson test set compared to the Samsung Encoder Track 2 – Samsung Decoder Track 2 pair.

Based on the Ericsson own decoder, a training is then done with the mixed dataset to obtain a new encoder (Ericsson own). The Ericsson own encoder is then paired with other companies’ own decoders and tested with the mixed testing dataset and the Ericsson testing dataset. The evaluation results can be seen in the following table. In the table, the range refers to the performance gap of companies’ own decoder to Ericsson own decoder.

Table 3	Performance of Ericsson own encoder with several companies’ own decoders. Dataset 1 = Dataset 2 = Dataset 3 = Mixed

| Test set | Encoder | Companies’ own decoders |  |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ERIC | HW | MTK | NOK | QC | VIVO |  |
| Mixed | Samsung Track 2 | 0.6922 | 0.7035 | 0.6857 | 0.6854 | 0.6979 | 0.6937 | -1.0% to 1.6% |
|  | Ericsson own | 0.6929 | 0.7000 | 0.6832 | 0.6830 | 0.6958 | 0.6921 | -1.4% to 1.0% |
|  | Gain/loss | +0.1% | -0.5% | -0.4% | -0.4% | -0.3% | -0.2% |  |
| Ericsson | Samsung Track 2 | 0.6777 | 0.6885 | 0.6697 | 0.6696 | 0.6833 | 0.6787 | -1.2% to 1.6% |
|  | Ericsson own | 0.6789 | 0.6854 | 0.6678 | 0.6680 | 0.6816 | 0.6772 | -1.6% to 1.0% |
|  | Gain/loss | +0.2% | -0.5% | -0.3% | -0.2% | -0.2% | -0.2% |  |

From the above table, we can observe that Ericsson own encoder is generally compatible with other companies’ decoders, i.e., the performance loss compared to the reference encoder (Samsung Track 2 Encoder) is less than 0.5%. From another point of view, the performance of Ericsson own Encoder paired with companies’ own decoder varies between 0.6830 to 0.7000 (2.4% variation) for the mixed test set and between 0.6678 to 0.6854 (2.6% variation) for the Ericsson test set. Such a variation is similar with the performance of the Samsung Track 2 Encoder when it is paired with companies’ own decoders, i.e., varies between 0.6854 to 0.7035 (2.6% variation) for the mixed test set and between 0.6696 to 0.6885 (2.8% variation) for the Ericsson test set. This suggests that a new encoder obtained from a reference encoder is compatible with decoders obtained from the reference encoder, at least when the new encoder and the decoder is trained using the same dataset.

For the case of Dataset 1 = Dataset 2 = Dataset 3, a new encoder obtained from a reference encoder is compatible with decoders obtained from the reference encoder.

## 3.2	Additional Result 2 (Dataset 1 = Dataset 2 = Mixed dataset; Dataset 3 = Ericsson dataset)

In this evaluation, Samsung Track 2 Encoder is used as the reference encoder (frozen). A decoder is then trained with that Encoder using a mixed training dataset. The decoder is referred to as own decoder, e.g., Ericsson own decoder. Based on the Ericsson own decoder, a training is then done with the Ericsson dataset to obtain a new encoder (Ericsson own). The Ericsson own encoder is then paired with other companies’ own decoders and tested with the Ericsson testing dataset and the mixed testing dataset. The evaluation results can be seen in the following table. In the table, the range refers to the performance gap of companies’ own decoder to Ericsson own decoder.

Table 4	Performance of Ericsson own encoder with several companies’ own decoders. Dataset 1 = Dataset 2 = Mixed; Dataset 3 = Ericsson

| Test set | Encoder | Companies’ own decoders |  |  |  |  |  | Range |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ERIC | HW | MTK | NOK | QC | VIVO |  |
| Ericsson | Samsung Track 2 | 0.6777 | 0.6885 | 0.6697 | 0.6696 | 0.6833 | 0.6787 | -1.2% to 1.6% |
|  | Ericsson own | 0.6795 | 0.6868 | 0.6700 | 0.6696 | 0.6827 | 0.6782 | -1.5% to 1.1% |
|  | Ericsson own | +0.3% | -0.2% | +0.0% | +0.0% | -0.1% | -0.1% |  |
| Mixed | Samsung Track 2 | 0.6922 | 0.7035 | 0.6857 | 0.6854 | 0.6979 | 0.6937 | -1.0% to 1.6% |
|  | Ericsson own | 0.6873 | 0.6949 | 0.6783 | 0.6776 | 0.6903 | 0.6861 | -1.4% to 1.1% |
|  | Gain/loss | -0.7% | -1.2% | -1.1% | -1.1% | -1.1% | -1.1% |  |

From the above table, we can observe that Ericsson own encoder is generally compatible with other companies’ decoders. The performance of Ericsson own Encoder paired with companies’ own decoder varies between 0.6696 to 0.6868 (2.6% variation) for the Ericsson test set and between 0.6776 to 0.6949 (2.5% variation) for the mixed test set. Such a variation is similar with the performance of the Samsung Track 2 Encoder when it is paired with companies own decoder, i.e., varies between 0.6696 to 0.6885 (2.8% variation) for the Ericsson test set and between 0.6854 to 0.7035 (2.6% variation) for the mixed test set.

As the Ericsson own encoder is trained with Ericsson dataset, the encoder shall have a better performance than the Samsung Track 2 Encoder when being tested with Ericsson test set. From the third row of Table 4, however, we can observe some losses occurs for some decoders (up to 0.2%). We shall note, however, obtaining the Ericsson own encoder requires several steps where each step may contain training imperfection. From the last row of Table 3, there is a performance degradation of up to 0.5% when the dataset used to train the encoder (Dataset 3) is the mixed dataset. This suggests that training the Encoder with Ericsson dataset (slightly) improves the performance of the models when it is being tested with Ericsson test set.

When the Encoder – decoder pairs are tested with mixed dataset, however, we can observe a performance degradation up to 1.2%. Part of the reason for the performance degradation comes from the training imperfection that may come from each training step to obtain the Ericsson own encoder (e.g., by seeing the third row of Table 3 where there is up to 0.5% performance loss). Additional performance degradation comes from the fact that although the mixed dataset is generated under one simulation assumption, the dataset characteristics from each company is different.

In the below figure, the performances of Ericsson own Encoder that is trained with Ericsson dataset and Ericsson own Encoder that is trained with mixed dataset when being paired with Ericsson own Decoder are given for each part of mixed dataset.

Figure 5	Performance comparison of Ericsson own encoder with Dataset 3 = Mixed vs Dataset = Ericsson. Ericsson own decoder and Mixed test set are used.

Similar to the above figure, the performance for the case of the Ericsson own Encoder is paired with Nokia own decoder is given in the below figure.

Figure 6	Performance comparison of Ericsson own encoder with Dataset 3 = Mixed vs Dataset = Ericsson. Nokia own decoder and Mixed test set are used.

From the above figure, we can observe that training the encoder with Ericsson dataset may improve the performance of the model when being tested with the Ericsson part of the mixed test set. The gain is non substantial most likely due to the Ericsson dataset characteristics are covered well by the mixed dataset. In contrary, performance loss is observed when the model pair is tested with the Mediatek part of the mixed test set, most likely as the dataset characteristic is different to some extend although is generated under the same simulation assumption.

For the case of Dataset 1 = Dataset 2, a new encoder obtained from a reference encoder is compatible with decoders obtained from the reference encoder.

Compared to train a new encoder with Dataset 3 = Dataset 2, training a new encoder using Dataset 3 ≠ Dataset 2 may provide a performance gain when being tested with test set corresponds to Dataset 3. The value of the gain depends on the similarity between the Dataset 3 and Dataset 1 and 2.

## 3.3	Additional Result 3 (Dataset 1 = Dataset 2 = Dataset 3 = Mixed dataset; Lower complexity encoder)

In this evaluation, a plurality of encoder sizes is being considered. In particular, some convolution sizes and number of Resnet blocks are being used. The configuration and the complexity of the models can be seen in the Table 2. In the table, Config 0 represents the configuration agreed for the RAN4 evaluation with CNN-based model.

To assess the performance of training with a reference encoder using a lower complexity own encoder, a performance baseline shall be determined, e.g., by evaluating the performance of a joint training between the lower complexity encoder and the RAN4 agreed decoder structure. Therefore, in the below figure, the performances for several encoder configurations with the agreed decoder obtained via joint training are also listed together with the performance of training of several encoder configurations trained via reference encoder. The evaluation is done by train a model using a mixed training dataset and test the model using a mixed test set. The loss for each lower complexity encoder in each training mechanism is calculated by comparing its performance to the performance of the agreed encoder using the same training mechanism. Note that for the joint training with Encoder Config 1, Encoder Config 2, and Encoder Config 3 configurations, we use epoch of 110, 70, and 100, respectively as we observe overfitting after that epoch.

Figure 3	Performance of training with a lower complexity encoder using Samsung Track 2 Encoder; Dataset 1 = Dataset 2 = Dataset 3 = Mixed; Test set: Mixed.

From the above figure, we can observe that the performance degradation of having lower complexity encoder compared to the agreed encoder are similar for both the joint training case and the frozen reference encoder training case. This implies that performance degradation of the lower complexity encoder during the frozen reference encoder training comes from the performance of the lower complexity encoder itself rather than the training incompatibility between the lower complexity encoder and the frozen reference encoder. Thus, we may conclude that training with a frozen reference encoder is feasible regardless of the encoder sizes/complexities. The overall performance, however, may be limited by the own encoder size/complexity.

Training with a frozen reference encoder is generally feasible across a plurality of new (own) encoder sizes/complexities. The overall performance, however, may be limited by the new (own) encoder size/complexity.

## 3.4	Additional Result 4 (Dataset 1 = Mixed; Dataset 2 = Dataset 3 = Ericsson dataset)

In this section, the performance of training a new encoder based on a reference encoder, where Dataset 1 = Mixed dataset and Dataset 2 = Dataset 3 = Ericsson dataset is evaluated. The simulation results can be seen in the following table. In the table, the performances of having Dataset 1 = Dataset 2 = Dataset 3 = Mixed dataset and having Dataset 1 = Dataset 2 = Mixed dataset and Dataset 3 = Ericsson dataset are also given for a comparison.

Table 5	Performance comparison of training an encoder with a reference encoder using several training dataset setups.

| No | Training dataset |  |  | Test set |  |
| --- | --- | --- | --- | --- | --- |
|  | Dataset 1 | Dataset 2 | Dataset 3 | Mixed | Ericsson |
| 1 | Mixed | Mixed | Mixed | 0.6929 | 0.6789 |
| 2 | Mixed | Mixed | Ericsson | 0.6873 | 0.6795 |
| 3 | Mixed | Ericsson | Ericsson | 0.6680 | 0.6826 |
| 4 | Gain/loss of 3 to 1 |  |  | -3.6% | 0.54% |
| 5 | Gain/loss of 3 to 2 |  |  | -2.8% | 0.46% |

From the above table, we can observe that using Dataset 2 = Dataset 3 = Ericsson dataset could improve the performance of the model when being tested with the Ericsson dataset. Compared to the {Dataset 1 = Dataset 2 = Dataset 3 = mixed dataset} case and the {Dataset 1 = Dataset 2 = mixed dataset and Dataset = Ericsson dataset} case, the performance improvement is 0.54% and 0.46%, respectively. The gain is quite limited due to the fact that the Ericsson dataset distribution is generally covered well by the mixed dataset.

The above performance improvement, however, comes at the cost of performance degradation when the model is being tested with the mixed test set. Compared to the {Dataset 1 = Dataset 2 = Dataset 3 = mixed dataset} case and the {Dataset 1 = Dataset 2 = mixed dataset and Dataset = Ericsson dataset} case, the performance loss is 3.6% and 2.8%, respectively. The performance degradation occurs as the Ericsson dataset characteristics/distributions is generally a subset of the mixed dataset characteristics/distributions. As can be seen in the following table, there can be a performance gain or loss if the model is being tested with the subset of the mixed test set.

Table 6	Performance comparison of training an encoder with a reference encoder using several training dataset setups, tested with each part of the mixed test set.

| No | Training Dataset |  |  | Test Set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dataset 1 | Dataset 2 | Dataset 3 | Ericsson | Mediatek | Nokia | Oppo | Vivo |
| 1 | Mixed | Mixed | Mixed | 0.6812 | 0.7058 | 0.6966 | 0.7102 | 0.6705 |
| 2 | Mixed | Mixed | Ericsson | 0.6821 | 0.6782 | 0.6969 | 0.7096 | 0.6697 |
| 3 | Mixed | Ericsson | Ericsson | 0.6854 | 0.5832 | 0.6962 | 0.7070 | 0.6680 |
| 4 | Gain/loss of 3 to 1 |  |  | 0.6% | -17.4% | -0.1% | -0.5% | -0.4% |
| 5 | Gain/loss of 3 to 2 |  |  | 0.5% | -14.0% | -0.1% | -0.4% | -0.3% |

Similar to the previous observations for the mixed test set, training with Dataset 2 = Dataset 3 = Ericsson dataset improves the performance of the Ericsson part of the mixed test set. However, this comes at the cost of performance degradation for other parts of mixed test set, in particular the Mediatek part.

Training a model with a frozen reference encoder with Dataset 2 = Dataset 3 = Ericsson dataset improves the model performance compared to models trained with Dataset 2 = mixed when being tested with Ericsson test set or the Ericsson part of the mixed test set. However, performance degradation occurs when being tested with other part of the mixed test set.

# 4	Non-simulation considerations

## 4.1	What should be specified for option 3

The WF from RAN4#114bis was as follows:

Agreements

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

For further discussion:

The following topics should be further discussed:

- For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured.

- Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available

  - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments

- Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

Reference encoder

During RAN4#114bis, it was agreed that for option 3, a reference encoder(s) will be retained, and that the reference encoder will be the encoder(s) that was trained with the test decoder. A couple of issues remain open from the agreement. Firstly, whether there is a single reference encoder per test decoder or several reference encoders.

During the existing simulation campaign, a single encoder-decoder pair was jointly trained in order to obtain the “frozen” test decoder. Then, companies generated a single new encoder for the decoder.

Previously it has been speculated that if there are different applicable conditions or UE capabilities, then possibly there may be different test decoders for the different conditions. No investigation has taken place on whether separate decoders would be needed; there has not been much discussion on generalization for the two sided case. It would obviously be preferable to keep the number of test decoders to a minimum, preferably one, but is for further study whether this would be possible.

If multiple test decoders would be generated, then presumably there would be a reference encoder associated with each decoder and each of the pairs would be kept within 3GPP.

Possibly more than one test decoder might be considered if there are requirements covering quite different scenarios; the need for different scenarios is part of the discussion on ensuring functionality generalization. Even with several requirement scenarios, whether multiple decoders would really be needed would need carefuly study.

If there would be several test decoders, then there would be at least one reference encoder per test decoder.

The proposal during RAN4#114bis was somewhat different; it was to consider the possibility of several reference encoders for a single decoder. One reason given why this might be done is that there could be several encoders of differing complexity level, and that the decoder could be trained with each of them. This would give the UE vendor the option of differing complexity level encoders.

If the performance of all of the lower complexity encoders together with the test decoder would be very similar, then it would not be obvious why a UE vendor would not simply use the lowest complexity encoder to achieve the performance requirements, and there would be no obvious need to store multiple reference encoders.

If lower complexity encoders are feasible with the same performance, then the reference encoder could simply be the lowest complexity encoder.

If different complexity levels of encoder would achieve different levels of performance with the same test decoder, then potentially several reference encoders could be captured. However, this would first of all require an agreement on standardizing several levels of performance requirement (presumably with different UE capabilities); it is not clear whether this would be particularly beneficial.

If there could be multiple performance levels for the same requirement scenario and decoder, possibly multiple reference encoders might be considered.

It is unusual for RAN4 to create several performance levels for the same requirement scenario.

Even if there would be several levels of performance and associated encoders of different complexity, it is not obvious whether the test decoder would need to be trained with all of the encoders, or whether it would be sufficient that the test decoder is trained with one of the encoders, which could be a reference. Some insight into this question may be gained from the simulation campaign, which is considering lower complexity encoder interoperation with the decoder.

It is also important to note that in the current study, the decoder/encoder structure are too complex. In a real WI, more care should be given to ensuring that the structure is not too complex. If a more reasonable structure is agreed, then lower complexity encoders may not be as feasible.

Dataset used for training of the test decoder

A second question is whether to store the training dataset used for the training of the test decoder. This question is predicated on the dataset being open and available during the WI phase. It is at least theoretically possible that the training could be carried out using non-public data and then models merged or similar. This theoretical possibility may be difficult to do in an open standardization effort, however.

Assuming that the dataset is generated from simulation assumptions and made publicly available, as has been the case in the simulation campaign then the question becomes whether to dispose of or retain the training dataset.

It is not strictly necessary to keep the dataset. However, it could be useful for network vendors in creating real decoders or UE vendors in creating real encoders, or even as a future reference in case the test decoder should be updated.

If a dataset has been made available during the WI phase, it could be useful to retain the dataset as a reference when creating real encoders/decoders.

Assuming that the dataset is kept on a 3GPP server during a WI phase, then the question is actually on the retention policy for the training dataset.

If a dataset has been made available during the WI phase then the question whether to keep the dataset is a question of retention policy on the 3GPP server.

Dataset used for verification of the test decoder

It may be discussed whether it is useful to keep a small dataset for verifying operation of the test decoder. Potentially this may be a question for RAN5.

Test dataset

A further open question in the WF is on whether a test dataset is needed. An alternative to a test dataset is to define a channel model for testing, as is usually the case for demodulation requirements.

At this stage, it is rather early to decide on this question. Conventionally, a channel model is used when defining requirements. However, as has been seen for track 3 option 1, even with aligned simulation assumption, different companies’ interpretation of the assumptions and implementations of the simulation can lead to quite different performance. Hence, an agreed test dataset may be useful to ensure consistent testing.

It is not clear whether a test dataset or channel model is more useful in the specifications. This may depend on whether it can be expected that different implementations (by different TE vendors) of the channel model provide consistent performance.

## 4.2	What should be specified for option 4

During RAN4#114bis, the WF for option 4 was as follows:

Agreement:

- Dataset is specified for option 4a.

- FFS on dataset is captured for option 4b.

- Assume reference decoder is captured.

- FFS on reference encoder for option 4a

- Test decoder structure is (i) fully specified or (ii) partially specified

- NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase.

For further discussion:

The above FFS; i.e.

- Whether a dataset is captured for option 4b

- Whether a reference encoder is specified for option 4a

Option 4 consists of option 4a, in which a labelled test dataset is standardized, and option 4b, in which at least an encoder is standardized. For option 4a, by definition a dataset is standardized and for 4b an encoder.

At RAN4#114bis, it was agreed that a reference decoder will be captured in 3GPP. The reference decoder should be the decoder that was jointly trained with the encoder for option 4b, or the encoder that was used for creating the dataset for option 4a. Similarly to the option 3 reference encoder, it is not obvious whether the reference decoder should be referred to from a specification or should simply be kept within a 3GPP database together with the encoder.

For option 4b, RAN4#114bis left it open whether a dataset should be kept within 3GPP. In our understanding, even if a dataset is not kept, a very exact description of the expected contents of the dataset would be required, otherwise option 4b would fail to produce interoperable test decoders. (For example, if one TE vendor would take a dataset for rural, another for indoor and another for high speed, it is very unlikely that the latent space would be interoperable from each TE vendor. Considering the learning from option 3 track 1, in which even with aligned simulation assumptions, different performance was seen with different vendors datasets, in our view, in order to get fully interoperable test decoders, it is highly likely that a dataset needs to be included in the standard for option 4b.

It is highly likely that a dataset needs to be standardized for option 4b, considering experiences from option 3 track 1.

A further question in the WF is on whether the test decoder structure should be fully or partially standardized. We note that the option of not standardizing anything relating to the decoder structure was not captured in the WF. It is not clear in the WF what is meant by a partially standardized structure is, and of course it would need to be clear in order to decide on a partially standardized structure. The simulation campaign for option 4 is based on an agreed structure, which is in effect the fully standardized structure option.

The current investigation of option 4 assumes a fully standardized model structure.

For option 4, assume a fully standardized model structure.

## 4.3	Capturing of the model(s) in 3GPP

For exchanging models during the study phase, ONNX has been used, as this enables swapping of models independently of AI tools used by different companies. This is useful for a study phase, however for permanent capture of a test decoder in the specifications it may not be suitable. ONNX is an open source tool that is subject to variation between versions and that is not under the control of 3GPP. A 3GPP specification needs to be open access and not be subject to becoming invalid due to variations in the platform used for representing the model. The 3GPP specification should also not be tied to a specific AI/ML platform; both because it restricts implementation freedom and also because this would again link the 3GPP specification with a third-party tool.

Thus, there may be a need to develop a 3GPP specification method for a test decoder in the TS, with the ambition to handling other future AI/ML models.

The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed (for CSI compression and future models). Possible means to achieve this may be discussed if time allows.

The 3GPP representation may also need a process for updating of the test decoder. This may be handled by the CR and release process, although some thought may be needed on managing backward compatibility. For example, if encoders would not be directly compatible with an updated decoder, then a mismatch could occur between the version of a decoder assumed by the network and the version against which UE encoders have been trained; such mismatches should be avoided or handled by signalling.

The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

If it is agreed to capture the dataset associated with the test decoder then there will also be a need to agree on how to capture the dataset. This is a significantly less complex task than capturing a model, however; in principle the dataset format agreed as part of the SI has proved successful and could be used.

## 4.4	Identification of a dataset

For the simulation exercise, the dataset has been based on system simulation assumptions adopted by RAN1 during the performance study. Although in principle all of the datasets should be the same due to aligned simulation assumptions, in practice during the first round of simulations some differences between the characteristics of different datasets were observed, potentially due to different amplitude/phase normalization.

If a real test decoder and RAN4 testing is devised, it is important that it is based on a dataset that is realistic for the range of real-world situations that the model could encounter. The reason for this is that the test decoder will capture an available latent space that can be used by encoders. Also, RAN4 requirements need to cover a range of situations that ensure that a model can have a reasonable expectancy of working properly in real-world scenarios.

In order to achieve this, in a real work item, there would be a need to identify a dataset that contains a sufficient diversity of data. This is regardless of how many tests RAN4 develop for ensuring generalization; even if there would be no testing of generalization in RAN4 it would be good for the test decoder to capture a latent space that can be expected to be realistic for most deployment scenarios.

One way to create such a dataset would be to accumulate data from real world measurement. However accumulating such data would be non-trivial and very likely not possible within the 3GPP RAN4 context. Alternatively, a sufficient diversity of simulation conditions would need to be agreed. Care would also need to be taken that the simulated datasets with different companies could be aligned.

Conclude that, for a real WI, a sufficient diversity of simulation assumption sets would be needed to make the training dataset for the test decoder. Also, some steps to ensure alignment of different companies’ datasets are needed.

## 4.5	Comparison of RAN4 options and RAN1 inter-vendor collaboration directions

During the study item, RAN1 has considered potential inter-vendor collaboration options and identified so-called “directions”. RAN4 has considered means of achieving consistent and reproducible testing.

RAN1 currently discusses two main directions, “direction A” and “direction C”. The most similar direction to the RAN4 work is “direction C”:

Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

According to direction C, reference models are fully standardized. The purpose of fully standardizing reference models is to ensure interoperability between the network side and the UE side. Interoperability is highly important in achieving reasonable implementation complexity and is the basic purpose of developing a specification. If there is no or compromised interoperability, and individual vendors of network equipment and UEs have to interact then the development effort needed to get a feature to work increases exponentially.

RAN4 option 3 specifies a test decoder and reference encoder. The reason for specifying these is in effect to achieve interoperability between the test equipment and UE in order that testing can be repeatable. Since the standardized test decoder will capture a latent space, it will also define the performance of an encoder/decoder pair when deployed. By means of ensuring an interoperability with the test decoder, option 3 also ensures a basic interoperability during operation as long as the network side decoder also uses the same latent space.

During discussions at RAN4#114bis, there was some discussion whether the intentions behind the RAN1 direction C and RAN4 option 3 are the same, and also whether the RAN1 reference decoder/encoder would be the same as the RAN4 test decoder and reference encoder. There was also some suggestion that the RAN4 test decoder could be a “simple model”.

The RAN4 requirements should be as representative as possible of real world deployments; there is no point in defining requirements that are unrelated to actual conditions. The RAN4 test decoder needs to be sufficient to ensure that good performance can be achieved when testing to the RAN4 requirements, and thus cannot be over-simplified. Furthermore, since the test decoder will set the latent space used in actual operation, it should be designed and trained to relate to real operation as well as possible.

For this reason, it would not be useful to create some kind of “simplified” test decoder. Also, the goals of the RAN4 test decoder, to be useful for real conditions do not differ from the RAN1 reference encoder.

The RAN4 test decoder should not be a “simple” decoder; it should establish a latent space that is expected to be useful in real conditions and all requirement conditions.

Although the intentions may differ somewhat (a test decoder, compared to a reference for achieving interoperability), in principle the design goals for the standardized model are the same and in our understanding, RAN4 option 3 is the same as RAN1 direction C. Note that this has already been agreed in RAN1:

RAN1#116bis

Conclusion:

- Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors).

- It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4.

RAN1#120

Conclusion

- Direction C with standardized model ensures a minimum performance requirement.

  - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4.

Although the intentions for the models may differ between RAN1 option C and RAN4 option 3, the models to be captured/standardized would be the same.

RAN1 direction C and RAN4 option 3 models are the same.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Possibly more than one test decoder might be considered if there are requirements covering quite different scenarios; the need for different scenarios is part of the discussion on ensuring functionality generalization. Even with several requirement scenarios, whether multiple decoders would really be needed would need carefuly study.

Observation 2	If there would be several test decoders, then there would be at least one reference encoder per test decoder.

Observation 3	If lower complexity encoders are feasible with the same performance, then the reference encoder could simply be the lowest complexity encoder.

Observation 4	If there could be multiple performance levels for the same requirement scenario and decoder, possibly multiple reference encoders might be considered.

Observation 5	It is unusual for RAN4 to create several performance levels for the same requirement scenario.

Observation 6	If a dataset has been made available during the WI phase, it could be useful to retain the dataset as a reference when creating real encoders/decoders.

Observation 7	If a dataset has been made available during the WI phase then the question whether to keep the dataset is a question of retention policy on the 3GPP server.

Observation 8	It is not clear whether a test dataset or channel model is more useful in the specifications. This may depend on whether it can be expected that different implementations (by different TE vendors) of the channel model provide consistent performance.

Observation 9	It is highly likely that a dataset needs to be standardized for option 4b, considering experiences from option 3 track 1.

Observation 10	The current investigation of option 4 assumes a fully standardized model structure.

Observation 11	The RAN4 test decoder should not be a “simple” decoder; it should establish a latent space that is expected to be useful in real conditions and all requirement conditions.

Observation 12	Although the intentions for the models may differ between RAN1 option C and RAN4 option 3, the models to be captured/standardized would be the same.

Observation 13	There is no RAN4 equivalent for RAN1 direction A, although there may be some relevant learnings from the RAN4 studies.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For option 4a, capture a reference encoder in 3GPP.

Proposal 2	For option 4, assume a fully standardized model structure.

Proposal 3	The study on option 3 should consider that a 3GPP internal method for capturing test decoder AI models is needed (for CSI compression and future models). Possible means to achieve this may be discussed if time allows.

Proposal 4	The study should consider how a procedure for updating the test decoder would work, considering in particular whether updates should always be backward compatible or whether signalling of versions could be needed.

Proposal 5	Conclude that, for a real WI, a sufficient diversity of simulation assumption sets would be needed to make the training dataset for the test decoder. Also, some steps to ensure alignment of different companies’ datasets are needed.

Proposal 6	RAN1 direction C and RAN4 option 3 models are the same.