3GPP TSG-RAN WG4#115	R4-2507065

Malta, MT, May 19th – 23rd, 2025

Agenda item:	4.1.1

Source: 	Qualcomm France

Title: 	Discussion UL/DL bandwidths for overlapping NR SUL and NR bands

Document for:	Approval

# Introduction

Bandwidths of NR SUL band which overlap with NR bands are discussed along with channel confinement for n28-n83.

- Discussion

There are several SUL bands who match with NR FDD or TDD band frequencies.

Table 1 Overlapping NR SUL bands and NR bands

Asymmetric BW’s are defined in section 5.3.6 as follows for the NR bands above.

In current SUL band combination with inter-band CA in FR1, there is no mention whether NR SUL band and the NR band whose UL overlaps with SUL for combinations have same or different bandwidth. This leads to a lot of ambiguity and hence should be clarified in the specification. Asymmetric bandwidths table does not provide a lot of help either as there is very limited set, if any, defined for the band in Table 1.

For instance, in n28-n83 case, one extreme interpretation of the specification is that the UL could be 5MHz and DL could be 30MHz, or UL could be 30MHz and DL could be 5MHz. This would be quite illogical given n28 does not support asymmetric BW’s with the except of 3MHz UL/5MHz DL.

In table 7.3C.2-1 Supplementary uplink configuration for reference sensitivity, the UL-DL channel BW’s are same for SUL band as for the DL band, however having this restriction for REFSENS does not necessary mean that the BW’s were same for other than REFSENS configuration.

Observation 1: There is ambiguity in current SUL band combination with inter-band CA in FR1 on whether NR SUL band and respective overlapping NR band have same bandwidth or not.

Proposal 1: Clarify this ambiguity by adding note “Unless otherwise stated, the UL/DL channel bandwidths for overlapping SUL/DL bands are symmetrical.” into table 5.2C-4.

For case of n28-n83 there is no mention of Tx-Rx carrier center frequency separation elsewhere but in table 7.3C.2-1, which is is problematic because it only applies for REFSENS configuration. For instance, there is no mention on if n28 and n83 channels need to be both confined within lower or upper sub-band of n28 frequency range.

Observation 2: There is ambiguity in current SUL band combination with inter-band CA including both n28 and n83 on whether n28 and n83 must be both confined within same sub-band of n28.

Proposal 2: Clarify this ambiguity by adding note “Channels for both n28 and n83 are confined either to lowest 30MHz or to highest 30MHz of the band.” into table 5.2C-4 for combination including both n28 and n83 from Rel-17.

- Conclusion

Considerations on bandwidths of NR SUL band which overlap with NR bands were discussed along with channel confinement for n28-n83.

Proposal 1: Keep 3MHz optional across Releases for all bands except for band n106

Proposal 2: Keep or remove 5MHz CBW from band n41/n90

Proposal 3: Keep 35MHz n71 UL optional across releases unless otherwise agreed in discussions with interested parties

- Reference

[1] R4-2505214, “WF on handling optional channel bandwidth across releases”, Skyworks