3GPP TSG-RAN WG4#115	R4-2507070

Malta, MT, May 19th – 23rd, 2025

Agenda item:	5.3.3

Source: 	Qualcomm France

Title: 	MSD considerations related to handling of Optional bandwidths across releases

Document for:	Approval

# Introduction

MSD considerations related to handling of few optional bandwidths across releases is discussed.

- Background

WF was agreed in RAN4-114bis [1]:

WF#3 on MSD evaluation

Agreement: interested companies are invited to evaluate the following MSDs [1]

CA_n5A-n71A:

- Consider changing 1st test point to 2nd optional test point based on [2],

- Evaluate the following band n5 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | [FFS] | >ACLR2 |

CA_n71A-n85A:

PC3: Consider voiding footnotes 6 and 7 in Table 7.3A.6-1.

PC2: Consider voiding footnotes 4 and 5 in Table 7.3A.6-1a-1	.

CA_n77A-n102A, CA_n78A-n102A:  Evaluate the following PC3 cross-band isolation MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n102 | n77 | 5975 | 100 | 30 | 270 (RBstart=0) | 4195 | 10 | [FFS] | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [FFS] | >ACLR2 |

CA_n3A-n74A, CA_n3A-n75A:

- Consider the following change of 1st test point to 2nd optional test point based on [2].

- Evaluate the following band n74 and band n75 MSD requirements,

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | [FFS] | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | [FFS] | >ACLR2 |

- Discussion

MSD:

CA_n5A-n71A

Proposal 1: Consider the following MSD for CA_n5-n71 in case 35MHz becomes mandatory

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 871.5 | 5 | 4.0 | >ACLR2 |

CA_n71A-n85A:

Proposal 2: If 35MHz becomes mandatory, then footnotes in CA_n71A-n85A can be voided

CA_n3A-n74A, CA_n3A-n75A:

- Consider the following change of 1st test point to 2nd optional test point based on [2].

- Evaluate the following band n74 and band n75 MSD requirements,

Proposal 3: Consider the following MSD for CA_n3A-n74A and CA_n3A-n75A

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | 6.3 | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | 6.6 | >ACLR2 |

- Conclusion

MSD considerations related to handling of optional bandwidths across releases were provided with the following proposals.

Proposal 1: Consider the following MSD for CA_n5-n71 in case 35MHz becomes mandatory

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 871.5 | 5 | 4.0 | >ACLR2 |

Proposal 2: If 35MHz becomes mandatory, then footnotes in CA_n71A-n85A can be voided

Proposal 3: Consider the following MSD for CA_n3A-n74A and CA_n3A-n75A

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1730 | 40 | 15 | 50 (RBstart=0) | 1515.5 | 5 | 6.3 | >ACLR2 |
| n3 | n75 | 1730 | 40 | 15 | 50 (RBstart=0) | 1514.5 | 5 | 6.6 | >ACLR2 |

- Reference

[1] R4-2505214, “WF on handling optional channel bandwidth across releases”, Skyworks