3GPP TSG-RAN WG4#115	R4-2507071

Malta, MT, May 19th – 23rd, 2025

Agenda item:	7.1.2.1.1

Source: 	Qualcomm France

Title: 	MSD beyond PC3

Document for:	Approval

# Introduction

Considerations and proposals for HPUE MSD are provided in this contribution.

- WF from RAN4#114bis

# WF on 1 band 1 UL CC HPUE MSD Simplification

Agreement 1 on simplification Look-Up Tables (LUTs):

- Scope: In Rel-19, the 1 band UL 1CC HPUE MSD simplification only applies to PC2 1Tx, PC2 2Tx and PC1.5 achieved with 2Tx. Other Rel-19 1UL HPUE power classes are out of scope, e.g. PC1.

- For new Rel-19 inter-band CA combinations, and for all valid 1UL HPUE configurations, specify the 1 band 1 UL CC HPUE MSD requirements due to uplink harmonic, harmonic mixing, cross-band isolation caused by the same aggressor UL band as:

PCx MSD = PC31Tx MSD + MSD where,

- x = 2 1Tx, 22Tx, or 1.52Tx and,

MSD is specified in three LUT columns with a maximum MSD (“MSDmax”) of 3, 6 and 9dB for harmonic, cross band isolation, harmonic mixing. One example of such LUT is captured below. Coarser granularity and other tabulated values are not precluded.

| PC31Tx MSD (dB) | MSDmax (dB) |  |  |
| --- | --- | --- | --- |
|  | 3 | 6 | 9 |
| 0.1≤ ≤0.3 | 0.3 | 0.4 | 1.0 |
| 0.4≤ ≤0.6 | 0.5 | 0.8 | 1.8 |
| 0.7≤ ≤0.9 | 0.7 | 1.1 | 2.5 |
| 1.0≤ ≤1.9 | 1.2 | 2.1 | 4.1 |
| 2.0≤ ≤2.9 | 1.6 | 2.7 | 5.1 |
| 3.0≤ ≤3.9 | 1.8 | 3.3 | 5.9 |
| 4.0≤ - ≤4.9 | 2.0 | 3.8 | 6.5 |
| 5.0≤ - ≤6.9 | 2.3 | 4.5 | 7.3 |
| 7.0≤ - ≤9.0 | 2.5 | 5.0 | 7.9 |
| ≥10 | 3 | 6 | 9 |

- and where the mapping of the 1UL MSD type to the “MSDmax” LUT is specified with the following companion table.

| MSD type |  | MSDmax (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3 | 6 | 9 |
| 1UL | UL harmonic | PC21Tx | PC22Tx | [PC1.52Tx] |
|  | Harmonic mixing | PC21Tx | PC22Tx | PC1.52Tx |
|  | Cross-band isolation | PC21Tx | PC22Tx | PC1.52Tx |

- It should be noted that for some band combinations, LUTs may represent a slight tightening of the PC22Tx/PC1.52Tx MSD requirements, especially for UL harmonic, while for other band combinations LUTs may represent a slight relaxation of the PC22Tx/PC1.52Tx MSD requirements, especially for harmonic mixing.

- Agreement 2 on handling band combinations without MSD for PC3 but with small PC2/PC1.5 MSD:

- For new HPUE band combinations, do not specify the MSD requirements for 1UL HPUE PC21Tx PC22Tx and PC1.5 (2Tx).

- For legacy PC3 band combinations (i.e. band combinations specified prior to the agreement of the MSD simplification CR for 1UL HPUE), where no PC3 MSD is specified, companies are encouraged to browse through these band combinations and bring correction CRs if 1UL HPUE MSD requirements are needed.

Background on the companion table.

This table is needed for two reasons:

  - to anticipate future 1UL power classes and / or new MSD requirements. For example, at the time of writing, in Rel-19 there are no PC1.52Tx MSD requirements due to UL harmonic interference. Hence, no mapping exists for this case yet. In anticipation of these new 1UL CC HPUE MSD requirements, it is proposed to capture this mapping using square brackets.

  - to facilitate the possible introduction of a single “unified” mapping table for both 1UL and 2UL HPUE MSD simplification [9]. An extension of the 1UL companion table is anticipated to address 2UL mapping cases of Rel-19 different power classes.

# WF on 2 band 2 UL CC HPUE MSD Simplification

Agreement:

- 2UL HPUE MSD requirements can be mapped to a separate LUT with maximum of [8] columns, e.g. at least with ΔMSDmax = [3], 6, 9, 12, 15, [18, 24, 30] dB.

- Other values are not precluded and the need for additional columns is FFS.

FFS on optimized mapping to avoid penalizing 2UL combos with too high ΔMSDmax

- Discussion

1UL Cases:

We have our old proposal for 1UL LUT which is as follows. We are ok to further discuss the granularity at which the PC3 MSD is listed i.e. the number of lines for the LUT.

Proposal 1: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for further streamlining discussions.

Regarding the task on checking current 0dB PC3 MSD cases, we browsed through existing NR CA and EN-DC Cross-band MSD cases. We looked at which combinations do not have PC3 cross-band MSD specified and do not have Power class 2 specified. We looked at their max UL BW’s and the minimum frequency distance to victim DL. To limit the work we only picked cases whose minimum UL-DL distance is up to 5 times UL BW and which do not have PC3 MSD into the table below.

We note that all except two are LB-LB combinations, unless we missed some from the specified combinations which obviously is possible. Also, nothing prevents that there could be a case with more than 5x UL BW distance which might need MSD due to filtering characteristics (e.g. TDD bands).

The question is if MSD is to be added, when and to which power class is it added? It would be strange to add MSD into PC3 as for that it is not needed, but on the other hand it cannot be added to PC2 as that is not defined yet. On the other hand, if we add it only to PC2, then there is no straightforward derivation for PC1.5 MSD as only PC3 can be used as such a reference. Having said this, it seems rather unlikely that PC1.5 were used for these combinations anytime soon so that problem may be less of an issue.

Among the combinations above, there are several combinations for which FDD HPUE operation may be unlikely. As quick assessment, CA_n3A-n34A, CA_n5A-n12A, CA_n5A-n14A, CA_n5A-n29A, CA_n26A-n28A, and CA_n5A-n28A could be used in HPUE according to understanding of current regional rules. For these combinations, we have indicated that as [Yes] in the PC2 MSD needed? - column.

We propose that small MSD, e.g as shown in table below which matches the first row of our 1UL LUT can be added when HPUE operation is enabled.

Proposal 2: Add small MSD into CA_n3A-n34A, CA_n5A-n12A, CA_n5A-n14A, CA_n5A-n29A, CA_n26A-n28A, and CA_n5A-n28A at the same time when HPUE operation within these band combinations is enabled by removing notes in band combinations table

We made similar assessment to EN-DC combinations as well. We note that at least cases DC_13A_n71A and DC_40A_n41A require analysis as it looks MSD for PC3 should have been specified, at least for case when 35MHz wide n71 UL interferes band 13 DL.

Proposal 3: Evaluate PC3 MSD for DC_13A_n71A (NR aggressor) and DC_40A_n41A (LTE aggressor). Add small MSD into DC_2A_n66A, DC_3A_n40A, DC_5A_n28A, DC_12A_n5, DC_13A_n5A, DC_66A_n25A at the same time when HPUE operation within these band combinations is enabled.

Unfortunately, we also realized one issue that RAN4 probably has not looked into earlier, that is case when combination does not have PC3 MSD but does have PC2 MSD but PC1.5 is not defined. In this case there is no derivation mechanism for PC1.5 MSD unless we create one.

Observation 1: 1UL LUT is not currently covering PC1.5 MSD for combinations which have PC2 MSD specified but no PC3 MSD.

We propose to address this by adding wording associated with 1UL LUT in the specifications along the lines of “PC1.5 MSD for cases where only PC2 MSD exists is derived using 1UL LUT by looking at the line whose PC2 MSD matched with the specified PC2 MSD”

Proposal 4: To make 1UL LUT suitable also for cases which have only PC2 MSD specified in 38.101-1/3 PC2 MSD tables, the following wording is added associated with 1UL LUT in the specifications along the lines of “PC1.5 MSD for cases where only PC2 MSD exists is derived using 1UL LUT by looking at the line whose PC2 MSD matched with the specified PC2 MSD”

2UL Cases:

We propose to use simple format of MSD table without having different mapping for different combinations of TX lineup, instead mapping should be just for UE power class and IMD order. In our view, the whole purpose of this exercise has been to simplify the MSD definition process. Optimizing MSD numbers was not at least the main motivation.

We looked at how IMD’s go with different TX line-ups, using a simplified RFFE as an example with conventional assumptions in RFFE component linearities. In the figure below, black numbers in [] mean victim PA power at different point of RFFE and red numbers in [] mean aggressor PA power at different point of RFFE.

We measured PA IMD’s for few cases and calculated the IMD’s across the RFFE shown in example above. Please note that the PA measurements were not exhaustive and as such they should not be used as absolute values. We note that the PA dominates both IMD3 and IMD5 cases for PC3 but going to PC2 or especially PC1.5 the LNA and also to some extent Duplexer portion of the total interference increase and at PC1.5 the LNA clearly dominates the interference. At the end, all of this is a matter how RFFE is constructed and what kind of assumptions are used. In  the figure above and with the assumptions, we did not tweak the number to aggressive or pessimistic. We suggest that RAN4 should not spend time on trying to agree which component dominates at which conditions.

Proposal 5: RAN4 should not spend time deciding which component dominates the IMD in each case, instead RAN4 should just focus on finalizing the 2UL LUT

Comparing these number to our proposal for 2UL LUT in previous meeting our observations are as follows. For IMD3, the differences are in our view rather small. For IMD5 the differences vary. We note that this is just one example calculation, and the number may vary really a lot from this example. We don’t observe very big difference between case 2 and 3b, even in 3b the victim PA is PC2 PA backed off to PC3 power level. Even if the PA contribution were smaller than what we show in the example, the LNA IMD would dominate causing the interference to remain at same level.

Reflecting examples above, we observe that calculating MSD mapping separately for all possible cases in rigorous manner does not seem possible within Rel-19. Instead, we propose to using just one mapping which is based only on the PC3 MSD and PC2/PC1.5 IMD order. The mapping is hence agnostic to FDD+FDD, FDD+TDD, TDD+TDD or number of TX (1+1, 1+2, 2+1).

We agree that there are some differences in resulting MSD delta for different kind of TX line-ups (cases 1-10) but accounting the impacts of LNA and RFFE passives, the differences are in our view not work having different mapping for different TX lineups and for 2UL/2DL and 2UL/3DL. All HPUE MSD IMD requirements which are specified have been only accounting power increase and IMD order per band.

Observation 2: It is very difficult unless impossible to find good rules how interference for each case listed in WF behaves

Our main point in making just one 2UL LUT without mapping different TX lineups to different max delta columns is that we are genuinely afraid that there will be no consensus about the mapping and whole thing falls apart.. Instead, if needed, we are fine discussing how some high order IMD max deltas could be capped.

Proposal 6: Agree that 2UL LUT mapping is based only on PC3 MSD and PC2/PC1.5 IMD order.

Proposal 7: Use the following 2UL LUT mapping for all PC2 and PC1.5 UE power classes for all cases independent of number of TX per band and the power capability per single band

Our example numbers for LUT in Proposal 7 are the same as in previous meeting. As before, we are ok to discuss the granularity of PC3 MSD (number of lines).

Proposal 8: Use the following LUT for 2 UL cases for further streamlining discussions.

In case there is common drive to cap some high order IMD deltas, we would be ok discussing to cap some of the deltas to a lower values especially for higher IMD orders. We would not prefer to use different mapping e.g for PC1.5 with PC2+PC2 and PC3+PC1.5 because in that case only IMD4 with coefficients 1+3 and IMD5 with coefficients 1+4 and 2+3 would result in meaningfully lower max MSD delta.

Proposal 9: If needed, RAN4 could consider if some high order IMD deltas could be capped in Proposal 6/7

Finally, there are some cases in current PC1.5 MSD IMD specifications for 2UL/2DL and 2UL/3DL, which would be kind of “tight” compared to 2UL LUT where only UE power class increase and IMD order is accounted. This is because all current PC1.5 IMD cases are for 23+27.7dBm power set-up meaning the interference increase is smaller compared to if interference increase is calculated using 26+26dBm.  RAN4 should consider how to address these cases. We there are probably several options. One is to keep current Note stating the power per band configuration unchanged and use LUT for those combos only when PC2+PC2 is used. Another would be to take that note away and make MSD applicable for both 23+27.7 and 26+26, but that could be challenging especially for IMD4 and IMD5 for PC3 band.

Proposal 10: RAN4 should consider how to address PC1.5 MSD for 26+26dBm case for combinations which have PC1.5 IMD currently specified in 38.101-1 only using 23+27.7 dBm.

- Conclusion

Considerations on HPUE MSD were provided with the following observations and proposals.

Proposal 1: Use the following LUT for 1 UL cases (UL harmonics, Harmonic Mixing, Cross-band) for further streamlining discussions.

Proposal 2: Add small MSD into CA_n3A-n34A, CA_n5A-n12A, CA_n5A-n14A, CA_n5A-n29A, CA_n26A-n28A, and CA_n5A-n28A at the same time when HPUE operation within these band combinations is enabled by removing notes in band combinations table

Proposal 3: Evaluate PC3 MSD for DC_13A_n71A (NR aggressor) and DC_40A_n41A (LTE aggressor). Add small MSD into DC_2A_n66A, DC_3A_n40A, DC_5A_n28A, DC_12A_n5, DC_13A_n5A, DC_66A_n25A at the same time when HPUE operation within these band combinations is enabled.

Observation 1: 1UL LUT is not currently covering PC1.5 MSD for combinations which have PC2 MSD specified but no PC3 MSD.

Proposal 4: To make 1UL LUT suitable also for cases which have only PC2 MSD specified (i.e no PC3) in 38.101-1/3 PC2 MSD tables, the following wording is added associated with 1UL LUT in the specifications along the lines of “PC1.5 MSD for cases where only PC2 MSD exists is derived using 1UL LUT by looking at the line whose PC2 MSD matches with the specified PC2 MSD”

Proposal 5: RAN4 should not spend time deciding which component dominates the IMD in each case, instead RAN4 should just focus on finalizing the 2UL LUT

Observation 2: It is very difficult unless impossible to find good rules how interference for each case listed in WF behaves

Proposal 6: Agree that 2UL LUT mapping is based only on PC3 MSD and PC2/PC1.5 IMD order.

Proposal 7: Use the following 2UL LUT mapping for all PC2 and PC1.5 UE power classes for all cases independent of number of TX per band and the power capability per single band

Proposal 8: Use the following LUT for 2 UL cases for further streamlining discussions.

Proposal 9: If needed, RAN4 could consider if some high order IMD deltas could be capped in Proposal 6/7

Proposal 10: RAN4 should consider how to address PC1.5 MSD for 26+26dBm case for combinations which have PC1.5 IMD currently specified in 38.101-1 only using 23+27.7 dBm.

- Reference

[1] R4-2505143, “WF on MSD simplifcation for HPUE”, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE