3GPP TSG-RAN WG4 Meeting # 115	R4-2507074

# St Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.842 | CR | 0018 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AAS_BS_LTE_UTRA-Core) CR to TR 37.842 on correction of table reference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A non-existent table in TR 37.840 is being referred to in clause 5.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table reference in clause 5.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error remains and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 5.3.2	UE specific beamforming simulation assumptions

In case of UE specific beamforming, the AAS BS may generate  beams, each pointing to a specific UE. The overall radiation pattern of the  beams can be derived as:

where  is the element pattern defined in table 5.3.3.1-1, and and are the array factors defined in table 5.3.2-1.

Due to the non-linearization characteristics of the transmitters, the 3rd order intermodulation productions generated by the  beams may be pointed to different directions as the weighting vector for each of the beams are involved in the intermodulation process, and combinations of any three of the weighting vectors generated a different direction for the 3rd order intermodulation product. The overall radiation pattern of the IMD3 products can be derived as:

where is number of IMD3 beams generated, and . Please be noted that the triplet (p, q, l) is order sensitive and they represent any of the three UE specific beams with an intermodulation relation of p+q- l. Please also be noted that p, q, l can take the same value. Therefore:

=

and  is the weighting vector for the IMD3 products generated by the three beams and:

=

The weighting factor above is for the IMD3 products  which is generated by+-, where , , and  represent any frequency component in beam p, q, l correspondingly.

Based on the radiation pattern of wanted signal and IMD3 products, the ACLR pattern can be derived as:

+

where is the ACLR performance of each transmitter.

The assumption in table 5.4.3.1.4-1 in 3GPP TR 37.840 [2] was used for UE beamforming scenarios but the number of UEs activated in each cell is assumed to be 4. A 10x4 antenna array is employed to generate beams pointed directly at each UE dropped in cell.

# Table 5.3.2-1: Simulation assumptions for UE specific beamforming

| Parameters | Values |
| --- | --- |
| Cellular layout | Hexagonal, 3 sectors/cell (19 cell wrap-around), uncoordinated |
| UE distribution | Average 10 UEs per sector |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| Inter Site Distance (ISD) | 750m |
| Minimum distance UE<->BS | 35m |
| Log normal shadowing | Standard Deviation of 10 dB |
| Shadow correlation coefficient | 0.5 (inter site)/1.0 (intra site) |
| Scheduling algorithm | Round Robin, Full buffer |
| RB number per active UEs | DL: 12RBs |
| Number of active UEs | DL: 4UEs |
| UE max Tx power | 23 dBm |
| UE min Tx power | -40 dBm |
| Active array loss | 1 dB |
| Losses of legacy system | 2 dB |
| Legacy BS max Tx power | 46 dBm |
| AAS BS max Tx power | 46 dBm |
| Antenna configuration at UE | Omni-directional |
| The height of BS | 30 m |
| The height of UE | 1.5 m |
| Antenna array configuration (Row×Column) | 10×4 |
| ACS of LTE UE | 33 dB |
| Performance evaluation | Throughput loss criteria, as derived from the truncated Shannon bound approach of 3GPP TR 36.942 [5] |

<End of change>