3GPP TSG-RAN WG4 Meeting # 115	R4-2507078

# St Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0064 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Perf) CR to TS 38.181 on corrections of notes in tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) Reference to Note 2 in Table 4.7.2-1 is missing. 2) Note ‘1’ is missing in Table 10.3.5-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Align reference to Note 2 in Table 4.7.2-1. 2) Add Note ‘1’ in Table 10.3.5-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2, 10.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 4.7.2	Test signal used to build Test Configurations

The signal's channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to tables 4.7.2-1 and 4.7.2-2.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. |  |  |

Table 4.7.2-2: Signal to be used to build NR TCs for SAN type 2-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth shall be used. |  |  |

<Next change>

#### 10.3.5.2	Test requirements for SAN type 1-O

For each measured carrier, the throughput measured in step 9 of clause 10.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in tables 10.3.5.2-1 and 10.3.5.2-2.

The reference sensitivity level requirements for NB-IoT are specified in clause 10.3.2 of TS 36.181 [23].

Table 10.3.5-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -98.0 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -98.0 - ΔOTAREFSENS (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -98.1 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -95.2 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -91.6 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -91.6 - ΔOTAREFSENS (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -91.9 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -92.0 - ΔOTAREFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

Table 10.3.5-2: SAN LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -101.1 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -101.1 - ΔOTAREFSENS (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -101.2 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -98.3 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -94.7 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -94.7 - ΔOTAREFSENS (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -95.0 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -95.1 - ΔOTAREFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

<End of change>