3GPP TSG-RAN WG4 Meeting # 115	R4-2507084

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	7.13.4.2.3

Source: 	Nokia

Title: 	Simplification of BS TRP test methods

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#106 [1]. The objective of this WI on simplification of BS TRP test methods is revised as follow:

- TRP simplification: Simplify BS TRP estimation methods (e.g. Annex I in TS 38.141-2 on TRP measurement procedures) for FR1 and FR2 by improving their applicability and description and not removing any of the existing TRP test methods.

This topic was discussed at TSG RAN4#114bis [2] and the WF was agreed [3]. This contribution provides proposals to progress this topic in RAN4.

# 2.	Discussion

2.1	Procedure wise clarification for different TRP measurement

It was agreed in the WF [3]:

- For OTA base station output power, ACLR and operating band unwanted emissions, use the following option 1 as starting point. - Option 1: - Step 1: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Beam-based directions as described in annex I.10 may be performed if chosen for the test, go to step 2 if this step is not chosen or the BS under test fails the test in this step. - Step 2: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test. - For OTA base station spurious emission, use the option 1 as starting point - Option 1: - Step 1: Pre-scan as described in annex I.13 if chosen for the test, go to step 2 if this step is not chosen. - Step 2: [Orthogonal cut grid] (two-cuts and optional three cuts) as described in annex I.5 or Peak method as described in annex I.11 or Equal sector with peak average as described in annex I.12 may be performed if chosen for the test, go to step 3 if this step is not chosen or the BS under test fails the test in this step. - Step 3: One of the TRP measurement procedure as described in annex I.2 to I.9 except annex I.5 shall be performed for the test.

Note that orthogonal cut grid was put into square brackets because of the comments that it should only be used with clearly specified conditions to avoid underestimation of the TRP values.  For example, a list of conditions is specified in annex I.5.1 of TS 38.141-2 [4] to apply pattern multiplication when two cuts measurements are used:

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication: i. The vertical cut (and the main beam) is in the xz -plane ii. The frequency of the emission is within the downlink operating band. iii. The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal iv. The emission appears/disappears when the Tx power is turned on/off. v. The antenna arrays of the EUT 1) Have rectangular grids of antenna element positions 2) Have symmetry planes that are vertical and horizontal. 3) Have parallel antenna planes

However, it is not clearly specified in annexes I.5.2 and I.5.3 of TS 38.141-2 how to determine the TRP test outcome (pass/fail) from the TRP estimate. The current text (as extracted below) seems to indicate the test is failed if the TRP estimate is above the specified TRP limit.

## I.5.2 Operating band unwanted emissions The procedure is as follows: 1) Follow steps described in annex I.5.1 for the first two mandatory cuts and calculate the TRPEstimate. 2) Compare the TRPEstimate to the limit. 3) If the TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 2. ## I.5.3 Spurious unwanted emissions The procedure is as follows: 1) Follow steps described in annex I.5.1 for two cuts and calculate the preliminary TRPEstimate. 2) Add the appropriate correction factor ΔTRP according to table I.5.3-1 to ensure overestimation with 95% confidence. 3) Compare the corrected TRPEstimate (including ΔTRP) to the limit. 4) If the corrected TRPEstimate is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3. Table I.5.3-1: The correction factor for two or three cuts dense sampling Three cuts; Two cuts. Correction factor ΔTRP (dB); 2.0; 2.5.

On the other hand, a detailed flowchart-like procedure to determine the TRP test outcome was proposed in [5], which would help to clarify how to determine the TRP test outcome from the obtained measurements. Therefore, it is proposed to improve the current text in annex I.5 of TS 38.141-2 to clearly specify the conditions of application, e.g., measurement grid size, and determination of test outcome when orthogonal cut grid is used.

2.2	The improvement of Pre-scan approach

It was agreed in the WF [3]:

- Encourage companies to provide the draft CR focusing on how to improve the test procedures for Pre-scan approach next meeting and further discuss the improvement based on draft CR directly.

Note that pre-scan approach is currently described in annex I.13 of TS 38.141-2 as follows:

# I.13 Pre-scan Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP. An emission frequency identified by a pre-scan may be further investigated by any of the TRP measurement methods in this annex. The procedure for pre-scan is as follows: 1) Scan the entire surface around EUT. 2) Rotate test antenna to cover all possible polarizations of emissions to detect maximum emissions. 3) Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur. 4) Emissions which 20 dB or more below the specified limit shall not require further measurements.

Note that the current text is quite simple and missing the conditions of application, like the measurment grid to be used or the placment of the EUT and measurement antenna during the measurement procedure, as well as the calculation of the TRP estimate from the obtained measurements. Therefore, it is proposed to improve the current text on pre-scan in TS 38.141-2 to clearly specify the conditions of application and calculation of the TRP estimate when pre-scan approach is used.

2.3	Suggestion for improvement

The notes in Table 6 of ERC/REC 74-01 [6] (extracted below) can be used as a starting point for improvement in annex I of TS 38.141-2:

Table 6: Spurious domain emission limits for the land mobile service, maritime mobile service and short range devices

| Reference number | Type of equipment | Frequency of the spurious domain emission f | Limits: mean power or, when applicable, average power during bursts duration in the reference bandwidth |
| --- | --- | --- | --- |
| 2.1.1 | Terminals and Base Stations (in transmit mode), except the equipment specified below | 9 kHz f 1 GHz | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.2 | Short range devices; RLAN; CB; Cordless Telephones; Radio Microphones (all systems in transmit mode) | f within the bands: 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 9 kHz f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm |
| 2.1.3 | Short range inductive devices operating below 30 MHz (in transmit mode) (Note 3) | 9 kHz f 10 MHz | 27 dBA/m (at 9 kHz then decreasing by 10 dB/decade) (Note 4) |
|  |  | 10 MHz < f 30 MHz | -3.5 dBA/m (Note 4) |
|  |  | f within the bands: 87.5-118 MHz, 174-230 MHz, 470-694 MHz | -54 dBm |
|  |  | 30 MHz < f 1 GHz (except 87.5118 MHz, 174230 MHz, 470694 MHz) | -36 dBm |
|  |  | 1GHz < f Fupper (see recommend 3) | -30 dBm |
| 2.1.4 | Receivers and idle/standby transmitters except the equipment specified below | 9 kHz f 1 GHz | - 57 dBm |
|  |  | 1 GHz < f Fupper (see recommends 11) | - 47 dBm |
| 2.1.5 | Short range inductive receivers and idle/standby transmitters operating below 30 MHz | 9 kHz ≤ f < 4.78 MHz | 5.5 dBµA/m, decreasing by 3 dB/octave (Note 4) |
|  |  | 4.78 MHz ≤ f < 30 MHz | -22 dBµA/m (Note 4) |
|  |  | 30 MHz ≤ f < 1 GHz | -57 dBm |
| 2.1.6 | Base Stations using AAS and beamforming with integrated antennas operating below 6 GHz (Note 5) | 9 kHz f 1 GHz | -36 dBm(Note 6) |
|  |  | 1 GHz < f Fupper (see recommends 3) | -30 dBm (Note 6) |
| 2.1.7 | Base Stations using AAS and beamforming with integrated antennas operating above 24.25 GHz (Note 5) | 9 kHz f 1 GHz | -36 dBm (Note 5) (Note 6) |
|  |  | 1 GHz < f 18 GHz | -30 dBm (Note 5) (Note 6) |
|  |  | 18 GHz < f Fupper (see recommends 3) | -20 dBm/10 MHz (other limits apply for specific frequency separations, see Figure 7) (Note 6) |
| 2.1.8 | Terminals operating above 24.25 GHz using AAS and beamforming with integrated antennas | 9 kHz f 1 GHz | -36 dBm (Note 6) |
|  |  | 1 GHz < f 7.25 GHz | -30 dBm (Note 6) |
|  |  | 7.25 GHz < f Fupper (see recommends 3) | -13 dBm/MHz and -10 dBm/100 MHz (Note 6) (Note 7) (Note 8) |
| Note 1: ƒ is the frequency of the spurious domain emission Note 2: For systems that use digital modulation and narrow-band high power (1 Watt) analogue modulated systems, the reference bandwidth is specified in section 2 of this annex, while for any other analogue modulation the reference bandwidth specified in recommend 4 is applicable. Note 3: Limits for Wireless Power Transmission for Electric Vehicles systems are under study and may need further consideration. Note 4: Levels are H-field limit at 10 m distance, measured by shielded loop antenna as specified by CISPR. Note 5 : There is currently no information on base stations using AAS and beamforming with integrated antennas operating between 6 and 24.25 GHz. These could be considered in a future revision as necessary. Note 6: For Terminals and Base Stations using AAS and beamforming with integrated antennas, the metric for unwanted emission is defined as Total Radiated Power (TRP): and applies separately for the transmitting and receiving phases of operation. where PD(r,,) is the power density in W/m2 at a distance r of two orthogonal polarizations. Over-the-air spatial parameters are specified in a Cartesian coordinate system (x, y, z) using spherical coordinates (r,,). is the angle in the x/y plane and it is between the x-axis and the projection of the vector onto the x/y plane and is defined between -180° and +180°.  is the angle between the projection of the vector in the x/y plane and the vector and is defined between -90° and +90. The TRP measurement procedure for emissions in the spurious domain with large frequency separation is slightly different from that of the wanted signal and unwanted emissions closer to the wanted signal. For the wanted signal, if the directivity is known, it is enough to measure the peak e.i.r.p. and adjust it with the directivity to obtain TRP. Care must be taken for using this method for unwanted emissions since its directivity can be different from the wanted signal directivity, in particular at large frequency separations from the carrier. If the directivity is not known but it can be verified that the emission is from the antenna array, TRP can be obtained by measuring e.i.r.p. in the cardinal cuts and using pattern multiplication to extrapolate the e.i.r.p. values in other directions. Pattern multiplication exploits the fact that the array factor of a rectangular array can be separable in two factors along the two symmetry planes. If none of the above options are applicable, the final option is to measure the e.i.r.p. on a full-sphere grid with the reference angular step. The measured values can then be integrated to obtain the TRP. All the above options are accurate methods for TRP assessment provided that the mentioned pre-conditions are fulfilled. For spurious emissions at larger frequency separations from the carrier, it is important to first identify the frequencies with notable emissions for further measurement. This is achieved through a pre-scan which can be performed on a very sparse grid. If no notable emission is identified at a certain frequency during the pre-scan, no further investigation is needed for that frequency. For frequencies which are identified for further measurement, a number of orthogonal cuts (two or three) or a sparse spherical grid can be used. These methods are based on the assumption that spurious emissions at larger frequency separation are not fully correlated and are less directive. For spurious emissions, an upper bound assessment for TRP is enough to ensure the compliance and the exact TRP value is of less importance. Therefore, a systematic correction factor is added to the estimated TRP value in order to ensure an overestimate with a certain confidence. The systematic correction factor is dependent on the chosen grid type and angular step. Orthogonal cut measurements should be measured with the reference angular steps. The sparse full sphere measurement can be performed on angular steps larger than the reference step and smaller than 15 degrees. A larger angular step results in a shorter measurement time while it imposes a larger systematic correction. For studies, the TRP needs to be reflected in terms of conducted power (P). The link between conducted power and TRP is based on antenna efficiency that could be transformed in terms of additional losses (L). The relation between these terms are P=TRP+L. (L needs to be defined with a positive number) Note 7: Both limits are simultaneously applicable. Note 8: Limits in the frequency band 8.5-10.5 GHz might be reconsidered, if necessary when this Recommendation is revised. |  |  |  |

# 3.	Conclusion

This contribution has provided proposals to progress the objective of the WI for TRP simplification in RAN4, the proposals are summarized as follows.

Proposals:

1)	To improve the current text in annex I.5 of TS 38.141-2 to clearly specify the conditions of application and determination of test outcome when the orthogonal cut grid is used.

2)	To improve the current text in annex I.13 of TS 38.141-2 to clearly specify the conditions of application and calculation of the TRP estimate when pre-scan approach is used.

3)	To use the notes in Table 6 of ERC/REC 74-01 as a starting point for improvement in annex I of TS 38.141-2.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC.

[2]	R4-2504629, “Topic summary for [114bis][304] NR_BS_RF_Part3_OTA_TRP”, ZTE.

[3]	R4-2504740, “Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP”, ZTE.

[4]	3GPP TS 38.141-2 v19.0.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[5]	R4-2501647, “TS 38.141-2 Annex I: Discussion on improvement of TRP measurement methods”, Ericsson.

[6]	ERC/REC 74-01, “ERC/REC 74-01 of 1998 on unwanted Emissions in the Spurious Domain corrected May 2022 (SE#91)”.