3GPP TSG-RAN WG4 Meeting # 115	R4-2507085

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:	7.28.4

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#105 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#114bis [2] and the WF was agreed [3].

This contribution provides proposals to progress the open issues according to the agreed WF.

# 2.	Discussion

2.1	How to specify the BS manufacturer declaration for LP-WUS

It is stated in the agreed WF as follows:

- WF - Manufacturer shall declare the support of LP-WUS for each supported band, [frequency range] and channel bandwidth. - If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +[X]dB. - X can be different for different channel bandwidth. - FFS on the value of X and use 3 as starting point.

For the [frequency range], BS manufacturers are allowed to declare the support of a certain frequency range within an operating band instead of the whole band. This is specified in TS 25.141 [4], TS 36.141 [5], TS 38.141-1 [6], and TS 38.141-2 [7] as extracted below. This flexibility should continue to be allowed if LP-WUS is supported by the BS. Therefore, BS manufacturers shall declare the support of LP-WUS for each supported band, frequency range and channel bandwidth.

TS 25.141:

## 4.8 Specified frequency range The manufacturer shall declare: - which of the frequency bands defined in sub-clause 3.4 are supported by the BS. - the frequency range within the above frequency band(s) supported by the BS.

TS 36.141:

### 4.6.1 Operating band and frequency range The manufacturer shall declare which operating band(s) specified in clause 5.5 that is supported by the BS under test and if applicable, which frequency ranges within the operating band(s) that the base station can operate in. Requirements for other operating bands and frequency ranges need not be tested.

TS 38.141-1:

| D.3 | Operating bands and frequency ranges | List of NR operating band(s) supported by single-band connector(s) and/or multi-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Declarations shall be made per antenna connector for BS type 1-C, or TAB connector for BS type 1-H. | x | x |
| --- | --- | --- | --- | --- |

TS 38.141-2:

| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | c | x | x |
| --- | --- | --- | --- | --- | --- |

On the other hand, if BS manufacturers declare to support LP-WUS power boosting, it is sensible that a meaningful LP-WUS power boosting level should be declared, in which case a minimum of +3dB (which is also the minimum level that BS manufacturers may declare for NB-IoT power boosting [5, 6]) seems to be a reasonable level. Note that the declared level can be different for different channel bandwidth and in this case the minimum level shall apply to each declared level.

2.2	Whether to define EVM requirements for LP-WUS

It is stated in the agreed WF as follows:

- Options - Option 1: No need to define EVM requirements for LP-WUS. (Huawei) - Because simulation results show that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirements do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. (Nokia) - Because frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR. (ZTE) - Option 2: EVM requirements for LP-WUS is needed. (CMCC) - Option 2-1 (CATT): - RAN4 to specify EVM requirement only for “ON” chips of OOK symbols. - RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence. - RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%. - RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence. - Option 2-2 (Ericsson): - Define the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS as 8%. - Wait further RAN1 progress on PAPR discussion. - WF - Further check in next meeting whether EVM for LP-WUS needs to be specified.

The simulation results provided in [8] have shown that the impact on missed detection error (MDR) is minor for EVM up to 30%as such the EVM requirement do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. Therefore, there is no need to specify the EVM requirement for LP-WUS/LP-SS.

2.3	Whether/how to verify EVM requirements for LP-WUS

It is stated in the agreed WF as follows:

- Options - Option 1: There is no need to do the EVM verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS. (Nokia) - Option 2: Without specific EVM requirement defined for LP-WUS, just use a higher-order EVM test for NR with embedded LP-WUS as the verification metric. (Huawei) - Option 3: The OOK4 signal quality test can be dependent on the 64QAM EVM test. (Ericsson) - Option 4: Others. - WF - Consider together with EVM requirement for LP-WUS - If no EVM requirement is defined, no verification is needed - If EVM requirement is defined, FFS on whether verification is needed.

The simulation results provided in [8] have also shown that the impact on overall PAPR (at different percentiles) is minor (around 0.2 dB) when LP-WUS is transmitted together with the QAM signals for NR. The opening point on PAPR discussion at TSG RAN4#114bis was whether the same conclusion can apply to LP-SS transmission. On LP-SS design, the following agreements have been made at RAN1#120bis [9]:

Agreement For the length L of LP-SS binary sequence, supports - M=1, L= {6,8} - M=2, L= {12,16} - M=4, L= {16,32} Note: For each value of M, UE supports both L values – no UE capability. Note: There is no consensus in RAN1 that SNR of -3dB can be fulfilled with the smaller value of L for each M. Definition of the proper requirements for different values of L is up to RAN4. Agreement For LP-SS with M=4, supports set1 (balanced).

Note that with the (balanced) agreement for LP-SS with M=4, the overall PAPR of LP-SS will be the same as that of LP-WUS, and thus the simulation results provided in [8] remain valid for LP-SS transmission. Therefore, there is no need to do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

# 3.	Conclusion

This contribution has provided proposals to progress the issues according to the agreed WF in last RAN4 meeting.

Proposal 1: BS manufacturers shall declare the support of LP-WUS for each supported band, frequency range and channel bandwidth.

Proposal 2: If the BS supports LP-WUS power boosting, manufacturer shall declare the LP-WUS power boosting level it could support with a minimum of +3dB which shall apply to each declared level if different levels are declared for different channel bandwidths.

Proposal 3: There is no need to specify the EVM requirement for LP-WUS/LP-SS.

Proposal 4: There is no need to do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

# References

[1]	RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2504633, “Topic summary for [114bis][308] NR_LPWUS”, Huawei.

[3]	R4-2504744, “Way Forward for [114bis][308] NR_LPWUS”, Huawei.

[4]	3GPP TS 25.141 v18.0.0: “Base Station (BS) conformance testing (FDD)”.

[5]	3GPP TS 36.141 v19.0.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[6]	3GPP TS 38.141-1 v19.0.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[7]	3GPP TS 38.141-2 v19.0.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[8]	R4-2503168, “BS RF requirements for low-power wake-up signal for NR”, Nokia.

[9]	R1-2503100, “Summary of RAN1 agreements on LP-WUS/WUR for NR”, vivo.