3GPP TSG-RAN WG4 Meeting #115                                                     R4- 2507091

Malta , MT, 19th – 23st, May. 2025

Source:		Nokia

Title:	NR-NTN HPUE RF requirements for Tx

Agenda item:		7.10.3.2.1

Document for:	Discussion

# Introduction

Based on the agreement in the WF from RAN4 #114-bis meeting [1], in this paper, we’d like to present our view in NTN HPUE co-existence related issues.

# Topic #1: RF requirements ### Sub-topic 1-1: Power classes and UE types < Issue 1-7-2: PC1 ACLR Tentative agreement: - PC1 ACLR is [34] dBc - Companies to further evaluate this, and to be confirmed in next meeting.

# Discussion

  - Co-existence issue for PC1 ACLR

PC1 ACLR definition for NTN UE is related to the co-existence between TN and NTN. The main concern from the TN operators is the interference/impact to the existing TN network. The baseline is that the operation of NTN UEs should not impact TN work performance, or the impact can be neglected.

In the current 38.101-1 (Rel-18), the ACLR for PC1 is defined as 37 dB, which was a reuse of the simulation results based on PC2 UE, vehicle type UE in TS 36.837. However, this is very different from the simulation assumption and results presented by different companies based on PC1 handheld and non-handheld UEs in R4-2419642 [2] and [4]. We consider the current simulation results in Rel-19 are more relevant.

The simulation assumption in Rel-19 for PC1 NTN UE is very different from the simulation assumption for TN network in the current standard. The simulation results in Rel-19 for PC1 NTN UE are more relevant to solve the co-existence issue for TN and NTN.

Based on the simulation results presented by different companies, the range of ACLR for PC1 co-existence study with TN network, is between 20 to 35 dB based on 1.5 km isolation distance at 5%-tile TP loss and is between 20 to 40/45 dB based on Zero isolation distance. The current 37 dB PC1 ACLR is within the range of simulation results for Zero isolation distance at 5%-tile TP loss. Due to the difference of simulation tools, UE model, antenna model, and so on that are used in different companies, the simulation results from all the companies should cover all the situations for PC1 NTN UE used in the field. Therefore, the average value of simulated ACLR can be considered to be defined in the Rel-19 standard.

Suggest defining the PC1 ACLR requirements based on the average value of simulation results related to 5%-tile TP loss with Zero isolation distance.

We have presented our view in the previously submitted Tdoc [5] that the physical TN cell coverage is irregular shape based on the map. It follows the residential area, industrial area, and other populations physical distribution. Therefore, it is very difficult to guarantee the isolation distance in the real deployment of TN and NTN networks. Concerning the NTN UE may travel into the TN network coverage area, and it should be free will for humans to use the NTN UE, therefore, we want to clarify that should the requirement be defined for the NTN UE working outside of TN network only? Should Tx of NTN UE be disabled when NTN UE travels into the physical locations of TN cell coverage. Or NTN UEs can be still used but need to adjust the Tx transmission and follow all the TN specifications when it moves into the physical coverage of TN cells.

It is very difficult to deploy TN and NTN network in reality to guarantee the isolation distance between real TN coverage and real NTN coverage.

Clarify if the NTN UE can be used within the physical coverage of TN network or the Tx of NTN UE should be disabled when NTN UE travels into the physical locations of TN cell coverage.

Clarify whether the requirement only applies for NTN UE outside of TN network coverage based on Zero isolation distance. Also Clarify what requirement should be applied for NTN UE if it can be used within the coverage of TN.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

- Observation 1: The simulation assumption in Rel-19 for PC1 NTN UE is very different from the simulation assumption for TN network in the current standard. The simulation results in Rel-19 for PC1 NTN UE are more relevant to solve the co-existence issue for TN and NTN.

- Proposal 1: Suggest defining the PC1 ACLR requirements based on the average value of simulation results related to 5%-tile TP loss with Zero isolation distance.

- Observation 2: It is very difficult to deploy TN and NTN network in reality to guarantee the isolation distance between real TN coverage and real NTN coverage.

- Proposal 2: Clarify if the NTN UE can be used within the physical coverage of TN network or the Tx of NTN UE should be disabled when NTN UE travels into the physical locations of TN cell coverage.

- Proposal 3: Clarify whether the requirement only applies for NTN UE outside of TN network coverage based on Zero isolation distance. Also Clarify what requirement should be applied for NTN UE if it can be used within the coverage of TN.

# References

- R4-2417116, WF on UE RF requirements for MIMO enhancement

- R4-2419642, Discussion on coexistence studies and results (Samsung)

- R4-2417122, WF on co-existence study and HPUE for NR-NTN

- R4-2504331, Simulation assumption for co-existence study

- R4- 2501347, NR NTN HPUE (Nokia)