3GPP TSG-RAN WG4 Meeting #115                                                     R4- 2507092

Malta , MT, 19th – 23st, May. 2025

Source:		Nokia

Title:	IoT-NTN HPUE RF requirements for Tx

Agenda item:		7.10.3.3.1

Document for:	Discussion

# Introduction

Based on the agreement in the WF from RAN4 #114-bis meeting [1], in this paper, we’d like to present our view in IoT NTN HPUE SAR related issues.

# SAR Related issues ### SAR Related issues Issue 2-1-2: Whether and how to introduce duty cycle related method. - Proposals (Not necessarily mutually exclusive): - Scheme 1: (vivo) Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful. - Scheme 2: (oppo) UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2. - Scheme 3: (Samsung) Observation 3-1: Legacy dutycycle method does not fit the purpose of keeping as much power as possible for NTN scenario, as legacy solution requires UE to fall back 3/6 dBs, which is the power of entire power class. Observation 3-2: In Rel-19, NTN would not introduce any beam service ratio in time domain in NTN phase3 WI in parallel. Observation 4-1: To let UE drop certain ratio of uplink opportunities will face the same challenge as other duty cycle method, because there is no NW uplink ratio in time domain in a FDD NTN band. It means if we follow this way, there is no way we could specify or to guide UE behaviors, like how much ratio of uplink should UE drop, and for how long. Observation 4-2: To let UE go into a Tx UL Gap also faces same challenges as above. Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method, considering following: 1) P-MPR had been working well and leave space for implementation; 2) without uplink duty ratio from NW side, current solutions on the table (including legacy tdd duty cycle solution) cannot be optimized and clear enough for UE implementation; 3) There are many non-3GPP technologies already implemented in actual SAR compliance, and it helped to maintain power level well enough. - Scheme 4: (Qualcomm) - A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.” - For X and Y, consider value range from 100ms to 5s with 100ms granularity. - Starting time for the indicated values is the transmission time of the report. - Send an LS to RAN2 to specify energy headroom reporting. - Allow but do not mandate UE to reduce it’s maximum output power when Tx duty cycle exceeds 50% in a radio frame for PC2 and 2Tx PC1.5. Capture this allowance into specification. - Further evaluate potential specification impact from filter power handling for PC1. - Scheme 5: (Ericsson, Sony, Nokia) Observation 1: “Duty-cycle” based method is not well designed for FDD bands and is allegedly not used in the field. P-MPR method is used instead, which is on another hand “blind” for the network. Observation 2: “Duty-cycle” based and the P-MPR based methods in combination with reporting of the actual power-capability state of a UE could be considered for NTN HPUEs, similar to Rel-18 enhancements introduced for TN. Proposal 1: Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. Proposal 2: If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony) - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance: - DPC = ‘00’: the criteria to report DPC is not met - DPC = ‘01’: ΔPPowerClass = 0 dB - DPC = ‘10’: ΔPPowerClass = 3 dB - DPC = ‘11’: ΔPPowerClass = 6 dB - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c): - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB - Scheme 6: (ViaSat, Inmarsat) Observation 4: In a non-terrestrial network, if the UE is using a UL duty cycle significant enough to exceed SAR limits, it is likely at the edge of coverage (using a low MCS with lots of parity), in which case, transmit power backoff is not a preferred option as it will almost invariably lead to radio link failure. Proposal 3: For NTN, extend the existing TN signalling mechanism for UL duty cycle reporting to inform the RAN that it is close to the SAR limit, then the RAN will reduce the number of UL slots allocated to the UE. When the SAR limit has fallen, the UE can signal the improved condition back to the RAN so that the UL slots are picked back up. - WF - Further discuss the proposed schemes related to duty cycle related SAR scheme. - Other schemes or no duty cycle related scheme are not precluded.

# Discussion

  - SAR related issue

The SAR issue is common for high-power handheld devices. As the radiation power becomes higher and higher, it is highly likely that the human body will be exposed to radio frequency radiation if there is no solution to avoid it, and longtime exposure may be harmful to the human body.

There has been discussion and different opinions of the SAR issue for the high-power NTN IoT handheld device. Dropping the call, MPR reduction, legacy duty cycle, and new “duty cycle” scheme, etc. were presented and collected in the last meeting. In our view, dropping the Tx connection is not an optimized solution; the new “duty cycle” proposal may be hard to agree; and adding too much MPR will invalid the benefit of high-power UE.

Our view is that combining P-MPR and legacy duty cycle can be an optimal way to solve this issue. Based on the “maxUplinkDutyCycle-x" capabilities in the current 38.306, there is no restrict to TDD or FDD to support duty cycle for NTN/TN, and the required duty cycle has to be decided by RAN4.

However, based on the offline discussion, there may not be FDD configurations from TN/NTN network to support legacy duty cycle at the moment due to lack of use case before. We also notice that there is no duty cycle capability defined for PC1 in the current 38.306. So, a new capability to support PC1 can be created which will not impact the previous releases.

Send information LS to RAN2 for the definition of duty cycle to support PC1 NTN HPUE for FDD use cases. Create a capability “maxUplinkDutyCycle-PC1-Rel19” to indicate UE’s maximum tolerated percentage of scheduled uplink symbols to ensure the SAR compliance in NTN (TN) for both FDD and TDD configurations.

Approve the LS in the attachment

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Proposal 1: Send information LS to RAN2 for the definition of duty cycle to support PC1 NTN HPUE for FDD use cases. Create a capability “maxUplinkDutyCycle-PC1-Rel19” to indicate UE’s maximum tolerated percentage of scheduled uplink symbols to ensure the SAR compliance in NTN (TN) for both FDD and TDD configurations.

Proposal 2: Approve the LS in the attachment

# References

- R4-2505114, WF_Iot_NTN_HPUE

# Annex (LS to RAN2)

3GPP TSG-RAN WG4 Meeting #115                                                        R4-25xxxx

Malta , MT, 19th – 23st, May. 2025

Title:	LS on duty cycle configuration for Rel-19 NTN HPUE

Response to:	-

Release:	Rel-19

Work Item:	NR_IoT_NTN_req_test_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Yi Tan

yi.1.tan@nokia.com

# 1	Overall description

The SAR issue is common for high-power handheld devices, and duty cycle is an effective method to control SAR issues. Based on the RAN4 discussion, combining P-MPR and legacy duty cycle is an optimal method to solve SAR issue for IoT NTN PC1 HPUEs.

In the current TS 38.306, there is no “maxUplinkDutyCycle-x" capabilities to address the duty cycle capability for NTN/TN PC1 UEs. Therefore, RAN4 would like to create a capability “maxUplinkDutyCycle-PC1-Rel19” to indicate UE’s maximum tolerated percentage of scheduled uplink symbols to ensure the SAR compliance in NTN (TN) for both FDD and TDD configurations.

# 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN2 to consider the above information.

# 3	Dates of next TSG RAN WG 4 meetings

3GPPRAN4#116		        		25 – 29 Aug 2025		    	India (TBC) , IN

3GPPRAN4#116-bis		        	13 – 17 Oct, 2025				Prague , CZ