3GPP TSG-RAN WG4 Meeting #115                                                      R4-2507093

Malta , MT, 19th – 23st, May. 2025

Source:		Nokia

Title:	UE RF requirements

Agenda item:		7.24.3

Document for:	Discussion

# Introduction

Based on the agreement in the WF from RAN4 #114-bis meeting [1], in this paper, we’d like to present our view in MIMO_ph5 related issues.

# <Topic 1> FR1 ## <Sub-topic 1-1> 3Tx power class <Conclusion>: PC1.5 for 3Tx MIMO - There is no consensus for PC1.5 3Tx in RAN4 in Rel-19. ## <Sub-topic 1-2> 3Tx MPR <Agreement>: PC2 MPR for 3Tx - Reuse 2Tx PC2 MPR

# Discussion

  - 3Tx power class1.5 UE and PA architect to support 2 layers MIMO

The discussion for the power class 1.5 and PA architect to support TxD and multiple layers MIMO (2 layers) has not been agreed. In this Tdoc, we present our view on this issue.

In the 114-bis meeting, the options for PC1.5 3Tx UE to support multiple layers MIMO are as below:

  - Option 1: Include PC1.5 in 3Tx

    - 3Tx UE can support PC1.5 via 3Layer or single antenna port and without ULFTPx modes in Rel-19 (OPPO)

    - RAN1 does not generally address power class or transparent TxD (there is no transparent TxD in the RAN1 spec), so TxD was not precluded (or explicitly supported) for PC1.5 for ‘3Tx’ topic in this WI. (Ericsson)

    - Discuss whether 2Tx TxD can be enabled for 3Tx, and whether this is a valid scenario for 3Tx to be defined. (vivo)

  - Option 2: Exclude PC1.5 from 3Tx

    - As 3Tx UE cannot output 29dBm for all supported transmit precoder matrices as well as DCI format 0_0 it cannot declare support for PC1.5. (Qualcomm)

    - It was still challenged offline that a 2Tx TxD, even if allowed, is a fallback mode of 3Tx thus not suitable to be verified or even defined under 3Tx scope. (vivo)

    - It may be better not to rush for a wider spec implementation for 3Tx in this release especially considering how many limitations had been set during the WI establishing phase (Huawei/HiSilicon)

    - Considering the limitations of this WI, it is reasonable for RAN4 to consider PC1.5 in future releases for 3Tx. (Samsung)

  - Option 3: Up to TxD support for 3Tx

In our view, it is technically possible to support 2 MIMO layers for 3Tx PC1.5 UE. The architecture of 3 Tx PAs to support up to 2 layers can be illustrated as below examples:

- The 1st layer can be achieved by 2 port/2PA ULFP mode1, and the 2dn layer can be achieved by 1 port/1PA ULFP mode 0

- 3Tx downgrade to 2Tx, and both ports/PAs can achieve ULFP mode 0 for PC1.5 power level

Based on our understanding of the RAN1 discussion, ULFP mode 0/1/2 were discussed in the previous meetings. Though in the LS from RAN1[2], the ULFP mode0 should be supported, however, the latest agreement, there is no ULFP mode specified, even ULFP mode0. Therefore, there is a space for 3Tx UE to support up to 2 layers with implementation of agnostic PA architecture.

The PA architecture for 3Tx UE can implementation agnostic support 2 layers MIMO.

Suggest specifying 3Tx PC1.5 UE to support up to 2 layers MIMO.

In Rel-17, HPUE for NR FR1 FDD band n77 and n78 for power class 1.5 has been introduced, which covered smartphones and fixed wireless access (FWA) devices. However, we understand the concern that PC1.5 handheld devices may face power/battery issues and/or formfactor constraints support 2 layers MIMO. Therefore, it is reasonable to only mandate PC1.5 non-handheld UE to support 2-layer MIMO.

PC1.5 handheld devices may face power/battery and/or formfactor constraints supporting 2 layers MIMO.

Specify PC1.5 to support up to 2 layers MIMO at least for non-handheld UE in this release.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

- Observation 1: The PA architecture for 3Tx UE can implementation agnostic support 2 layers MIMO.

- Proposal 1: Suggest specifying 3Tx PC1.5 UE to support up to 2 layers MIMO.

- Observation 2: PC1.5 handheld devices may face power/battery and/or formfactor constraints supporting 2 layers MIMO.

- Proposal 2: Specify PC1.5 to support up to 2 layers MIMO at least for non-handheld UE in this release.

# References

- R4-2505111, WF on UE RF requirements for MIMO enhancement phase 5

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO