3GPP TSG-RAN4 Meeting #115	R4-2507099

Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | Draft | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Ku-band VSAT OTA Reference Sensitivity and Maximum Input Level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add OTA Reference Sensitivity and Maximum Input Level requirements for Ku-band VSAT |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For OTA Reference Sensitivity Add requirements for bands n248 & n247 and n509 & n508 into tables 10.3.2-1 and 10.3.2-2 - For Maximum Input Level Add requirements for bands n248 & n247 and n509 & n508 into table 10.4.2-1. Modify the table format of table 10.4.3-1 to align table formatting with modifications made to table 10.4.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | OTA Reference Sensitivity and Maximum Input Level requirements are missing for Ku-band VSAT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2, 10.4.2, 10.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

# 10	Radiated receiver characteristics

## 10.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA) at the RIB for Ka bands fixed and mobile VSAT. The power level for all DL wanted signals and interference is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 10.2	Polarization characteristics

The minimum requirements on the receiver characteristics apply under either LHCP (Left Hand Circular Polarization) or RHCP (Right Hand Circular Polarization) or Linear Polarization.

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the QPSK reference measurement channels as specified in Annexes A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

Figure 10.3.2-1 Example measurement grid for EIS with the declared supported minimum elevation angle

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, [30], 35, 50, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | SCS (kHz) | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | 60, 120 | ≤ -122 |
|  |  | 3 | 60, 120 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | 60, 120 | ≤ -122 |
| n248, n247 | Mobile VSAT | 4, 5 | 15, 30 | [≤ -122] |
| n509, n508 | Mobile VSAT | 4, 5 | 120 | [≤ -122] |

## 10.4	Maximum input level

### 10.4.1	General

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

### 10.4.2	Minimum requirement for Mobile VSAT

For mobile VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1 with parameters specified in Table 10.4.2-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.2-1: Mobile VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
|  | n248 n247 | 4, 5 | 15, 30 | dBm | [-114.9] (NOTE 5) | Not applicable |
|  | n509 n508 | 4, 5 | 120 | dBm | Not applicable | [-108.9] (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 NOTE 5: Reference measurement channel is specified in Annex A.3.2.1.1: QPSK with R= 0.3 and 16 QAM with R= 0.48 |  |  |  |  |  |  |

### 10.4.3	Minimum requirement for Fixed VSAT

For fixed VSAT, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1 with parameters specified in Table 10.4.3.-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.4.3-1: Fixed VSAT maximum input level

| Rx Parameter | Band | VSAT type | SCS (kHz) | Units | Channel bandwidth (MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5, 10, 15, 20, 25, 30, 35, 50, 100 | 50, 100, 200, 400 |
| Power in transmission bandwidth configuration | n512 n511 n510 | 4, 5 | 60, 120 | dBm | Not applicable | -109.6 (NOTE 2, 3, 4) |
| NOTE 1: The transmitter shall be set to [4 dB] below the PUMAX,f,c as defined in sub-clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. NOTE 2: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: QPSK with R=1/3 NOTE 3: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 16QAM with R=1/2 NOTE 4: Reference measurement channel is specified in Annex A.3.2.1.2 and A.3.2.1.3: 64QAM with R=1/2 |  |  |  |  |  |  |

< END OF CHANGE #1 >