3GPP TSG-RAN WG4 Meeting #115	R4-2507103

St Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.10.5.1

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE tests

Document for:	Discussion

# 1	Introduction

RAN4#114bis agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1].

This contribution discusses the remaining open issues such as propagation delay modeling and accuracy of propagation delay and Doppler shift.

# 2	NGSO NTN test environment overview

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 is an overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite position/velocity (ephemeris) information and UE’s location information at a given time. In the DUT, the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The UE uses the estimated Doppler shift and propagation delay to perform the pre-compensation before the baseband processing.

In the test environment, the TE needs to update the Doppler shift and propagation delay once per 1ms. The TE applies the updated Doppler shift and propagation delay to the transmit signals before the fading simulator, but the TE does not need to align the updating timing with the slot boundary.

To derive the Doppler shift and propagation delay, the TE need to calculate the satellite position/velocity based on the satellite propagator model. TE may use the Kepler’s equation or equation of motion. TE should update the satellite position/velocity every Tsat (sec), for example, every 1.024 seconds. During the period between the satellite potion updates, the TE may calculate the Doppler shift and propagation delay by interpolation once per 1ms. TE can choose the interpolation algorithm as far as the accuracy of Doppler shift and propagation delay is satisfied. The UE and TE should share the same UE location information to derive the Doppler shift and propagation delay.

During the test, the TE requires to generate the satellite ephemeris information specified in SIB19 ephemerisInfo-r17 for NR or SIB31 ephemerisInfo-r17 for IoT-NTN. TE needs to update the satellite ephemeris information and send to the DUT together with epochTime every Tsib (sec), where Tsat < Tsib. RAN4 assumes the ephemeris information is same as the satellite position/velocity calculated from the propagator model used for the Doppler shift and propagation delay calculation.

Figure 2 summarizes the update timing of the satellite ephemeris information, that is, ephemerisInfo, Doppler shift and propagation delay.

Figure 2	Update timing of satellite position/velocity, epehmerisInfo, Doppler shift, and propagation delay.

| Parameters | Unit | Values |
| --- | --- | --- |
| Tsat | sec | 1.024 |
| Tsib | sec | 10.24 for RF and demodulation tests 2.56 for RRM tests |

# 3	Discussion

## 3.1	Propagation delay

RAN4#114bis discussed whether to the earth rotation for propagation delay calculation.

FFS to consider the Earth’s rotation during signal flight time for more accurate estimations of propagation delay Agreement - Consider the earth rotation during signal flight time impact into TE-emulated channel modelling - Take the proposals from option 1 or option2/4 for DL propagation delay calculation - Companies are encouraged to cross check the equations to derive the total propagation delay and its error. - Note: The details of option 1, option 2 and option 4 can be referred as R4-2503710, R4-2503247 and R4-2504344, respectively - Not consider the earth rotation for doppler shift calculation

We show all the options are identical. If we consider the satellite transmits the signal at time tT and UE receives this signal at time tR, the propagation delay is the difference tR - tT. If both the satellite and UE do not move, the satellite delay is simply derived from the distance between the UE position at tR (UEtRECEI) and the satellite position at tT (rtTECEI), that is,

Delay=tR-tT=UEtRECEF-rtTECEFc

where c is the speed of light. However, we need to consider the earth rotation when we drive the propagation delay, as illustrated in Figure 3 (a).

| (a) Illustration of signal path | (b) Propagation delay estimated at satellite at time tT | (b) Propagation delay estimated at UE at time tR |
| --- | --- | --- |

Figure 3	Signal path from the time the satellite transmits signal at tT to the time the UE receives the signal at tR.

RAN4#114bis discussed two options to derive the propagation delay: one option is based on satellite transmission time; another option is based on UE reception time.

Figure 3 (b) illustrates how to derive the propagation delay based on the satellite transmit time. As explained in Appendix D.1.1, Option 1 calculates the propagation delay estimated at the satellite transmission time tT:

Option 1:

DtTSat=ρtT ECEF/c1-1cvtTUE,ECI⋅ρtT ECEFρtT ECEF

On the other hand, Figure 3 (b) illustrates how to derive the propagation delay based on the UE receive time. As explained in Appendix D.1.2, Options 2 and 3 calculate the propagation delay estimated at the UE reception time tR:

Options 2/3:

DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

The difference between the two options is the time at the propagation delay estimation.

Since DtTSat is the propagation delay estimated at the satellite at transmit time tT, from DtTSat=tR-tT,  the signal receiving time at the UE is given by:

tR=tT+DtTSat

This means the propagation delay estimated at the signal transmitting time at the satellite tT is equal to the propagation delay estimated at the signal receiving time at UE, that is,

DtTSat=DtT+DtTSatUE

Figure 4 illustrates DtTSat is equal to DtT+DtTSatUE.

Figure 4	Explanation of DtTSat=DtT+DtTSatUE.

Figure 5 shows the difference between DtTSat and DtT+DtTSatUE with LEO-600 scenario starting from 30 degrees. The figure shows the difference between DtTSat and DtT+DtTSatUE is very small (< 1ns).

Figure 5	Difference of propagation delays between DtTSat and DtT+DtTSatUE.

Observation 1: Propagation delay options 1 and 2/3 are equivalent.

Since the purpose of the NGSO channel model is to simulate the propagation delay observed at UE, TE need to apply the propagation delay at the UE reception time, i.e., tR. For this purpose, we propose to use Options 2/3 to derive the propagation delay.

Proposal 1: Use Option 2/3 (based on the UE reception time tR) to calculate the propagation delay.

## 3.2	Accuracy of interpolated Doppler shift and propagation delay

Another open issue is the required accuracy of interpolated Doppler shift and propagation delay.

To derive accuracy requirement, - Baseline assumption - number of iterations is fixed to 4 for solving Kepler’s equation based on Newton-Raphson method - Calculate the Propagation delay and Doppler shift every 1ms - For accuracy requirement - Apply linear interpolation to calculate the propagation delay and Doppler shift with 1ms granularity as baseline, no restrict the implementation on the TE side - Consider the propagation delay error is less than TBD (s) and Doppler shift error is less than TBD (Hz) - The time period to calculate the error is 1s - TE should meet the minimum accuracy requirement defined. - Companies are encouraged to provide the acceptable value with associated propagation delay error and Doppler shift error calculation method for accuracy requirement in the next meeting

We have considered the following assumptions to investigate the required accuracy at TE.

Baseline Doppler shift and propagation delay:

- TE runs the satellite propagator (Keplerian model) with the maximum iteration of 4, every 1ms.

- TE calculates Doppler shift Dopplerbaseline(t) and propagation delay Delaybaseline(t) every 1ms based on Appendix A.2.2.

Interpolated Doppler shift and propagation delay:

- TE runs the satellite propagator (Keplerian model) with the maximum iteration of 4, every 1 second (1000ms).

- TE calculates Doppler shift propagation delay every one second, based on Appendix A.2.2.

- TE calculates Doppler shift, Dopplerinterpt, and propagation delay, Delayinterpt, every 1ms by linear interpolation.

We have compared the difference between two methods every 1 second by the mean absolute error between baseline value and interpolated values, that is, t = 0, 1, 2, … (seconds).

ErrDopplert=11000n=11000Dopplerbaselinet+n1000-Dopplerinterpt+n1000

ErrDelayt=11000n=11000Delayrbaselinet+n1000-Delayrinterpt+n1000

Figure 6 and Figure 7 show the Doppler shift/propagation delay errors for LEO-600 and LEO-1200 scenarios, respectively. For both the scenarios, we set the initial elevation angle to 30 degrees.

Figure 6	Accuracy of Doppler shift (Hz/s) and propagation delay (µs/s) for LEO-600 with initial elevation angle of 30 degrees.

Figure 7	Accuracy of Doppler shift (Hz/s) and propagation delay (µs/s) for LEO-1200 with initial elevation angle of 30 degrees.

The comparison shows that the maximum mean absolute error of LEO-1200 is smaller than that of LEO-600 because of the smaller change rate of LEO-1200. Considering margin, we propose to set the Doppler shift error should be less than 1 (Hz/s) and the propagation delay error should be less than 0.03 (µs/s).

Proposal 2: Doppler shift calculation accuracy at TE should be less than 1 Hz/s.

Proposal 3: Propagation delay calculation accuracy at TE should be less than 0.03 µs/s.

## 3.3	Initial ephemeris information for LEO-600

RAN4#114bis discussed how to derive the ephemeris information for the LEO-600 scenario. Since RAN5 has not specified the ephemeris information for LEO-600 with the initial elevation angle of 30 agrees, we have used GMAT [2] to derive the satellite position/velocity vector corresponding to the elevation angle of 30 degrees. Table 1 shows the outputs from GMAT, where we referred to TS 38.521-5 Table 6.4.1_2.4.3-1 LEO-600 satellites (maximum positive Doppler) to set the initial value in GMAT.

Table 1	Ephemeris information for LEO-600.

| Elevation angle (degree) | positionX (step of 1.3 m) | positionY (step of 1.3 m) | positionZ (step of 1.3 m) | velocityVX (step of 0.06 m/s) | velocityVY (step of 0.06 m/s) | velocityVZ (step of 0.06 m/s) |
| --- | --- | --- | --- | --- | --- | --- |
| 10.004 | -2717617 | 4550419 | 852799 | 6164 | -19424 | 124281 |
| 10.103 | -2717331 | 4549520 | 858535 | 6230 | -19540 | 124259 |
| … | … | … | … | … | … | … |
| 29.814 | -2655356 | 4388896 | 1587549 | 14507 | -34352 | 120231 |
| 30.059 | -2654685 | 4387308 | 1593097 | 14569 | -34465 | 120190 |
| 30.307 | -2654011 | 4385715 | 1598644 | 14631 | -34578 | 120149 |

Since RAN4 agreed with the elevation angle more than 30 degrees, we propose to use the position/velocity vectors highlighted in yellow.

Proposal 4: RAN4 sets the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654685	, 4387308, 1593097, 14569, -34465, 120190), which corresponds to (-3451.0905 km, 5703.5004 km, 2071.0261 km, 0.87414 km/s, -2.0679 km/s, 7.2114 km/s).

# 4	Summary

Observations:

Observation 1: Propagation delay options 1 and 2/3 are equivalent.

Proposals:

Proposal 1: Use Option 2/3 (based on the UE reception time tR) to calculate the propagation delay.

Proposal 2: Doppler shift calculation accuracy at TE should be less than 1 Hz/s.

Proposal 3: Propagation delay calculation accuracy at TE should be less than 0.03 µs/s.

Proposal 4: RAN4 sets the initial ephemeris information for LEO-600 with (positionX, positionY, positionZ, velocityVX, velocityVY, velocityVZ) = (-2654685	, 4387308, 1593097, 14569, -34465, 120190), which corresponds to (-3451.0905 km, 5703.5004 km, 2071.0261 km, 0.87414 km/s, -2.0679 km/s, 7.2114 km/s).

# References

- R4-2504700, “Way Forward for [114bis][323] NTN_testing_NGSO_channel_model_and_demod”, Samsung.

- General Mission Analysis Tool, https://sourceforge.net/projects/gmat/

# Appendix

## A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

### A.1.1	Keplerian model based estimation

#### Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

##### Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0ECI=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0ECI=v0,xECIv0,yECIv0,zECI=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

##### Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0ECI=r0,xECI2+r0,yECI2+r0,zECI2

v=v0ECI=v0,xECI2+v0,yECI2+v0,zECI2

vr=v0ECI⋅r0ECIr=r0,xECIv0,xECI+r0,yECIv0,yECI+r0,zECIv0,zECIr

h=hxhyhz=r×v=r0,yECIv0,zECI-r0,zECIv0,yECIr0,zECIv0,xECI-r0,xECIv0,zECIr0,xECIv0,yECI-r0,yECIv0,xECI

h=h=hx2+hy2+hz2

##### Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

##### Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω corresponds to ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

##### Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0ECI-r0ECI⋅v0ECIv0ECI=1μv2-μrr0,xECIr0,yECIr0,zECIT-r⋅vrv0,xECIv0,yECIv0,zECIT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

##### Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

##### Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0ECIe⋅r=cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr≥02π-cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

#### Step 2	Determine the satellite position and velocity at time t (sec)

##### Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

##### Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If n≥4, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n :=  n+1.

##### Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

##### Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

### A.1.2	Equation of Motion based estimation

##### Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if updating the satellite position/velocity every one second.

##### Step 2	Solve the equation of motion with the fourth-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

where,

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

##### Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

## A.2	Varying Doppler shift and propagation delay generation

### A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is set by a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since the UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

### A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

ρtECEF=ρt,xECEFρt,yECEFρt,zECEF=rt,xECEF-UEt,xECEFrt,yECEF-UEt,yECEFrt,zECEF-UEt,zECEF

ρtECEF=ρt,xECEF2+ρt,yECEF2+ρt,zECEF2

RangeRate(t)=ρtECEF⋅vtECEFρtECEF=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFρtECEF

Doppler(t)=-RangeRate(t)×fcc

vtECI=vt,xECIvt,yECIvt,zECI=vtECEF+00ωE×rtECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

Delayt=ρtECEF/c1+1cvtECI⋅ρtECEFρtECEF

### A.2.3	Elevation angle and Azimuth angle (for information)

The elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t is:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEF⋅e=ρt,xECEF⋅ex+ρt,yECEF⋅ey+ρt,zECEF⋅ez

lt,n=ρtECEF⋅n=ρt,xECEF⋅nx+ρt,yECEF⋅ny+ρt,zECEF⋅nz

lt,u=ρtECEF⋅u=ρt,xECEF⋅ux+ρt,yECEF⋅uy+ρt,zECEF⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is:

Azt=180πtan-1lt,elt,n

## B.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.3182 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 | -3603.0046 (km) |  |
| positionY-r17 | 4509243 | 5862.0159 (km) |  |
| positionZ-r17 | 2455572 | 3192.2436 (km) |  |
| velocityVX-r17 | 20837 | 1.25022 (km/s) |  |
| velocityVY-r17 | -46176 | -2.77056 (km/s) |  |
| velocityVZ-r17 | 109528 | 6.57168 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

Table 7.6.2.1-3a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 | -3816.2176 |  |
| positionY-r17 | 4895460 | 6364.098 |  |
| positionZ-r17 | 1188458 | 1544.9954 |  |
| velocityVX-r17 | 8506 | 0.51036 |  |
| velocityVY-r17 | -23314 | -1.39884 |  |
| velocityVZ-r17 | 118253 | 7.09518 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

## B.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 | -2859.0536 (km) |  |
| positionY-r17 | 3404229 | 4425.4977 (km) |  |
| positionZ-r17 | 3535794 | 4596.5322 (km) |  |
| velocityVX-r17 | 35394 | 2.12364 (km/s) |  |
| velocityVY-r17 | -74414 | -4.46484 (km/s) |  |
| velocityVZ-r17 | 94682 | 5.68092 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

## C.1	UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

## D.1	Propagation delay

### D.1.1	Propagation delay based on the satellite transmission time (Option 1)

In this option, we modify UEtRECEF-rtTECEF=c ⋅DtTSat with the relation: UEtRECEF=UEtTECEF+vtTUE,ECI⋅DtTSat. That is,

UEtTECEF+vtTUE,ECI ⋅DtTSat-rtTECEF=c ⋅DtTSat

We should emphasize the UE velocity vectors, vtTUE, ECI, shall be inertial reference frame, given by:

vtTUE,ECI=vtTUR,ECEF+00ωE×rtTUR,ECEF=00ωE×rtTUR,ECEF

Note we assume the UE is stationary in ECEF, that is, vtTUR,ECEF=0.

By defining, ρtT ECEF:=UEtTECEF-rtTECEF

ρtT ECEF+vtTUE,ECI⋅DtTSat=c⋅DtTSat

c⋅DtTSat2=ρtT ECEF+vtTUE,ECI⋅DtT2≈ρtT ECEF2+2vtTUE,ECI⋅ρtT ECEF⋅DtTSat

By taking a square root of the equation,

c⋅DtTSat=ρtT ECEF2+2vtTUE,ECI⋅ρtT ECEF⋅DtTSat

=ρtT ECEF1+2vtTUE,ECI⋅ρtT ECEFρtT ECEF2⋅DtTSat

≈ρtT ECEF1+vtTUE,ECI⋅ρtT ECEFρtT ECEF2⋅DtTSat=ρtT ECEF+vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat

Finally, we get the equation below.

c⋅DtTSat≈ρtT ECEF+vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat

Solve the above equation by DtT,

c-vtTUE,ECI⋅ρtT ECEFρtT ECEF⋅DtTSat=ρtT ECEF

DtTSat=ρtT ECEFc-vtTUE,ECI⋅ρtT ECEFρtT ECEF=ρtT ECEF/c1-1cvtTUE,ECI⋅ρtT ECEFρtT ECEF

### D.1.2	Propagation delay based on the UE receiption time (Options 2/3)

In this option, we modify UEtRECEF-rtTECEF=c ⋅DtRUE with the relation: rtRECEF=rtTECEF+vtRECI⋅DtRUE. That is,

UEtRECEF-rtRECEF-vtRECI⋅DtRUE=c⋅DtRUE

We should emphasize the satellite velocity vectors, vtRECI, shall be inertial reference frame, given by:

vtRECI=vtRECEF+00ωE×rtRECEF

By defining, ρtR ECEF:=rtRECEF-UEtRECEF

c⋅DtRUE=UEtRECEF-rtRECEF+vtRECI⋅DtRUE

c2DtRUE2=UEtRECEF-rtRECEF+vtRECI⋅DtRUE2

c⋅DtRUE2=-ρtRECEF+vtRECI⋅DtRUE2=ρtRECEF-vtRECI⋅DtRUE2

≈ρtRECEF2-2vtRECI⋅ρtRECEF⋅DtRUE

By taking a square root of the equation,

c⋅DtR=ρtRECEF2-2vtRECI⋅ρtRECEF⋅DtRUE

=ρtRECEF1-2vtRECI⋅ρtRECEFρtRECEF2⋅DtRUE

≈ρtRECEF1-vtRECI⋅ρtRECEFρtRECEF2⋅DtRUE=ρtRECEF-vtRECI⋅ρtRECEFρtRECEF⋅DtRUE

Finally, we get the equation below.

c⋅DtRUE≈ρtRECEF-vtRECI⋅ρtRECEFρtRECEF⋅DtRUE

Solve the above equation by Δt,

c+vtRECI⋅ρtRECEFρtRECEF⋅DtRUE=ρtRECEF

DtRUE=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF