3GPP TSG-RAN WG4 Meeting #115	R4-2507108

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.27.5

Source:	Ericsson

Title:	Demodulation requirements for NR network energy saving enhancements

Document for:	Discussion

# 1	Introduction

Rel-19 NR network energy saving enhancement includes three main objectives [1]:

- On-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA.

- On-demand SIB1 for UEs in idle/inactive mode.

- Adaptation of common signal/channel transmissions in time domain.

  - Adaptation of SSB, PRACH and paging occasion.

This contribution shows our view on the UE and BS demodulation requirements for Rel-19 NES enhancements.

# 2	Discussion

## 2.1	On-demand SSB (OD-SSB)

This feature allows gNB to transmit SSB when it is necessary. According to the RAN1 agreements, Rel-19 NES-supporting gNB triggers OD-SSB in SCell 1) before the gNB sends SCell activation command, or 2) at the SCell activation.

The SSB-less SCell operation is not a new feature for NR. Rel-15 NR supports the SSB-less SCell operation for intra-band contiguous CA, and Rel-18 NR supports the SSB-less SCell operation for inter-band co-located CA. Rel-19 OD-SSB is also applicable only for SCell, but it is applicable for any band combination and any deployment scenario.

Since RAN4 did not define CA demodulation requirements for Rel-15 and Rel-18 SSB-less SCell operations, we do not think RAN4 needs to define new CA PDSCH demodulation requirements for Rel-19 OD-SSB.

## 2.2	On-demand SIB1 (OD-SIB1)

In this feature, RAN1 defines two types of cells: NES Cell and Cell A.

- NES cell: Cell supporting on-demand SIB1 (OD-SIB1) transmission. This means the NES cell usually does not transmit SIB1, and it transmits SIB1 when it is necessary.

- Cell A: Assisting cell that periodically transmits its own SIB1.

RAN1 has introduced the uplink wake-up signal (UL WUS) to request the NES Cell to transmit OD-SIB1. Rel-19 NES-capable UE (FG 61-5 [2]) sends the UL WUS via Rel-15 RACH Msg1, that is, PRACH.

According to the OD-SIB1 activation procedure, Rel-19 NES-capable UE first camps on Cell A and acquires the UL WUS configuration from Cell A. RAN1 agreed that the UL WUS configuration is same as the Rel-15 PRACH resource configuration. Once the Rel-19 NES-capable UE camps on the NES Cell, the UE can transmit the UL WUS to the NES cell to acquire OD-SIB1. When NES Cell receives the UL WUS, it sends random access response (RAR) as same as the existing random access procedure, then it transmits OD-SIB1. Figure 1 illustrates an overview of OD-SIB1 transmission procedure.

Figure 1	Overview of OD-SIB1 transmission procedure.

From the UE demodulation requirement point of view, RAN4 does not need to define new PDSCH demodulation requirements because OD-SIB1 uses PDSCH.

From the BS demodulation requirements point of view, RAN4 does not need to define new PRACH demodulation requirements because the UL WUS uses PRACH.

## 2.3	Common signal/channel transmission adaptation

### 2.3.1	SSB adaptation

This feature allows the gNB to configure multiple SSB burst patterns on SCell, e.g., one pattern with 160ms periodicity and another pattern with 5ms periodicity. Then gNB can switch (adapt) the SSB burst pattern according to e.g., the SCell activation. RAN1 agreed to trigger the SSB burst periodicity for SCell by DCI format 2_9 (FG 61-6 [2]).

Since the adaptation is for SCell only, we do not think RAN4 needs new UE demodulation requirements as well as OD-SSB on SCell.

### 2.3.2	PRACH adaptation

This feature allows the gNB to adapt the PRACH resources in time domain by additional RACH resources in RRC idle/inactive/connected mode (FG 61-7 [2]). RAN1 agreed that gNB configures the additional PRACH resource via RRC, and gNB triggers the configured additional PRACH resource via DCI 1_0. RAN1 also agreed that the additional PRACH resource configuration is same as the Rel-15 PRACH configuration.

Since RAN1 agreed to reuse the Rel-15 PRACH configurations to configure the additional PRACH resource, and to use DCI 1_0 to trigger the additional PRACH resources, we do not think RAN4 need new UE and BS demodulation requirements.

### 2.3.2	Paging occasion adaptation

This feature introduces the condensed paging resources in time-domain. This is the IDLE mode procedure, and it does not affect to UE demodulation requirements.

## 2.4	Summary

Based on our observations, RAN4 does not need to define new UE and BS demodulation requirements in Rel-19 NES enhancements.

Proposal 1: RAN4 does not define new UE demodulation and CSI reporting requirements for Rel-19 Netw_Energy_NR_enh.

Proposal 2: RAN4 does not define new BS demodulation requirements for Rel-19 Netw_Energy_NR_enh.

# 3	Summary

Proposal 1: RAN4 does not define new UE demodulation and CSI reporting requirements for Rel-19 Netw_Energy_NR_enh.

Proposal 2: RAN4 does not define new BS demodulation requirements for Rel-19 Netw_Energy_NR_enh.

# References

- RP-250343, “Revised WID on enhancements of network energy savings for NR”, Ericsson.

- R1-2502977, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”, RAN1.