3GPP TSG-RAN WG4 Meeting #115	R4-2507109

St. Julian’s, Malta, 19 May – 23 May 2025

Agenda Item:	7.28.7

Source:	Ericsson

Title:	UE demodulation requirements for LP-WUS/WUR

Document for:	Discussion

# 1	Introduction

RAN1/RAN2 has almost completed the specification of LP-WUS in Rel-19 LP-WUS/WUR WI [1]. This contribution shows our view on the UE demodulation requirements for LP-WUS/WUR.

# 2	Core part discussion status

## 2.1	UE procedures

One of the main objectives in this WI is to specify the low-power wake-up signal (LP-WUS). The main purpose of LP-WUS is to indicate PDCCH monitoring in IDLE/INACTIVE and CONNECTED mode UEs. RAN1 assumes the UE uses low-power wake-up receiver (LP-WUR), which is designed to receive LP-WUS. The LP-WUR has less complex architecture (e.g., single receive antenna) compared with the legacy NR receiver (called the main radio (MR) in contrast with LP-WUR).

When gNB configures LP-WUS for connected mode UE, the UE uses LP-WUR to monitor LP-WUS, and UE can set the MR in deep sleep mode to save power consumption. Once the LP-WUR detects the indication signal in LP-WUS, the LP-WUR activates the MR, and the MR starts to receive PDCCH. There is a time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring, and the gap depends on the UE capability. RAN1 assumes 3 gap values corresponding to V1 = [3,5,6], V2 = [10,13,20], V3 = [33,42] ms so far. This means the largest time gap between LP-WUS and PDCCH could be 42ms.

When gNB configures LP-WUS for IDLE/INACTIVE mode UE, the UE monitors the LP-WUS for paging indication with LP-WUR. Since the DRX cycle in IDLE/INACTIVE is much longer than that in CONNECTED, the LP-WUR sleeps longer period compared with CONNECTED mode, and it causes the larger receive frequency drifting. To compensate the frequency error after the longer sleep, RAN1 introduced the dedicated synchronization signals for LP-WUR, called LP-SS. RAN1/RAN4 assume the UE uses LP-SS not only for re-synchronization, but also for RRM measurements.

Observation 1: The purpose of LP-WUS is to trigger the PDCCH monitoring in C-DRX in CONNECTED mode.

Observation 2: The purpose of LP-WUS is to trigger the paging and RRM measurements in IDLE/INACTIVE modes.

## 2.2	LP-WUS design

RAN1 agreed to use On-Off Keying (OOK) to modulate the LP-WUS. When gNB transmits the LP-WUS information bits, it encodes the information bits by Reed-Muller code, performs rate-matching, and then encodes by Manchester code. The gNB applies DFT to the coded bits over 11 RBs (=132 subcarriers) and then performs IFFT together with other DL channel/signals. This means the gNB transmitter overlays the LP-WUS OOK symbols with NR downlink OFDM sequence. According to RAN1 agreements, the number of information bits is five bits per UE or UE group.

When the gNB applies DFT to map the coded bits into 132 subcarriers, it processes M bits per DFT processing, where M is 1, 2 or 4. For example, in the case of M=1, called OOK-1, one coded bit is transmitted in one OFDM symbol duration. On the other hand, OOK-4 configuration transmits four coded bits in one OFDM symbol duration. It is up to UE implementation whether it can support M=1 only or support M = 1, 2, or 4, according to UE capability signaling [4].

Observation 3: gNB transmits LP-WUS information bits by overlaying with OFDM sequences by OOK-M, where M = 1, 2, or 4.

## 2.3	LP-WUR design

RAN1/RAN4 assumes two types of LP-WURs [2] to achieve low power consumption:

- OOK-based wake-up receiver (OOK-WUR) such as a zero-IF architecture, which detects amplitude of the received signal (envelop detection in time domain).

- OFDM-based receiver (OFDM-WUR), which is same as the NR receiver, but it uses only one receive antenna branch for FR1.

Since these receivers have different RF characteristics such as noise figure (NF) and impairment margin (IM) according to TR [3], RAN4 RF has agreed to specify two sets of RF requirements, e.g., REFSENS.

RAN4 RF has already agreed to define REFSENS requirements based on the LP-WUS missed detection rate (MDR) [5], but the issue is the testability because UE does not send feedback to the gNB regardless of the LP-WUS reception results. One feasible option is to introduce test mode, but RAN4 RF concluded that it is up to RAN5 for the verification method of LP-WUR Rx requirements.

Observation 4: RAN4 RF are going to define the LP-WUR RF Rx requirements by the LP-WUS missed detection rate (MDR).

# 3	LP-WUS demodulation requirements

When UE receives the LP-WUS, UE does not send feedback signal regardless of the decoding results because the purpose of LP-WUS is to trigger PDCCH monitoring or paging monitoring. This means we cannot verify the LP-WUS detection rate directly. RAN4 RF has already agreed to define the LP-WUR Rx requirements based on LP-WUS MDR, but they concluded it is up to RAN5 for the verification method.

We think the LP-WUR RF Rx requirement would be almost same as LP-WUS demodulation requirements (e.g., SNR to achieve LP-WUS MDR 1%). The difference could be channel model only; RF test uses the static channel, but the demodulation performance usually uses the fading channel such as TDLA30. We think the LP-WUR RF Rx requirements are sufficient to verify the LP-WUS demodulation performance.

Observation 5: LP-WUR RF Rx requirements are sufficient to verify the LP-WUS demodulation performance.

Proposal 1: RAN4 does not define LP-WUS demodulation requirements.

# 4	Summary

Observations:

Observation 1: The purpose of LP-WUS is to trigger the PDCCH monitoring in C-DRX in CONNECTED mode.

Observation 2: The purpose of LP-WUS is to trigger the paging and RRM measurements in IDLE/INACTIVE modes.

Observation 3: gNB transmits LP-WUS information bits by overlaying with OFDM sequences by OOK-M, where M = 1, 2, or 4.

Observation 4: RAN4 RF are going to define the LP-WUR RF Rx requirements by the LP-WUS missed detection rate (MDR).

Observation 5: LP-WUR RF Rx requirements are sufficient to verify the LP-WUS demodulation performance.

Proposals:

Proposal 1: RAN4 does not define LP-WUS demodulation requirements.

# References

- RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

- TR 38.869, “Study on low-power Wake-up Signal and Receiver for NR”, 3GPP RAN1.

- TR 38.774, “Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver”, 3GPP RAN4.

- R1-2502977, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis”, RAN1.

- R4-2505234, “WF for [114bis][131] NR_LPWUS_UERF”, vivo.