3GPP TSG-RAN WG4 Meeting #115							         		     R4-2507118

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.1.2.1.1 General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Title: 	On HPUE MSD simplification scope and needed corrections for 1UL 2CC and 2UL 3CC

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b, the way forward [1] scoped the work for the HPUE MSD simplification. NR-U HPUE combination are excluded from the work and the way of working for the remainder of Release 19 has been clarified. In this contribution, we further discuss the scope for 1UL 2CC and 2UL 3CC cases and point at the necessary corrections in 38.101-1 and 38.101-3 that were already covered in [2].

# Discussion

## 2.1 Way Forward input

Way Forward [1] provided an input on the general aspects as follows in italics.

It is expected that CR for HPUE MSD topic is agreed in August RAN4#116 meeting when Rel-19 is completed.

Until that in RAN4 #115 and #116 basket WI work continues without any change.

If CR for 1 band 1 ULCC HPUE MSD topic is approved in September RAN#109 and if an exception to extend the Rel19 HPUE Baskets WIs are allowed after RAN#109, then the Rel19 HPUE Basket WIs may be revised to reflect that the new HPUE MSD rules can be applied,

NR-U is not part of this WF.

When the CR for HPUE MSD is implemented (after RAN#109)

- All specified PC2 MSD tables and PC1.5 MSD tables remain untouched in TS 38.101-1 and TS38.101-3

- If for some new PC2/1.5 CA configuration there is a need to define a MSD differently from LUTs it can be further discussed how to apply the exception case for the specific band combinations, but the expectation is that this is a rare case.

Handling of the Rel-19 baskets during the extension after CR for HPUE MSD is implemented (after RAN#109):

- The procedure is unchanged for the default power class (i.e.PC3): MSD must be studied, and, where applicable, the default PC3 MSD is captured in the TPs for TR.

Observations:

- It is stated that NR-U is not part of this forward as highlighted in yellow above but does not explicitly exclude it from the HPUE MSD simplification like proposed in [3], Also, the NR-U cases identified in [3] have not .

- The current 1UL and 2UL technical discussion in scope of the way forward does not include the 1UL band with 2CC  and 2UL bands with 3CC cases that involves intra-band ULCA in one of the bands and have the following MSD characteristics specified in the clause 7.3A.5 of TS 38.101-1:

  - 1UL band with 2CC with IMD orders beyond 5 and that have PC2 cases for 2DL.

  - 2UL bands with 3CC with triple beat IMD3 and has one PC2 case for 1UL.

It should be clear to everyone that the IMD mechanisms involved in the 1UL 2CC and 2UL 3CC cases are not the same as for the 2UL 2CC cases and the LUTs and equations do not apply as for these cases, the UL band with 2CC is allowed to use MPR and is assumed to meet SEM and spurious emissions thus the IMD evolution with the band combination power class is different.

Proposal:

- HPUE Band combinations including one or more NR-U band is not in scope of Release 19 HPUE MSD simplification work.

- For reference, the list of NR-U HPUE cases in [3] is captured.

- 1UL 2CC and 2UL 3CC MSD due to ULCA IMD and triple beat cases are not supported by the current 2UL HPUE MSD simplification work.

- FFS if these two cases will be supported in Release 19 HPUE MSD simplification.

## 2.2 Corrections needed to the 1UL 2CC and 2UL 3CC MSDs.

In [2], we already pointed at the fact that 1UL 2CC and 2UL 3CC HPUE MSDs had errors as in most cases, it assumed that the interference power would increase by 3x the IMD order. This is not that case as the IMDs are maintained to the SEM and SE limit by accounting that the UE applies MPR/A-MPR to the intra-band ULCA UL band.

This affects the MSD for the different cases:

- For 1UL 2CC case, IMD3 and IMD5 must meet the SEM and spurious emission limits thus regardless of the ULCA band power class the interfering odd order IMD power does not increase. Thus, the MSD does not increase between PC3 and HPUE cases.

- For 1UL 2CC case, the even order IMD power does not increase with 3x IMDorder as MPR is applied on the ULCA band. To simplify, we could also consider that there is no increase of the IMD power.

- For the 2UL 3CC case, the only HPUE case is CA_n3-n41 and it is a PC3 + PC3 case that probably needs to be re-evaluated as if the FDD band will see a 3dB power increase, the TDD band will see MPR.

Table 1 below summarizes the HPUE cases for 1UL 2CC and 2UL 3CC:

- The PC3 and PC2 MSD values are provided and:

- For 1UL 2CC, the values to be corrected are highlighted in red, while the good values are highlighted in green. Note that for CA_n13-n77 there is a PC2 MSD but no corresponding PC3 MSD

- For the 2UL 3CC case, the value to be studied is highlighted in yellow

- Some cases are also to be corrected to align the PC3 and PC2 test points (RBstart, carriers frequency….)

Table 1: 1UL 2CC and 2UL 3CC MSD test points with PC3 and PC2 values

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | PC3/PC2MSD (dB) | Duplex mode |  |
| CA_n1-n774 | n1 | N/A | 5 | N/A | 2130 | 17/31 | FDD | IMD515 |
|  | n7712 | 3310 | 10 | 1 (RBSTART=25) | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | N/A | 5 | N/A | 2140 | 1.7/13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| CA_n3-n41 | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 15.3/23.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |
| CA_n3-n774 | n3 | N/A | 5 | N/A | 1877.5 | 2.2/13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | N/A | 5 | N/A | 1877.5 | 2.2/13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| CA_n5-n77 | n5 | N/A | 5 | N/A | 880 | 8.6/18.5 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 (RBSTART=25) | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 (RBSTART=25) | 3850 | N/A | TDD | N/A |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 8.6/18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 (RBSTART=25) | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=25) | 3780 |  |  |  |
| CA_n7-n77 | n7 | N/A | 5 | N/A | 2687.5 | 15/29.9 | FDD | IMD515 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 | N/A | TDD | N/A |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 15/29.9 | FDD | IMD515 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | N/A | 5 | N/A | 750 | ??/8.6 | FDD | IMD415 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 |  |  |  |
|  | n13 | 781 | 5 | 25 | 750 | ??/18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 (RBSTART=25) | 3885 | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5/8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |
|  | n25 | 1860 | 5 | 25 | 1940 | ??/15.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 (RBstart = 25) | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 (RBstart = 208) | 2556 |  |  |  |
| CA_n25-n77 | n25 | N/A | 5 | N/A | 1987.5 | 2.7/13.6 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=0) | 3945 | N/A | TDD | N/A |
| CA_n25-n78 | n25 | N/A | 5 | N/A | 1980 | 2.7/13.6 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| CA_n28-n77 | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 (RBSTART=25) | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 (RBSTART=25) | 3900 | N/A | TDD | N/A |
| CA_n28-n78 | n28 | N/A | 5 | N/A | 780 | 8.6/18.5 | FDD | IMD414 |
|  | n7812 | 3310 | 10 | 1 (RBSTART=7) | 3310 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=0) | 3700 |  |  |  |
| CA_n41-n66 | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5/32.5 | FDD | IMD5 |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7/2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 17/32 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n66-n77 | n66 | N/A | 5 | N/A | 2197.5 | 15/31 | FDD | IMD515 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 | N/A | TDD | N/A |
| CA_n66-n78 | n66 | N/A | 5 | 25 | 2150 | 1.7/13.6 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=7) | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 (RBSTART=0) | 3750 |  |  |  |
| CA_n71-n77 | n71 | N/A | 5 | N/A | 640 | 8.6/18.5 | FDD | IMD414 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |

Proposal for 1UL 2CC and 2UL 3CC HPUE MSDs:

- 1UL 2CC odd order IMD MSDs do not increase for HPUE as limited by SEM and SE.

- 1UL 2CC even order IMD MSDs do not increase for HPUE as MPR is applied on intra-band ULCA to meet emissions.

- 2UL 3CC CA_n3-n41 HPUE triple beat MSD is check/re-evaluated.

- A correction CR is implemented before end of the Release 19 to:

  - Aligns PC3 and PC2 test points definition for PC3 and PC2 including missing cases.

  - Aligns 1UL 2CC PC2 MSD value with PC3 value.

  - Corrects the 2UL 3CC MSDs if needed.

# Conclusion

In this contribution, we have discussed the 2UL operation aspect of CA_n28-n71 and have the following proposal.

Proposal:

- HPUE Band combinations including one or more NR-U band is not in scope of Release 19 HPUE MSD simplification work.

- For reference, the list of NR-U HPUE cases in [3] is captured.

- 1UL 2CC and 2UL 3CC MSD due to ULCA IMD and triple beat cases are not supported by the current 2UL HPUE MSD simplification work.

- FFS if these two cases will be supported in Release 19 HPUE MSD simplification.

Proposal for 1UL 2CC and 2UL 3CC HPUE MSDs:

- 1UL 2CC odd order IMD MSDs do not increase for HPUE as limited by SEM and SE.

- 1UL 2CC even order IMD MSDs do not increase for HPUE as MPR is applied on intra-band ULCA to meet emissions.

- 2UL 3CC CA_n3-n41 HPUE triple beat MSD is check/re-evaluated.

- A correction CR is implemented before end of the Release 19 to:

  - Aligns PC3 and PC2 test points definition for PC3 and PC2 including missing cases.

  - Aligns 1UL 2CC PC2 MSD value with PC3 value.

  - Corrects the 2UL 3CC MSDs if needed.

# References

[1] R4-2505143 WF on MSD simplifcation for HPUE, Skyworks, Qualcomm, Murata, Nokia, Samsung, Huawei, MediaTek, Vivo, ZTE, R4#114b

[2] R4-2417793 Observations on HPUE 2UL IMD MSDs, Skyworks Solutions Inc., R4#113

[3] R4-2503169 Simplification of HPUE 2UL DeltaMSD, Skyworks Solutions Inc., RAN4#114b