3GPP TSG-RAN WG4 Meeting #115				                              R4-2507127

Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	7.1.2.2.1

Document for:	Approval

# Background

In RAN4#114bis, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed, where it is agreed that the boundary and integral region of OOBE will be based on original UE CBW rather than the extended UE CBW to meet the ITU requirement.

The major remaining issues are how to determine the extended CBW and how the IBE can be applied, where our views on how to resolve the remaining issues are discussed in this contribution.

# Extended RB numbers

In RAN4#114bis, the following agreement has been made for on the extended RB number:

  - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature

    - In addition, consider a limited number of additional fixed values with capability

As discussed in our previous contributions, adding more configurations of the extended RB number less than 50% UE original bandwidth would evitable adding more effort on testing and verification, with very little gain of on MPR reduction. Therefore, the set of additional value needs to well justified for clear purpose.

Proposal 1: Considering the gain is very limited on the additional fixed value of extended RB number, any additional values need to have a good motivation to be introduced in the standard.

# Guardband

The guard band for the extended UE channel bandwidth was also discussed in RAN4#114bis. To some extended, the extended UE bandwidth already play a role of guardband which shift the SEM and ACLR that is further away from the UE transmission bandwidth. Therefore, it is unclear if there is any need to introduce a additional guardband for the extended UE CBW

Proposal 2: Do not introduce any additional guardband on the edge of the extended UE channel BW.

# IBE

As previously agreed, IBE will be applied to the edge between the original UE CBW and the extended UE CBW. However, the exact IBE limitation are still under debating. For (e)Redcap UE, it does not apply any IBE requirement out of 20 MHz CBW. Therefore, it is a bit strange now with intention to relax the emission limit we actually introducing additional requirements on (e)Redcap. Meanwhile, a (e)Redcap UE is capable of meeting SEM which is a more stringent requirement in the extended UE CBW without any MPR reduction. Now, a with just roughly 1 dB MPR reduction, it would be mostly likely to be within any IBE limits even without explicitly specify it. Therefore, as an alternative, RAN4 may consider do not introduce any IBE requirement in the region of the extended UE CBW.

Proposal 3: RAN4 can consider to not introduce any IBE requirement in the extend UE CBW at least for (e)Redcap UEs.

# signaling aspects

For UE capability, the following agreement made in RAN4#114bis:

  - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature

  - To discuss the following aspects

    - Whether to have separate UE capabilities for scenario 1 and scenario 2

    - Whether to indicate multiple values of fixed extension capability

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Proposal 4: It should be an optional feature for UE with per-band capability.

From UE side, the network deployment scenario is not aware, and therefore, there is no need to any UE capability with respect to the network deployment scenario.

Proposal 5: Do not introduce any UE capability related to deployment scenario.

For the NW signalling, since fixed extension with 0.5 CBW has been agreed as the default mechanism for the MPR reduction, there is no need to introduce any additional signalling from NW side to indicate the value of extended UE CBW. However, a single bit signalling via either MAC or RRC layer would be needed so that the network can enable/ disable the feature in the field based on the level of interference.

Proposal 6: Define 1 bit network signalling in RRC layer so that the network can enable/disable the feature.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Proposal 1: Considering the gain is very limited on the additional fixed value of extended RB number, any additional values need to have a good motivation to be introduced in the standard.

Proposal 2: Do not introduce any additional guardband on the edge of the extended UE channel BW.

Proposal 3: RAN4 can consider to not introduce any IBE requirement in the extend UE CBW at least for (e)Redcap UEs.

Proposal 4: It should be an optional feature for UE with per-band capability.

Proposal 5: Do not introduce any UE capability related to deployment scenario.

Proposal 6: Define 1 bit network signalling in RRC layer so that the network can enable/disable the feature.

# References

- R4-2504670 		Topic summary for [114bis][116] NR_ENDC_RF_Ph4_part1 		Moderator (Huawei)

- R4-2505099 		WF on power domain enhancement 	Huawei