3GPP TSG-RAN WG4 Meeting #115		                R4-2507129

Malta, MT, 19th – 23rd May, 2025

Source:	Sony

Title:	Consideration on the OTA test of ambient IoT device 1

Agenda item:	7.26.5

Document for:	Approval

# Background

The OTA test of ambient IoT (AIoT)device type 1 has been further discussed in RAN#114bis [1], and agreements on basic test setup were made [2]. In this contribution, we provide some further views on key issues that we think are critical to be studied for OTA test setup.

# Basic test setup

## Polarization of CW antenna

Although most aspects of basic test setup have been discussed and agreed upon in RAN#114bis, one missing issue is the CW antenna polarization. For a given AIoT tag, the strength of backscattered power is not only on the angle of the CW antenna but also the polarization of the CW antenna with respect to the polarization of the AIot devices. As an example, the bistatic RCS pattern of an AIoT tag under reflect and absorb mode is simulated with two orthogonal polarizations, respectively. The incident angle of CW is assumed from the boresight of the AIoT tag, and the results are shown in Fig.1.

Fig.1.  The bistatic RCS of AIoT tag under different polarized CW signals.

It is worth mentioning that the backscattering power level on the first sideband is determined by the modulation depth, e.g., the difference between the reflection coefficient of the two states (e.g., reflect and absorb states), as analyzed in [3]. Based on this, with a CW that is orthogonal polarized to the AIoT antenna, the backscattering loss will significantly increase compared to the case where the CW is in the same polarization of AIoT devices.

Therefore, to avoid a polarization mismatch between the CW signal and the AIoT tag, it is proposed that the CW signal be sent in two polarized sequentially, and the measured backscattered loss shall be the averaged value between the two polarizations.

Proposal 1: The CW signal shall be sent in two polarized sequentially, and backscattered loss shall be defined as the averaged value between the two polarizations.

## Offset of CW antenna and testing antenna

For the offset between testing antenna and CW antenna, the following agreement has been made in RAN4#114bis

Whole sphere scan is supported

The position of CW antenna is a set of fixed position(s) as baseline (depending on core requirements discussion).

- FFS on relative positions between reader antenna and CW

An illustration of a possible test setup is shown in Fig. 2. Firstly, we would like to clarify our understanding of whether the position of the CW antenna is fixed with respect to the AIoT devices or if it is fixed with respect to the test system.

Proposal 2 RAN4 shall clarify if the CW antenna is fixed with respect to the AIoT devices or if it is fixed with respect to the test system.

Fig. 2. OTA test chamber setup

The minimum offset between the testing antenna and CW antenna would be the key issue to be determined from the test perspective. This would be up to the minimum coupling between the CW antenna and test antenna, and further input from TE vendor is needed.

Proposal 3: The minimum offset between the CW and testing antenna depends on the acceptable coupling level between the CW and testing antenna, so further input from TE vendors may be needed here.

# Other aspects of OTA test

Another major difference between AIoT devices and legacy UEs is the device behavior due to the energy harvesting. In [4], it is assumed that the duration of one device's unavailability due to charging by energy harvesting can be assumed to be up to several tens of seconds, which was endorsed in RAN#103.

For example, we illustrate in Fig. 2 how the energy harvesting would affect the test procedure, which assumes the device must charge before each measurement. In this case, the TE needs to understand when to initiate the connection with the device and when to start performance measurement. Therefore, the energy harvesting/charging time duration and periodicity in Fig. 3 may need to be declared to the TE during the conformance test. Such an impact is particularly critical if multiple points need to be measured.

Figure. 3. Impact on energy harvest on the device test procedure.

Proposal 4:  The UE charging time duration and periodicity may need to be declared in the OTA test.

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been made:

Proposal 1: The CW signal shall be sent in two polarized sequentially, and backscattered loss shall be defined as the averaged value between the two polarizations.

Proposal 2 RAN4 shall clarify if the CW antenna is fixed with respect to the AIoT devices or if it is fixed with respect to the test system.

Proposal 3: The minimum offset between the CW and testing antenna depends on the acceptable coupling level between the CW and testing antenna, so further input from TE vendors may be needed here.

Proposal 4:  The UE charging time duration and periodicity may need to be declared in the OTA test.

# References

- R4-2500686 	Topic summary for [114][134] A-IoT_device​ Moderator	(CMCC)

- R4-2502858 	WF on requirements for A-IoT device 	CMCC

- R4-2507128	RF requirements of ambient IoT device 1	Sony

- RP-240116		View on Ambient IoT scope 	Qualcomm.