3GPP TSG-RAN WG4 Meeting #115	R4-2507134

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on Conditional Intra-CU LTM

Agenda item:	7.29.3.3

Document for:	Discussion

Introduction

In this contribution, we continue the discussion about RRM core requirements for CLTM and share our views on the following topics:

- Requirements for L1 measurements on deactivated Scell and UE-based TA estimation

- CLTM cell switch delay

- Subsequent CLTM

Discussion

General aspects and early synchronization

Issue 1-1-1: L1 measurement on deactivated Scell and UE based TA measurement RAN4#114 < Way forward > FFS the following options: - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated. - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA. - Option 3: Not define requirement for any of them.. < Way forward > FFS the following options: Conclude this issue in the next meeting. - Option 1: Define requirements for L1 measurement on deactivated Scell. - Option 2: Define requirements for UE based TA measurement. - Option 3: Not define requirement for any of them.

In the last RAN4 meeting, the chairman guidance was to conclude this issue in the next meeting, with the instruction that if no consensus is reached regarding one option to keep among Options 1 and 2, then Option 3 will be agreed.

Considering the necessity of the requirements being discussed, we think that defining new L1 measurement requirements for deactivated SCells is not necessary in the scope of Rel-19 WI.

Regarding UE-based TA estimation, we believe that there are multiple critical issues from the network point of view that need to be addressed (e.g., reference signals, TA acquisition time, interruptions, UL timing accuracy requirements). We raised these issues more in detail in our earlier contributions [1-3] and proposed that if these issues cannot be solved within Rel-19, then Option 3 is preferred.

Considering the time limitation (only one more meeting for Rel-19 WI) and the unclear and divergent companies’ views regarding the requirements to be defined for this feature, we believe that it is not possible to define new requirements for UE-based TA estimation in the scope of Rel-19 WI.

Hence, taking these points into account, we support Option 3.

RAN4 to not define requirements for L1 measurements on deactivated SCell and UE-based TA estimation in the scope of Rel-19 WI.

Conditional LTM delay

The following agreements and WFs were made during RAN4#114-bis meeting in relation to the CLTM cell switch delay definition.

Issue 1-2-1: General aspects of Conditional LTM delay < Agreement > - CHO requirements framework can be used as baseline for Conditional LTM. Issue 1-2-2: The starting point of conditional LTM delay < Agreement > - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. Issue 1-2-3: The components of conditional LTM delay Issue 1-2-4: Tinterrupt < Agreement > - Keep either TCLTM_execution or TCLTM-RRC-processing as one component for the conditional LTM delay. - Note: The legacy value(s) in previous releases will be reused. - TCLTM_execution or TCLTM-RRC-processing is not included in Tinterrupt. - Not include the components for TCI state activation and/or PDCCH order PRACH in C-LTM delay. - TEvent_DU can be updated based on RAN2 progress if needed. - Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell. - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered. - If SSB based L3-RSRP measurement is used in the event, reuse Tmeasure in CHO. - FFS: If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to be introduced in R19. < Way forward > FFS - Tfirst-RS and TRS-proc are included in Tinterrupt - Reuse the values in Rel-18 LTM, and further discuss whether the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM can be reused or not.

Issue 1-2-5: TRRC < Way forward > FFS the following proposals: - Proposal 1: TRRC is the RRC procedure delay. Issue 1-2-6: TEvent_DU - The issue is merged into issue 1-2-3. Issue 1-2-7: Tmeasure - The issue is merged into issue 1-2-3. Issue 1-2-8: TCLTM_execution - The issue is merged into issue 1-2-3. Issue 1-2-9: TLTM-RRC-processing - The issue is merged into issue 1-2-3. Issue 1-2-10: TLTM-processing < Way forward > FFS the following proposal: - Proposal 1: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. - Proposal 1a: TLTM-processing can be up to 20ms. Issue 1-2-11: Tfirst-RS / TRS Issue 1-2-12: TRS-proc - The issues are merged into issue 1-2-3. Issue 1-2-13: TLTM-IU < Way forward > FFS the following proposal: - Proposal 1: TLTM-IU is the interruption uncertainty during LTM cell switch. Issue 1-2-14: Twait_RS < Way forward > FFS the following proposal: - Proposal 1: Twait_RS is the uncertainty in acquiring the first SSB for T/F tracking on the new cell.

Based on the latest agreements made in RAN4#114-bis meeting, the CLTM cell switch delay can be summarized as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TLTM-processing + TLTM-IU

Although several agreements regarding the components of the CLTM cell switch delay and interruption time were made during RAN4#114-bis meeting, few issues regarding some delay components, like TRRC, Tmeasure, Tfirst-RS/TRS-proc, TLTM-processing, and TLTM-IU, remained open.

TRRC

According to the agreement made in RAN2#129, the UE starts execution condition evaluation once it receives CLTM configuration including CLTM execution condition. Hence, for a non-subsequent CLTM cell switch, TRRC component is needed in the CLTM delay and it is the RRC procedure delay defined in clause 12 in TS 38.331.

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

Tmeasure

It was agreed in the last meeting that Tmeasure in CLTM should follow the definition of Tmeasure in CHO if SSB-based L3-RSRP measurement is used in the event. However, the case where CSI-RS-based L3-RSRP measurements are used in the execution condition evaluation was not discussed.

The following RAN2 agreements show that CSI-RS-based measurements should be supported in CHO. However, by checking the measurement time definition in CHO, only SSB-based L3 measurements are considered.

RAN2 Agreements 1 Separate CHO execution condition(s) can be configured for each individual candidate cells. 2 Define a CHO execution condition by the measurement identity which identifies a measurement configuration. (FFS to be addressed in stage 3 which parts of the measurement configuration are used for the CHO triggering) 3 As a baseline CHO can be triggered based on a condition consisting of a single event, single RS type, singe quantity. 3.1 The single trigger quantity can be configured to be RSRP, RSRQ or RS-SINR 3.2 The single RS type can be configured to be SSB or CSI-RS FFS Whether multiple triggering conditions are required.

CHO requirements do not support the case where CSI-RS-based L3 measurements are used in the execution condition evaluation despite being supported by RAN2.

For CLTM, we believe that CSI-RS-based L3 measurements should be supported, and the measurement delay defined in section 9.10 can be used as the baseline for the definition of Tmeasure in this case.

RAN4 to define CLTM requirements for the case where CSI-RS-based L3-RSRP measurements are used in the execution condition evaluation.

When the CLTM execution condition is L1-based, the existing L1 measurement delay for SSB-based in Rel-18 and the to-be-defined measurement delay for CSI-RS-based in Rel-19 can be used as the baseline for the definition of Tmeasure. However, Treport is to be set to zero. In addition, and as can be seen from the definitions of L1-based conditions stated below, UE may need to measure both the beam of the serving cell and the beam of the candidate cell to be able to realize that the condition is met.

- LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell

- LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2

To make sure that the evaluation delay is defined to be until the (serving or candidate cell) measurement that triggers the cell switch, both beams the UE is evaluating need to be taken into account in the delay. As the discussion is identical to event-triggered L1 reporting delay under the corresponding agenda item of this work item, we propose that the evaluation delay that is being discussed for event-triggered L1 reporting is to be reused.

When the execution condition is L1-based, the definition of Tmeasure can follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item.

Tfirst-RS/TRS-proc

FFS 1:  Tfirst-RS and TRS-proc are included in Tinterrupt

We think it is reasonable to assume that Tfirst-RS and TRS-proc do not need to be considered as part of the interruption. When the fine DL synchronization is done before the cell switch in LTM, the procedures (e.g., early TCI state activation) do not cause an interruption. We proposed in Rel-18 that these procedures would not cause an interruption during the cell switch either, but at that time it was argued by some companies that since the network sends the cell switch command, the UE will not be scheduled after receiving the cell switch command anymore. This is not the case in CLTM, where the cell switch decision happens at the UE side. Hence, we think that DL synchronization components, if included in the cell switch delay, should not be part of the interruption time.

Tfirst-RS and TRS-proc should not be included in the CLTM cell switch interruption time.

FFS 2: Reuse the values and the conditions for Tfirst-RS = 0 and TRS-proc = 0 from Rel-18 LTM

In Rel-18 LTM, Tfirst-RS and TRS-proc have the following definitions:

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. Tfirst-RS = 0 and TRS-proc= 0 under the following conditions: - The target TCI state indicated in the LTM cell switch command is in the serving cell active TCI state list, or - The UE is configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8.25.3 and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or - the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the LTM candidate cell TCI state activation MAC-CE is received, or - The target cell is an FR1 cell, and the UE is not configured with LTM L1 intra- and/or inter-frequency measurements for the target cell, and - The target TCI state in the cell switch command is in the LTM candidate cell active TCI state list, and - the time between receiving the LTM candidate cell TCI state activation MAC-CE and the cell switch command is at least TCI state activation delay stated in section 8. 25.3, and not more than - TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3. - TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell, or - The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms]. Otherwise, Tfirst-RS is the time to the first SSB transmission on the target cell after Tcmd + TLTM-RRC-processing + TLTM-processing. TRS-proc = 2 ms.

RAN4 to reuse Rel-18 LTM definitions, values, and conditions for Tfirst-RS = 0 and TRS-proc = 0 as a baseline for the definition of these two components in the CLTM cell switch delay.

In the following part, we propose some adjustments to the Rel-18 LTM conditions for Tfirst-RS = 0 and TRS-proc = 0, that are needed in the CLTM cell switch delay definition.

RAN2 has agreed that the UE selection of the target beam is up to UE implementation. This means that even if there is at least one beam with an active TCI state for the candidate cell fulfilling the CLTM cell switch condition, the UE may not select this beam as the target beam for cell switch. If this happens, based on the existing Rel-18 conditions, Tfirst-RS and TRS-proc do not get zero as value. However, especially when CSI-RS-based measurements and beams are introduced, the UE may still have DL synchronization with the selected target beam, if both beams (TCI states) are associated with the same SSB. Hence, we think that it would be important to take this into account in the cell switch delay, especially for CLTM where the beam selection is up to UE implementation. Therefore, the conditions for Tfirst-RS = 0 and TRS-proc = 0 should be slightly adjusted as proposed:

Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

According to the Rel-18 conditions for Tfirst-RS = 0 and TRS-proc = 0, unless the periodicity of the SSB of the activated TCI state is up to 160 ms and UE is configured with L1 measurements, the TCI state activation command sent as part of the early DL synchronization step is considered to be useful for cell switch interruption reduction (Tfirst-RS and TRS-proc are zero) only if the cell switch command is sent within TCI state activation delay + 160 ms (for UE configured with L1 measurements) or within TCI state activation delay + 480 ms or 160 ms (for UE configured with L3 intra- or inter-frequency measurements, respectively) from the LTM candidate cell TCI state activation MAC-CE. Reusing these requirements for Rel-19 CLTM would mean that early TCI state activation is considered to be useful only if CLTM decision, and potentially UL synchronization steps, are performed within the delays stated previously.

Hence, assuming that all UEs have the limitation of keeping track of the fine DL timing synchronization of an activated TCI state only for 160/480 ms would reduce the usefulness of the early TCI state activation feature especially for Rel-19 CLTM, where the moment in time the cell switch condition is fulfilled, and cell switch is triggered by the UE, cannot be controlled. If some UEs would be able to keep track of the DL sync for a time longer than 160/480 ms, this is in no way visible to the network and hence the network may not consider it beneficial to activate TCI states for CLTM cell switch interruption reduction purposes.

Early TCI state activation may be useful for CLTM cell switch interruption reduction only if condition becomes met and cell switch is triggered within 160 ms/480 ms after TCI state activation is completed.

Taking the above into account, we would like to check with the group whether it is still reasonable to assume that all UEs supporting CLTM have the same limitation for early DL synchronization. If the limitation cannot be removed/relaxed for all UEs, an option is to define a UE capability that allows the network to know if the UE is able to keep track of the fine DL timing synchronization of an activated TCI state until cell switch or at least for a longer period than 160/480 ms as the minimum requirements state.

RAN4 to consider either removing the limitation of 160/480 ms for DL sync after TCI state activation or defining a UE capability to indicate UE can keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160 ms with L1 measurements, or for longer than 480 ms/160 ms with L3 measurements.

In addition, when the network sends PDCCH order and the UE sends the preamble to a candidate cell based on this, similar to L3-based Rel-18 LTM, the DL sync should be considered applicable based on a successful preamble transmission. For UE to be able to use the TA it received in TA MAC-CE for RACH-less cell switch, UE will need to have DL sync. Hence, while the TAT is running, it is beneficial that the UE keeps track of the DL sync and hence Tfirst-RS and TRS-proc should be zero.

If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TAT is running.

Twait_RS

Twait_RS was proposed in [4] as the uncertainty in acquiring the first SSB for T/F tracking on the new cell. The motivation behind introducing this component was to reduce the interruption time caused by waiting for the SSB occasion. It is worthwhile to mention that there was no delay component dedicated solely for waiting for the SSB occasion in NR HO, CHO, or LTM. It is not clear if this delay component needs to be also introduced for pre-Rel-19 mobility procedures, if introduced for CLTM. Therefore, we believe that there is no need to define such a delay component, especially if Tfirst-RS and TRS-proc are not included in the CLTM interruption time.

The waiting time for the SSB occasion is covered by Tfirst-RS and it will not contribute to the interruption time if Tfirst-RS and TRS-proc are not included in the CLTM cell switch interruption time.

TLTM-processing

For the UE processing time in Rel-19 CLTM cell switch delay, we believe that Rel-18 LTM definition and values for this delay component should be reused. The value of this parameter depends on UE capability of fast processing, and it is logical to assume that a UE supporting this capability for LTM also supports the capability if CLTM is configured.

TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

RAN4 to reuse Rel-18 LTM values for TLTM-processing in the CLTM cell switch delay.

TLTM-IU

For the interruption uncertainty in Rel-19 CLTM cell switch delay, we believe that Rel-18 LTM definition and values for this delay component can be reused. Although RACH-less cell switch is indicated by TA MAC-CE and TA timer instead of cell switch command for CLTM, whether the cell switch is RACH-based or RACH-less is defined in RAN2 specification, and RAN4 requirement does not consider this.

TLTM-IU is the interruption uncertainty during LTM cell switch.

RAN4 to reuse Rel-18 LTM values for TLTM-IU in the CLTM cell switch delay.

Subsequent Conditional LTM

Issue 1-3-1: General aspects of Subsequent Conditional LTM delay < Way forward > FFS the following options: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. - Proposal 1b: TEvent_DU needs to be updated. - Proposal 1c: Apart from the starting point of the delay. - Proposal 2: The subsequent CLTM cell switch delay DCLTM-subsequent is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell. Issue 1-3-2: The starting point of Subsequent conditional LTM delay < Agreement > - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

RAN4 agreed that the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. Hence, the subsequent CLTM cell switch delay can be defined as follows.

The subsequent CLTM cell switch delay is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell.

Issue 1-3-3: The components of subsequent conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. (Apple) - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18. - Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_interuption + Tfirst-RS + TRS-proc, where the TCLTM_interuption= TLTM-RRC-processing + TLTM-processing + TLTM-IU. - Proposal 3: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TCLTM_interrupt, where the TCLTM_interrupt = TCLTM-processing + TCLTM-IU.

The main difference between the initial and the subsequent CLTM cell switch delays is the starting point. In the subsequent CLTM, the starting point is when UE transmits RRCReconfigurationComplete message for the previous cell switch, which means that the TRRC component is not needed in the subsequent CLTM cell switch delay. Taking this and the agreements already made regarding the initial CLTM cell switch delay, we propose the following components for the subsequent CLTM cell switch delay. However, as few issues remain open regarding the interruption time in the initial CLTM, we propose to define the subsequent CLTM interruption time based on the conclusion of the CLTM interruption time.

DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TLTM-processing + TLTM-IU

RAN4 to define the subsequent CLTM interruption time based on the conclusion of the CLTM interruption time.

Issue 1-3-4: Other < Way forward > FFS the following proposal: - Proposal 1: Define initial and subsequent CLTM cell switch delay requirements in the same section under section 6.3.

Regarding how to capture the subsequent conditional cell switch delay in the specification, we think this can be merged into the same section with the initial CLTM cell switch delay (i.e., section 6.3). The only difference is the lack of TRRC parameter and the starting point, so there is no need to define a separate section to indicate this difference.

Define initial and subsequent CLTM cell switch delay requirements in the same section 6.3.

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: RAN4 to not define requirements for L1 measurements on deactivated SCell and UE-based TA estimation in the scope of Rel-19 WI.

Proposal 2: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

Observation 1: CHO requirements do not support the case where CSI-RS-based L3 measurements are used in the execution condition evaluation despite being supported by RAN2.

Proposal 3: RAN4 to define CLTM requirements for the case where CSI-RS-based L3-RSRP measurements are used in the execution condition evaluation.

Proposal 4: When the execution condition is L1-based, the definition of Tmeasure can follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item.

Proposal 5: Tfirst-RS and TRS-proc should not be included in the CLTM cell switch interruption time.

Proposal 6: RAN4 to reuse Rel-18 LTM definitions, values, and conditions for Tfirst-RS = 0 and TRS-proc = 0 as a baseline for the definition of these two components in the CLTM cell switch delay.

Proposal 7: Tfirst-RS = 0 and TRS-proc = 0 if the target TCI state is on the serving cell or LTM candidate cell active TCI state list or is associated with a TCI state on the active TCI state list.

Observation 2: Early TCI state activation may be useful for CLTM cell switch interruption reduction only if condition becomes met and cell switch is triggered within 160 ms/480 ms after TCI state activation is completed.

Proposal 8: RAN4 to consider either removing the limitation of 160/480 ms for DL sync after TCI state activation or defining a UE capability to indicate UE can keep track of fine DL synchronization of an activated TCI state without limitations (until cell switch or TCI state deactivation), or at least for longer than 160 ms with L1 measurements, or for longer than 480 ms/160 ms with L3 measurements.

Proposal 9: If UE received PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TAT is running.

Observation 3: The waiting time for the SSB occasion is covered by Tfirst-RS and it will not contribute to the interruption time if Tfirst-RS and TRS-proc are not included in the CLTM cell switch interruption time.

Proposal 10: TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

Proposal 11: RAN4 to reuse Rel-18 LTM values for TLTM-processing in the CLTM cell switch delay.

Proposal 12: TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 13: RAN4 to reuse Rel-18 LTM values for TLTM-IU in the CLTM cell switch delay.

Proposal 14: The subsequent CLTM cell switch delay is the delay from the end of the last TTI containing the RRCReconfigurationComplete message for the previous cell switch until the time the UE transmits the first UL message on the new target cell.

Proposal 15: DCLTM-subsequent = TEvent_DU + Tmeasure + TCLTM-RRC-processing + Tfirst-RS + TRS-proc + TLTM-processing + TLTM-IU

Proposal 16: RAN4 to define the subsequent CLTM interruption time based on the conclusion of the CLTM interruption time.

Proposal 17: Define initial and subsequent CLTM cell switch delay requirements in the same section 6.3.

- References

[1] R4-2405533, “On remaining aspects of LTM core requirements”, Nokia, RAN4#110bis, Changsha, China, April 15-19, 2024.

[2] R4-2500061, “On CLTM RRM requirements”, Nokia, RAN4#114, Athens, Greece, February 17-21, 2025.

[3] R4-2504202, “On CLTM RRM requirements”, Nokia, RAN4#114bis, Wuhan, China, April 07-11, 2025.

[4] R4-2503605, “Discussion on conditional LTM”, MediaTek Inc., RAN4#114bis, Wuhan, China, April 07-11, 2025.