3GPP TSG-RAN WG4 Meeting #115	R4-2507136

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	Discussion on Testing Issues for CSI Compression Two-sided Models

Agenda item:	7.22.3

Document for:	Discussion

Introduction

In this contribution, we continue the discussion about the testing issues for the CSI compression use case. In RAN4#114bis, the reduction of the volume of simulations was discussed and the following agreements were made:

Reduction of simulation results volume In order to simplify simulations, the following was agreed; - No more results are expected Option 3 track 1 - No more results are expected Unquantized model results - For option 3 track 2, use the Samsung decoder as the frozen decoder. Lower priority for Nokia decoder. - For option 4, use the Samsung encoder as the frozen encoder. Lower priority for Nokia encoder.

Following these agreements, we discuss in this contribution the simulation results related to Option 3 Track 2 and Option 4. We also provide our views on non-simulation issues regarding what to capture/specify for the studied options.

Discussion

Further simulation activity for Option 3 Track 2

The following agreements were reached in RAN4#114bis meeting regarding the simulation activity for Option 3 Track 2:

Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

Following the agreement made in RAN4#114bis meeting, we refer to the dataset used for training of the frozen decoder as Training Dataset 1, and the dataset used for training of the encoder based on the frozen decoder as Training Dataset 2. The results reported below are generated using quantized models.

Table 1 provides a summary of the additional results from Option 3 Track 2 expected for RAN4#115, using Samsung’s frozen decoder.

Table 1. Option 3 Track 2 results using Samsung’s frozen decoder.

| Results description | Encoder structure | Training Dataset 1 | Training Dataset 2 | Test dataset | SGCS |
| --- | --- | --- | --- | --- | --- |
| Results provided in RAN4#114bis | Reference CNN encoder | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.713 |
|  | Reference CNN encoder | Mixed training dataset | Mixed training dataset | Nokia’s test dataset | 0.714 |
| Additional results 1 for RAN4#115 | Reference CNN encoder | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.696 |
| Additional results 2 for RAN4#115 | Reference CNN encoder | Mixed training dataset | Nokia’s training dataset | Nokia’s test dataset | 0.702 |
| Additional results 3 for RAN4#115 | Lower complexity CNN encoder [1] | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.690 |
|  | Lower complexity TF encoder [1] | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.687 |

From the additional results 1 and 2, we can observe that the SGCS obtained when Training Dataset 2 is different from Training Dataset 1 is slightly lower than the SGCS obtained when assuming that Training Dataset 1 and Training Dataset 2 are the same.

Testing the pair of Samsung’s frozen decoder trained on mixed training dataset and Nokia’s encoder trained based on the frozen decoder and Nokia’s training dataset gives a slightly lower SGCS compared to training both encoder and decoder on the same mixed training dataset (~2.5% of SGCS reduction when testing the pair on mixed test dataset and ~1.7% of SGCS reduction when testing the pair on Nokia’s test dataset).

From the additional results 3, we can observe that using lower complexity encoder structure (both CNN- and transformer-based), as suggested in [1], introduces a small reduction in the SGCS results compared to the reference CNN encoder.

When training Samsung’s frozen decoder and Nokia’s encoder on the same mixed training dataset and testing the pair on the mixed test dataset, using lower complexity encoder structure introduces a small reduction in the SGCS results compared to using the reference CNN encoder structure (~3.2% of SGCS reduction when using the lower complexity CNN encoder and ~3.7% of SGCS reduction when using the lower complexity transformer encoder, as suggested in [R4-2505156]).

Table 2 provides a summary of the additional results from Option 3 Track 2 expected for RAN4#115, using Nokia’s frozen decoder.

Table 2. Option 3 Track 2 results using Nokia’s frozen decoder.

| Results description | Encoder structure | Training Dataset 1 | Training Dataset 2 | Test dataset | SGCS |
| --- | --- | --- | --- | --- | --- |
| Results provided in RAN4#114bis | Reference CNN encoder | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.709 |
|  | Reference CNN encoder | Mixed training dataset | Mixed training dataset | Nokia’s test dataset | 0.710 |
| Additional results 1 for RAN4#115 | Reference CNN encoder | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.700 |
| Additional results 2 for RAN4#115 | Reference CNN encoder | Mixed training dataset | Nokia’s training dataset | Nokia’s test dataset | 0.711 |
| Additional results 3 for RAN4#115 | Lower complexity CNN encoder [1] | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.688 |
|  | Lower complexity TF encoder [1] | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.679 |

The SGCS results obtained using Nokia’s frozen decoder show a similar performance to those obtained using Samsung’s frozen decoder. Hence, the same observations can be drawn from the two sets of results. Slightly lower SGCS results are obtained when: (1) using different datasets for training the frozen decoder and the encoder/frozen decoder pair compared to the case with same training datasets, and (2) using lower complexity encoder structures compared to the case with reference encoder structure.

Based on our Option 3 Track 2 simulations using both Samsung’s and Nokia’s frozen decoders, it is feasible to train Nokia’s encoder assuming different training datasets and lower complexity encoder structures (i.e., comparable SGCS results with no significant decrease compared to when assuming same training datasets and reference encoder structure).

Based on our results, Option 3 Track 2 is feasible, using both reference and lower complexity encoders.

Further simulation activity for Option 4

The following agreements were reached in RAN4#114bis meeting regarding the simulation activity for Option 4:

Further simulation activity for option 4 - All interested companies are requested to upload their “own” decoders to the server as soon as possible. - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset. - Definitions - Dataset 1 = Dataset used for training the frozen encoder - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder) - Dataset 3 = Dataset used for training the own encoder (using the own decoder) - First results priority results for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders) - Second priority results for RAN4#115: - Dataset 1 = Dataset 2 = Mixed training dataset - Dataset 3 = Company own dataset - Test Dataset: Mixed Dataset, also consider own test dataset if interested. - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - Results for consideration of lower complexity encoder for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156] - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

Following the agreement made in RAN4#114bis meeting, we refer to the dataset used for training of the frozen encoder as Dataset 1, the dataset used for training of the own decoder based on the frozen encoder as Dataset 2, and the dataset used for training of the own encoder based on the trained own decoder as Dataset 3. The results reported below are generated using quantized models.

First priority results for RAN4#115

Table 3 provides a summary of the first priority results from Option 4 expected for RAN4#115. The first step to generate these results is to train Nokia’s decoder using Samsung’s frozen encoder from Option 3 Track 2 and mixed training dataset. The next step is to train Nokia’s encoder using Nokia’s decoder (from previous step) and other companies’ decoders, and the mixed training dataset. The last step is to test the trained encoder/decoder pairs using the mixed test dataset.

Table 3. Option 4 first priority results for RAN4#115, with 2 bits scalar quantization.

| Encoder | Decoder | Dataset 1 | Dataset 2 | Dataset 3 | Test dataset | SGCS |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia’s encoder (reference CNN structure) | Nokia’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.673 |
|  | Samsung’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.688 |
|  | Qualcomm’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.686 |
|  | Ericsson’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.679 |
|  | Huawei’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.692 |
|  | MediaTek’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.672 |
|  | Vivo’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.680 |

The SGCS values obtained in this set of results are very comparable (only 0.02 difference between minimum and maximum SGCS values).

Based on our Option 4 simulations, it is feasible to train Nokia’s encoder using other companies’ decoders, which are trained using Samsung’s frozen encoder, and the mixed training dataset. The SGCS values obtained from testing the trained encoder/decoder pairs using the mixed test dataset are very comparable.

Second priority results for RAN4#115

Table 4 provides a summary of the second priority results from Option 4 expected for RAN4#115. Compared to the first priority results, the only difference in this second priority set is that Nokia’s dataset is used for training Nokia’s encoder together with Nokia’s decoder and other companies’ decoders, which are trained using Samsung’s frozen encoder and the mixed training dataset.

Table 4. Option 4 second priority results for RAN4#115.

| Encoder | Decoder | Dataset 1 | Dataset 2 | Dataset 3 | Test dataset | SGCS |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia’s encoder (reference CNN structure) | Nokia’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.677 |
|  | Samsung’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.690 |
|  | Qualcomm’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.687 |
|  | Ericsson’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.681 |
|  | Huawei’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.692 |
|  | MediaTek’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.677 |
|  | Vivo’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Nokia’s training dataset | Mixed test dataset | 0.682 |

As part of testing the interoperability between encoders and decoders in Option 4, our results show that Nokia’s encoder works well with other companies’ decoders not only when the mixed training dataset is used for all training purposes but also when using Nokia’s dataset for encoder training.

Results for consideration of lower complexity encoder for RAN4#115

Table 5 provides a summary of the results for consideration of lower complexity encoder from Option 4 expected for RAN4#115. Similar to first priority results, the mixed training dataset is here used for all training purposes (i.e., for training frozen encoder, training own decoder based on frozen encoder, and training own encoder based on trained decoder). The mixed test dataset is used here also for testing the trained encoder/decoder pairs, same as in the other two sets of results.

Table 5. Option 4 results for consideration of lower complexity encoder for RAN4#115.

| Lower complexity encoder | Decoder | Dataset 1 | Dataset 2 | Dataset 3 | Test dataset | SGCS |
| --- | --- | --- | --- | --- | --- | --- |
| Nokia’s lower complexity CNN encoder | Nokia’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.659 |
|  | Samsung’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.673 |
|  | Qualcomm’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.669 |
|  | Ericsson’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.664 |
|  | Huawei’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.674 |
|  | MediaTek’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.659 |
|  | Vivo’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.665 |
| Nokia’s lower complexity TF encoder | Nokia’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.642 |
|  | Samsung’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.651 |
|  | Qualcomm’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.649 |
|  | Ericsson’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.645 |
|  | Huawei’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.654 |
|  | MediaTek’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.641 |
|  | Vivo’s decoder trained using Samsung’s frozen encoder | Mixed training dataset | Mixed training dataset | Mixed training dataset | Mixed test dataset | 0.645 |

Although using lower complexity encoder structures reduces the SGCS values compared to when using the reference CNN encoder, the results obtained from testing the trained encoder/decoder pairs using the mixed test dataset are still very comparable within the same encoder structure.

Non-simulation issues

What needs to be captured or specified for Option 3

- The following agreements and FFS points regarding what to capture/specify for Option 3 were reached in RAN4#114bis meeting:

Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) For further discussion: The following topics should be further discussed: - For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured. - Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments - Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

In the last meeting, RAN4 agreed that the encoder that corresponds to the standardized test decoder is the reference encoder to be captured and/or specified for Option 3. In our view, a reference encoder would be required not only for test decoder verification but potentially for the definition of RAN4 requirements. Therefore, it should be captured in RAN4 specifications.

To verify test decoder and to define performance requirements, RAN4 needs to specify reference encoder. It should be captured in the TR conclusion as well as in the TS.

Regarding whether several encoders would be captured due to different complexity, we believe that the results generated for testing several encoder structures during the feasibility study can be used to decide about this discussion point. For instance, if the SGCS results when using reference and lower complexity encoders will be comparable, then there is no need to capture different encoders in the specifications.

The results and observations generated for testing several encoder structures during the feasibility study can be used to decide about whether several encoders need be captured in the specifications due to different complexity.

For the datasets that would be captured for Option 3, the FFS points reached in RAN4#114bis meeting cover the dataset used for training the standardized test decoder, the dataset used for training the test encoder, and the test dataset. We believe that we need to first identify which datasets can be useful to capture/specify for Option 3. Once these are identified, we need to discuss whether they should be defined in the specification or simply stored in a 3GPP database.

RAN4 to first clarify which datasets need to be captured/specified for Option 3 and then discuss how to capture/specify the identified datasets (in TR, in TS, in a 3GPP database, etc.).

What needs to be captured or specified for Option 4

- The following agreements and FFS points regarding what to capture/specify for Option 4 were reached in RAN4#114bis meeting:

Agreements - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. For further discussion: The above FFS; i.e. - Whether a dataset is captured for option 4b - Whether a reference encoder is specified for option 4a

In the last meeting, it was agreed that a dataset is specified for Option 4a. For the encoder-based approach, i.e., Option 4b, the encoder selected from Option 3 can be the candidate for the reference encoder to be specified. However, the selection criterion for the reference encoder in Option 4a and for the dataset in Option 4b is unclear.

RAN4 to further discuss the selection criterion for the reference encoder in Option 4a and the dataset in Option 4b.

Relation between RAN4 option 3 and RAN1 inter-vendor collaboration direction C

- The following FFS point regarding the relation between RAN4 option 3 and RAN1 inter-vendor collaboration direction C was reached in RAN4#114bis meeting:

For further discussion: - From the online discussion: FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions. The relation between the RAN4 options and RAN1 inter-vendor interoperability directions should be further clarified.

- We quote here the RAN1 agreements about inter-vendor collaboration direction C:

RAN1#118 Agreement Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b) - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1) RAN1#118bis Agreement - For Direction C: - It is clarified that, the purpose of a fully specified CSI generation (reconstruction) part is to enable development/training of compatible CSI reconstruction (generation) parts at the NW (UE) side. RAN1#120 Conclusion - Direction C with standardized model ensures a minimum performance requirement. - The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. RAN1#120bis Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection RAN1#120bis Agreement For inter-vendor collaboration Direction C, - Use standardized quantization codebook RAN1#120bis Conclusion - For Direction C, based on evaluation results, RAN1 concludes that scalable model structure specification and model parameters specification is feasible. - Note: RAN4 feasibility is a separate study to be confirmed by RAN4. RAN1#120bis Conclusion - RAN1 concludes that both Direction A and Direction C are feasible.

According to RAN1 agreements listed above, the fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model for performance requirements in RAN4. However, it is unclear whether and how this equivalence assumption can be useful for the RAN4 discussions about test decoder Option 3, especially that RAN1 has already concluded about the feasibility of Direction C.

In RAN1#120bis, it was concluded that Direction C is feasible and that RAN4 feasibility is a separate study to be confirmed by RAN4. Hence, we believe that RAN4 should also conduct the feasibility study of Option 3 independently from RAN1 conclusion about Direction C.

The feasibility study of RAN4 test decoder Option 3 should be concluded independently from the feasibility of RAN1 inter-vendor collaboration Direction C.

Conclusion

The following observations and proposals were made in this contribution:

Observation 1: Testing the pair of Samsung’s frozen decoder trained on mixed training dataset and Nokia’s encoder trained based on the frozen decoder and Nokia’s training dataset gives a slightly lower SGCS compared to training both encoder and decoder on the same mixed training dataset (~2.5% of SGCS reduction when testing the pair on mixed test dataset and ~1.7% of SGCS reduction when testing the pair on Nokia’s test dataset).

Observation 2: When training Samsung’s frozen decoder and Nokia’s encoder on the same mixed training dataset and testing the pair on the mixed test dataset, using lower complexity encoder structure introduces a small reduction in the SGCS results compared to using the reference CNN encoder structure (~3.2% of SGCS reduction when using the lower complexity CNN encoder and ~3.7% of SGCS reduction when using the lower complexity transformer encoder, as suggested in [R4-2505156]).

Observation 3: Based on our Option 3 Track 2 simulations using both Samsung’s and Nokia’s frozen decoders, it is feasible to train Nokia’s encoder assuming different training datasets and lower complexity encoder structures (i.e., comparable SGCS results with no significant decrease compared to when assuming same training datasets and reference encoder structure).

Proposal 1: Based on our results, Option 3 Track 2 is feasible, using both reference and lower complexity encoders.

Observation 4: Based on our Option 4 simulations, it is feasible to train Nokia’s encoder using other companies’ decoders, which are trained using Samsung’s frozen encoder, and the mixed training dataset. The SGCS values obtained from testing the trained encoder/decoder pairs using the mixed test dataset are very comparable.

Observation 5: As part of testing the interoperability between encoders and decoders in Option 4, our results show that Nokia’s encoder works well with other companies’ decoders not only when the mixed training dataset is used for all training purposes but also when using Nokia’s dataset for encoder training.

Observation 6: Although using lower complexity encoder structures reduces the SGCS values compared to when using the reference CNN encoder, the results obtained from testing the trained encoder/decoder pairs using the mixed test dataset are still very comparable within the same encoder structure.

Proposal 2: To verify test decoder and to define performance requirements, RAN4 needs to specify reference encoder. It should be captured in the TR conclusion as well as in the TS.

Proposal 3: The results and observations generated for testing several encoder structures during the feasibility study can be used to decide about whether several encoders need be captured in the specifications due to different complexity.

Proposal 4: RAN4 to first clarify which datasets need to be captured/specified for Option 3 and then discuss how to capture/specify the identified datasets (in TR, in TS, in a 3GPP database, etc.).

Proposal 5: RAN4 to further discuss the selection criterion for the reference encoder in Option 4a and the dataset in Option 4b.

Proposal 6: The feasibility study of RAN4 test decoder Option 3 should be concluded independently from the feasibility of RAN1 inter-vendor collaboration Direction C.

- References

- R4-2505156, “Simulation assumptions on AI/ML CSI compression”, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#114bis meeting, 7th – 11th April 2025, Wuhan, China.