3GPP TSG-RAN WG4 Meeting #115	R4-2507137

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	Discussion on general aspects of NR SCM for demodulation

Agenda item:	7.16.2

Document for:	Discussion

# Introduction

In RAN4#114-bis meeting, companies have agreements on common CDL model derivation and antenna configurations for alignment and comparison separately. Regarding many candidate models and test cases are on the table, the TR content should be well structured to be clearer and more readable.

In this contribution, we give some views on TR content and generalization of CDL model.

# Discussion

Until RAN4#114-bis meeting, there are at least 5 CDL based model (827 CDLC model A1, four 901 CDLC based model) and 3 TDL based model on the table for discussion. Furtherly, there are at least 4 antenna AAV configurations during the discussion. To have clearer statement and reference in the final TR, it would be better to refine the label for each configuration. Although it won’t impact the current study, it will confuse readers.

For CDLC A1 model, it is the initial CDL model in this study for a long time, but it seems not to be used for final alignment and comparison. However, it could be better to capture some results and reason why it will not be used anymore. The information could be captured in conclusion part and Annex part.

Proposal 1	Refine the naming of discussing models and AAV configurations in final TR to make it easier for reading.

Proposal 2	The study conclusion and output on initial CDLC model A1 (based on 38.827 FR1 Uma CDLC model) can captured in conclusion part and Annex part for reference.

One open issue in previous meeting is about the CDL model generalization. Companies pointed out the current CDLC model derivation is according to TR 38.827 FR1 Uma CDLC randomness reduction. In that case, most of parameters are derived based on 3.5GHz Uma scenario including AS, delay spread, Doppler, AAV etc.

If we check current TDL model approach, there are three typical models with fixed delay spread and Doppler spread (TDLA30-10, TDLB100-400 and TDLC300-100), but we can also scale delay and Doppler value according to specific scenario, such as apply different Doppler shift based on HST speed and center frequency. In that case, we can follow similar way on CDL models to have some typical delay spread and Doppler values for each CDL model, for example CDLB100-3 (100ns delay and 3km/h UE speed) and CDLC365-30 (365ns delay and 30km/h speed). The center frequency could start with 3.5GHz or 6GHz to cover whole FR1 bands. In some special cases, we can also use 2GHz to specially cover low bands.

AS is more relevant to deployment scenario such as Uma, Umi etc. TR38.827 Table 7.2-4 capture some values for Uma and Umi scenario, RAN4 could furtherly do some combinations with different CDL model and take it as typical one, for example, applying Uma AS for CDLC and Umi AS for CDLA etc.

AAV configuration could start from 1 AE subarray configuration for all FR1 bands since the performance seems OK based on current simulations with truncated CDLC model.

In summary, we think CDL model derived from this study could be general for whole frequency range if we can derive several typical combinations between model and parameters. Of course, the derived model profile and final performance could be aligned among companies.

The existing TDL model with typical delay and Doppler configurations are considered as general model for whole frequency range. But the Doppler could be scaled by center frequency according to specific scenario.

Center frequency relevant parameters for CDL model, like Doppler, AAV etc., could take typical configurations for whole frequency range.

Proposal 3	Take some typical values for AS, delay spread, Doppler spread and AAV3 for CDL models and apply it for whole frequency range. RAN4 could start from AS_desired, delay value tables in TR38.827 and Doppler calculation could base on typical frequency or highest frequency per FR.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The existing TDL model with typical delay and Doppler configurations are considered as general model for whole frequency range. But the Doppler could be scaled by center frequency according to specific scenario.

Observation 2	Center frequency relevant parameters for CDL model, like Doppler, AAV etc., could take typical configurations for whole frequency range.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Refine the naming of discussing models and AAV configurations in final TR to make it easier for reading.

Proposal 2	The study conclusion and output on initial CDLC model A1 (based on 38.827 FR1 Uma CDLC model) can captured in conclusion part and Annex part for reference.

Proposal 3	Take some typical values for AS, delay spread, Doppler spread and AAV3 for CDL models and apply it for whole frequency range. RAN4 could start from AS_desired, delay value tables in TR38.827 and Doppler calculation could base on typical frequency or highest frequency per FR.

# References

[1] 	R4-2504756, Way Forward for [114bis][320] FS_NR_demod_SCM, Nokia