3GPP TSG-RAN WG4 Meeting #115	R4-2507141

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	General view on SBFD demodulation requirements

Agenda item:	7.25.6

Document for:	Discussion

# Introduction

In WID of SBFD RP-240789 [1], following objectives are indicated for performance part.

The objectives are as follows [RAN4]:

- Specify BS RF conformance requirements to support SBFD operation at gNB

- Specify necessary RRM performance requirements to support SBFD operation

- Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In this paper, we provided our view on SBFD demodulation requirements.

# Discussion

## 2.1	Background

The current network, DL heavy TDD pattern is typical which is benefit on the high DL traffic demanding scenario. But it could be a bottle neck for UE with very high UL traffic or low latency demand. By SBFD method, the gNB could schedule a UE for UL transmission while other UEs are DL receiving, see figure below. It is considered as a full duplex technology. The corresponding issues caused by SBFD include resource confliction handling, interference migration and handling etc.

Figure 2.1-1 Typical SBFD deployment

In TR 38.858, some conclusions and recommendations are made for RAN1 and RAN4 furtherly discussion.

- For RAN1 area, four areas are mentioned including UPT (User Perceived Throughput), coverage performance, SBFD operation scheme and CLI (Cross Link Interference).

  - UPT is not relevant to demodulation requirement discussion.

  - SBFD operation scheme mainly focus on how to make it possible to do transmission and reception under current standard framework. This part would have medium impact on demodulation requirement discussion.

  - CLI include gNB-to-gNB and UE-to-UE co-channel interference handling. Many schemes are considered in TR, but down-selection is mentioned in WID. Based on the description of these schemes, more resource coordination is focused so this part have low impact on demodulation requirement discussion.

Relevant RAN1 agreements

According to recommendations in TR, RAN1 have discussions on Tx/Rx/Measurements, random access and CLI handling.

- In Tx/Rx/Measurements part, different collision cases are studied. The agreed solutions don’t introduce new physical channel format, reference signal pattern etc., which won’t impact the demodulation aspect.

- In random access part, several questions are studied, such as “whether to support RA on SBFD symbols?”, “RO configuration and SSB-RO mapping on SBFD”, “separate PRACH power control”, “enhancement for RA on unpaired spectrum” and “PRACH repetition on SBFD”. From demodulation perspective, following agreements could be relevant:

  - No new PRACH format is introduced.

  - Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

  - For SBFD aware UEs, at least support the following:

    - PRACH transmission with preamble repetitions within additional-ROs (not across additional-ROs and legacy ROs)

    - Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

  - Update the equations to determine the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration.

  - There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

- In CLI handling part, there is no agreement on gNB-to-gNB CLI handling but mainly focus on UL resource muting for PUSCH. The L1-based UE-to-UE CLI measurement and reporting part discussed the measurement and reporting on CLI-RSSI and SRS-RSRP. From demodulation perspective, following agreements on UL resource muting could be relevant:

  - For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

    - The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on 𝑀scUCI(𝑙) after excluding the UL muting resource.

  - For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions.

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions.

  - For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM

    - A comb offset {0, 1} is configured for the up to two UL muting symbols

  - If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

  - If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

  - For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, 3 dB power boosting for both PUSCH and PTRS REs if present, regardless of the PTRS density K and number of PUSCH layers L.

  - For the configuration of UL resource muting

    - Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

  - A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled.

  - For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

  - For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined according to Section 6.2.5, TS38.212 as follows                  𝑘0=(𝑘0′+𝐻+𝜏)𝑚𝑜𝑑𝑁𝑐𝑏

and 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs.

Relevant RAN4 RF agreements

In RAN4 RF discussion, operation band, channel bandwidth, declaration/applicability rule and Tx/Rx/co-existing requirement are discussed. Following agreements might impact on the demodulation requirement discussion:

- SBFD operation shall not be applied in NR-U bands, i.e., n46, n96 and n102.

- In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

- RAN4 shall capture RF requirements in SBFD-capable BS in TS 38.104. To use a separate chapter for requirements for SBFD-capable BS in TS 38.104 is the baseline.

- The restriction of SCS values for UL and DL subbands:

  - It is RAN4 common understanding the same SCS value should be used for UL and DL subbands

  - The restriction if any should be given in RAN2 spec, rather than in RAN4 spec.

- For SBFD, BS type 1-C is precluded.

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

  - The supported channel bandwidth(s) is declared by gNB vendors

  - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

  - The supported channel bandwidth shall be no less than X MHz

    - X = 40 as the default value

    - For band n39, X = 20MHz is used instead.

## 2.2	Demodulation requirements

According to the background information of RAN1 and RAN4 discussion above, there is no DL physical channel format or feature enhancement for SBFD scenario. There are two types of UE discussed including SBFD-transparent UE and SBFD-aware UE. For both UE types, the new resource scheduling and muting are introduced. For SBFD-aware UE, some new indication parameters on physical channel resources are introduced. However, all these improvements won’t impact the demodulation requirements. In that case, RAN4 don’t need to consider new UE demodulation requirements for SBFD scenario.

Proposal 1	Don’t consider UE demodulation requirements for SBFD.

PUSCH requirements

For PUSCH, the UCI multiplexing on PUSCH, PUSCH repetition type A and B, and UL TBoMS with UL resource muting for SBFD are discussed in RAN1.

According to agreements, the DM-RS and PT-RS overlapping with muting symbol will be kept and might boost 3dB for both PUSCH and PT-RS in some cases. Furthermore, PUSCH with CP-OFDM would use comb offset to avoid confliction with muting symbols. All these improvements won’t cause more critical situation for receiver, and the normal PUSCH demodulation won’t be impact in that case.

For UCI multiplexing on PUSCH, new UCI rate matching method is agreed regarding muting resources. But the UCI bits mapping method is same as legacy way.

For PUSCH repetition type A and B with two muting configurations, agreements confirm that using same time location for all repetitions according to muting symbol numbers. Also regarding the agreements on DM-RS above, the demodulation algorithm for repetition won’t be changed comparing to legacy implementation.

For PUSCH with TBoMS, a new calculation method for the number of multiple slots is introduced. It won’t impact the legacy TBoMS demodulation algorithm.

In summary, no new PUSCH format and feature is introduced by SBFD and no impact on demodulation algorithm. In that case, no new PUSCH requirement is needed.

Proposal 2	Don’t introduce new PUSCH demodulation requirements for SBFD.

For the features mentioned above, Rel-15 PUSCH requirements are mandatory for all BS types and other features’ tests depend on manufactory declaration. If the declaration for SBFD can be captured in the same declaration table as other features, it won’t be a problem for a SBFD capable BS to declare other features and pass corresponding tests.

It won’t be a problem for a SBFD capable BS to declare other features and pass corresponding tests if the declaration for SBFD can be captured in the same declaration table as other features.

PUCCH requirements

For normal PUCCH transmission, RAN1 introduce a new calculation method for the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration in random access stage. The general thinking is to replace the UL BWP PRB indices by UL usable PRB indices. After this update, the lowest PRB indices of the first and second hop might be different from legacy configuration, but the index offset is limited.

For PUCCH repetition, the transmission limitations are defined for validation of repetition slot and avoid confliction with SSB in SBFD. But the basic transmission procedure and receiver algorithm are not impacted.

In summary, the PUCCH demodulation performance impact could be ignored considering the changing for SBFD scenario in RAN1 discussion. In that case, no new demodulation requirement is needed.

Proposal 3	Don’t introduce new PUCCH demodulation requirements for SBFD.

PRACH requirements

In RAN1 discussion, no new PRACH format is introduced, but additional RO is introduced for SBFD random access stage and PRACH repetition is also possible within additional RO or legacy RO. The new defined behavior won’t impact the PRACH demodulation algorithm. In that case, no new requirement is needed for SBFD.

Proposal 4	Don’t introduce new PRACH demodulation requirements for SBFD.

Specification impact

Regarding RAN4 RF agreements on declaration and requirement capture method (agreement 3), RF requirements will be captured in a new chapter as baseline. But this approaching is not finally agreed. It indicates that SBFD is a new capability for BS which could depend on declaration. If it is agreed in final CR, it could be possible to have separate sub-clause for BS demodulation requirements under same chapter, see below example. This approach could explicitly distinguish SBFD BS performance requirements from other BS type requirements. Using this new sub-clause could give clear applicability rule to indicate which legacy BS demodulation requirement could be reused for the tests.

- Requirements for SBFD-capable BS

  - Conducted transmitter characteristics

  - Conducted receiver characteristics

  - Conducted performance requirements

  - Radiated transmitter characteristics

  - Radiated receiver characteristics

  - Radiated performance requirements

RF sessions would introduce a new chapter for all SBFD-capable BS requirements.

However, if RAN4 finally agree not to introduce new SBFD demodulation requirements, a new sub-clause might not be needed. In that case, the question would be how to choose proper demodulation requirements from existing requirements. Rel-15 requirements are mandatory, and all other enhanced features have requirement applicability rules depending on manufactory declaration. Then the SBFD capable BS could declare supporting features and take corresponding requirements. But it might not so clear to the readers and testers. The better way seems to still have sub-clause for demodulation and adding new applicability rule for SBFD to indicate which existing requirements are mandatory and which requirements could depend on declarations.

SBFD-capable BS could apply existing demodulation requirements based on manufactory declarations.

Proposal 5 	New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree NOT to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured.

# Conclusions

In the previous sections we made the following observations:

Observation 1	It won’t be a problem for a SBFD capable BS to declare other features and pass corresponding tests if the declaration for SBFD can be captured in the same declaration table as other features.

Observation 2	RF sessions would introduce a new chapter for all SBFD-capable BS requirements.

Observation 3	SBFD-capable BS could apply existing demodulation requirements based on manufactory declarations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Don’t consider UE demodulation requirements for SBFD.

Proposal 2	Don’t introduce new PUSCH demodulation requirements for SBFD.

Proposal 3	Don’t introduce new PUCCH demodulation requirements for SBFD.

Proposal 4	Don’t introduce new PRACH demodulation requirements for SBFD.

Proposal 5 	New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree NOT to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured.

# References

[1] 	RP-243285: Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom, RAN #106, Madrid, Spain