3GPP TSG-RAN WG4 Meeting #115	R4-2507142

# St Julian’s, Malta, May 19th – 23rd 2025

Source:	Ericsson

Title:	General view on ATG enhancement demodulation requirements

Agenda item:	7.12.7

Document for:	Discussion

# Introduction

In RAN4#114-bis meeting, the Rel-19 NR ATG enhancement UE and BS demodulation part are discussed. Some agreements are achieved [2].

In this paper, we will continue discuss the UE demodulation requirements for intra-band co-located and inter-band co-located DL CA enhancement, and the necessary of UL MIMO 2Tx for BS demodulation requirements.

# Discussion

## 2.1 	UE demodulation

### 2.1.1 	DL CA

For the downlink CA, it’s agreed from the WID that the following CA will be considered for the requirements:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

Since only intra-band contiguous CA and inter-band CA are within the WID, we agree with the following proposal [2] for the CA configuration selection rules:

Issue 1-1-4: CA configuration selection rules - Proposal 1: - Define the CA configuration selection rules as CA capability; Step 1; Step 2; Step 3; Step 4. CA_C; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; N/A; N/A. CA_AX; Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1.; Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2.; Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3..

Proposal 1	 Consider the following CA configuration selection rules:

| CA capability | Step 1 | Step 2 | Step 3 | Step 4 |
| --- | --- | --- | --- | --- |
| CA_C | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | N/A | N/A |
| CA_AX | Select the CA configurations with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 1. | Select the CA configurations with the largest number of bands and with the maximum number of CCs, for which the supported maximum number of MIMO layers is not lower than 2. | Select any one of CA configurations, which contain CA bandwidth combination with the largest aggregated channel bandwidth and supported maximum data rate is not lower than the tested date rate, among all the selected CA configurations from Step 3. |

Together with the consideration of test applicability rules, we in general agree with proposal 1 below but with some modification to align the format with the existing CA applicability rules in TS38.101-4.

Issue 1-1-5: Test applicability rules - Proposal 1: - Define the test rules as: Tests; CA capability where the tests apply; CA configuration from the selected CA capability where the tests apply; CA Bandwidth combination to be tested in priority order; PCell CC configuration. FDD 15 kHz + FDD 15 kHz; CA_C, CA_AX; Table TBD; Largest aggregated CA bandwidth combination; Any of CCs. TDD 30 kHz + TDD 30 kHz; CA_C, CA_AX; Table TBD; Largest aggregated CA bandwidth combination; Any of CCs. FDD 15 kHz + TDD 30 kHz; CA_AX; Table TBD; Largest aggregated CA bandwidth combination; FDD CC if supported, otherwise TDD CC.

Proposal 2	Consider the following test applicability rules for ATG DL CA;

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order | PCell CC configuration |
| --- | --- | --- | --- | --- |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_C, CA_AX | Table TBD | Largest aggregated CA bandwidth combination | Any of CCs |
| Test x in Clause 5.2A.2.x and 5.2A.3.x | CA_AX | Table TBD | Largest aggregated CA bandwidth combination | FDD CC if supported, otherwise TDD CC |

Regarding the rank of DL CA requirements, we propose to consider only rank 1 since rank 2 with MIMO is introduced in Rel-19 as a new feature which has its own corresponding requirements, it is not necessary to connect two new features with one requirement. Plus, RAN4 only defined downlink requirements with rank 1 for Rel-18 ATG single carrier requirements. Therefore, we propose to consider only rank 1 for the newly introduced Rel-19 ATG downlink CA requirements.

Proposal 3	Consider only rank 1 for the downlink CA requirements

As for the MCS, it’s been agreed to evaluate the following candidates:

Issue 1-1-3-3: MCS Agreement: - 16QAM MCS 13, 64QAM MCS 22, 256QAM MCS 24 for initial simulation

Based on our simulation results below, we observed good performance with configurations of MCS13 (16QAM) and MCS22 (64QAM).

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 10 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | AWGN | 220Hz | 2x2, ULA Low | 70 | 3.40 |
| 10 | R.PDSCH.1-3.6 FDD | 64QAM, 0.65 | AWGN | 220Hz | 2x2, ULA Low | 70 | 10.62 |

Considering that the propagation channel is chosen to be as AWGN plus higher modulations are more likely to be scheduled, we propose to consider MCS22 (64QAM) for ATG downlink CA requirements.

Proposal 4 	Consider 64QAM MCS22 for ATG downlink CA requirements

### 2.1.2		DL MIMO

Another objective is to specify demodulation requirements for the support of DL MIMO for UE with limited test cases.

RAN4 only considered 1 layer for ATG PDSCH demodulation requirements in the previous release since companies had concerns regarding potential inter-layer interference brought by multiple layers transmission causing by the agreed high correlation channel. Therefore, we propose to evaluate the performance of 2 layers first, configuring with both 2Rx and 4Rx, to see the feasibility of introducing requirements for 2 MIMO layers.

Currently, there are several candidates [2] for the MCS value, together with the consideration of XP-low antenna correlation.

Issue 2-1-4: MCS Agreement: - For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 Issue 2-1-1: Antenna configuration Agreement: - 2T2R and 2T4R FFS the antenna correlation: - Proposal 1: XP-low

To facilitate the parameter assumption, we provided our initial simulation results based on the legacy propagation condition but different MCS and antenna configurations.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 6.4 |
| 2 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 13.5 |
| 3 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 22.46 |
| 4 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 3.4 |
| 5 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 10.5 |
| 6 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 19.45 |

Note 1: Apply the static propagation condition specified in TS 38.101-4 B.1.

From the simulation results we can observe that the performance of configuring three different MCS are all good enough for the requirement. We also observed expected MIMO gain on the maximum throughput over the result of rank 1, see figures below.

Figure 2.1.2-1 Performance with 16QAM

Figure 2.1.2-2 Performance with 64QAM

Figure 2.1.2-3 Performance with 256QAM

Thus, we have verified that applying MIMO with 2 layers to ATG scenario is reasonable. For the requirement, we propose to consider 64QAM as its SNR point of 70% maximum throughput is within a preferred range.

Proposal 5	Consider 64QAM for the rank 2 requirements

Proposal 6 	Consider both 2x2 and 2x4 for the antenna configurations

As for the issue of RI requirements, RI is an estimation of the channel rank, which is determined by actual multipath fading. However, for ATG scenario, we have agreed on using non-fading channels. Thus, we propose not to consider RI requirements for ATG scenario.

Proposal 7 	Do not consider RI requirements

## 2.2	BS demodulation

In the last meeting, all companies agree to reuse Rel-15 2Tx PUSCH requirements for ATG BS as mandatory tests. As for the detailed reusing cases, it is FFS in WF. By checking Rel-15 requirements in TS38.104, only Clause 8.2.1 have 2Tx requirements. In that case, the statement of WF is feasible.

Issue 3-1-1: Whether to reuse existing requirements for ATG UL MIMO

Agreement:

- The legacy PUSCH test cases in Clause 8.2.1with 2Tx 2 layer will be applied for ATG

FFS:

Whether other legacy test cases are needed.

No Rel-15 2Tx BS demodulation requirements in other clauses of TS38.104 expect 8.2.1.

Issue 3-1-2: Whether to introduce new test cases for ATG UL MIMO

- Proposals

  - Proposal 1:

    - incremental PUSCH 2 layers test cases need to be introduced

  - Proposal 2:

    - Do not introduce new 2Tx PUSCH requirements for ATG scenario

The next issue is whether considering new test case for ATG BS.

By observing companies’ proposals in RAN4#114-bis, the proponent of introducing new requirement would reuse almost all Rel-15 requirement configurations except channel model. Thus, we need to check if there is any benefit to consider a new requirement on top of Rel-15 2Tx PUSCH requirements.

MCS

In Rel-18 ATG PUSCH demodulation requirements, higher MCS than Rel-15 are introduced regarding high capacity is expected by ATG scenario. The motivation is to avoid duplicated cases as Rel-15 by just changing fading channel to AWGN.

In Rel-15 2Tx PUSCH requirements, only MCS2 and MCS16 are considered. If we follow similar approach, higher MCS might be better to be considered for ATG scenario, but the feasibility needs to be checked by simulation.

Channel model

Generally, the AWGN channel might not be suitable for higher layer transmission due to high correlation between layers, especially with low number of Rx antennas. For ATG scenario, the phase of polarization can be easily shifted due to changing of airline footprint. In current demodulation requirements, only static AWGN channel is assumed, then it could be possible to keep polarization matching between UE and BS to support higher layer transmission.

The channel matrix of AWGN is normally a unit matrix, but some companies propose to use different channel matrix for 2Tx AWGN channel H= 1j1-j to have better performance.

Furthermore, XP-low correlation is proposed, but there is no XP-low correlation matrix defined because it is same as ULA low correlation matrix.

In Rel-15 2Tx PUSCH requirements, the fading channel TDLB100-400 and TDLC300-100 are used for MCS2 and MCS16 separately. If we only focus on relatively high MCS configuration, such as MCS16, we can replace TDLC300-100 by AWGN channel to check the performance.

Our simulation results [2] show that the target SNR of MCS16 or even MCS20 are OK with 2Tx2Rx configuration under proposed AWGN channel matrix and low correlation matrix.

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | Normal | AWGN + 500Hz Low | 70 % | 16 | pos1 | 8.7 |
|  |  |  |  |  | 20 | pos1 | 11.7 |

The target SNR of MCS16 and MCS20 are OK with 2Tx2Rx under ATG channel matrix in 38.101-4 and low correlation matrix.

To address the operator’s concern and effort of vendors, limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements.

Proposal 8	Limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements.

Issue 3-1-3: Antenna configuration for incremental test

- Proposals

  - Proposal 1:

    - 2T2R, 2T4R and 2T8R with X-pol low correlation

  - Proposal 2:

    - 2T2R, 2T4R and 2T8R

  - Proposal 3:

    - 2T2R

Based on the simulation, it seems 2Rx is feasible for 2Tx under existing AWGN channel model for ATG. Comparing to higher Rx antenna configurations, 2Rx would be the worst case. Under AWGN channel, once 2Rx performance is tested, the 4Rx/8Rx performance could be predicable. Another benefit of testing 2Rx is that the test setup is much simpler than 4/8Rx. Furthermore, only 2Rx is considered in existing ATG 1Tx PUSCH demodulation requirements, it would be better to keep same configuration.

As for correlation matrix, low correlation should be necessary to achieve reasonable target SNR. But as we mentioned above that the X-pol low should be the same as ULA low if we don’t consider the common 45o rotation in X-pol comparing to ULA. In existing specification, there is no X-pol low matrix used on both UE and BS sides.

The AWGN performance of 4/8Rx could be predictable once 2Rx performance is tested.

The existing ATG 1Tx PUSCH demodulation requirements only have 2Rx antenna configuration.

The impact of X-pol low correlation is same as ULA low correlation.

Proposal 9	Take antenna configuration as 2T2R with low correlation.

Issue 3-1-4: Propagation condition for incremental test

- Proposals

  - Proposal 1:

    - For incremental test case, use AWGN+200Hz for FDD, and AWGN+500Hz for TDD. The static propagation condition model from TS38.101-4 can be taken as the reference

The AWGN+200Hz for FDD, and AWGN+500Hz for TDD can be reused. The static propagation condition model from TS38.101-4 is feasible for 2Tx performance check. In current BS specification, there is no explicit static channel matrix captured in Annex. If RAN4 agree to use channel matrix from 38.101-4, it would be better to capture it in Annex. But the 1Tx static channel matrix should also be added either.

Proposal 10	Take channel model AWGN+200Hz for FDD, and AWGN+500Hz for TDD in ATG PUSCH demodulation requirements.

Proposal 11	If RAN4 agree to use H= 1j1-j as 2Tx static channel matrix, it would be better to capture it in BS specification. RAN4 to furtherly consider if 1Tx static channel matrix should be also explicitly indicated in BS specification.

Issue 3-1-5: Bandwidth & SCS for incremental test

- Proposals

  - Proposal 1:

    - 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS

To reduce the simulation effort, only minimum channel bandwidth per SCS should be enough for requirement definition based on previous experience in all enhanced features. Existing ATG 1Tx PUSCH demodulation requirements only have minimum channel bandwidth. It would be better to have same coverage.

Proposal 12	Consider 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS for ATG 2Tx PUSCH demodulation requirements.

Issue 3-1-6: Rank for incremental test

- Proposals

  - Proposal 1:

    - Rank 2

Issue 3-1-8: DMRS port for incremental test

- Proposals

  - Proposal 1:

    - DMRS port 2 {0,1}

In BS demodulation requirements, 2Tx always means 2 layers. The Tx diversity condition has never been considered in existing requirements. The corresponding DM-RS port should be {0, 1} which is used in existing 2Tx PUSCH requirements.

Proposal 13	Only consider rank 2 and DM-RS port 2 {0, 1} for ATG 2Tx PUSCH demodulation requirements.

Issue 3-1-7: MCS for incremental test

- Proposals

  - Proposal 1:

    - 16QAM Table 1 MCS12, 64QAM Table 1 MCS 20 and 256QAM Table 2 MCS 20 for initial simulation

  - Proposal 2:

    - consider low modulation order for high rank transmission

  - Proposal 3:

    - MCS 16

In Rel-15 PUSCH 2Tx requirements, only QPSK MCS2 and 16QAM MCS16 in MCS Table 1 are tested due to very high target SNR is expected for 64QAM and higher modulation levels under fading channel. For AWGN model used in ATG demodulation, the inter-layer interference could be ignored and the performance of 2Tx could be predicted once 1Tx performance is tested with same MCS (e.g., 3dB gain to 1Tx) based on our simulations [2].

In Rel-18 ATG PUSCH 1Tx requirements, 64QAM MCS 28 in MCS Table 1 and 256QAM MCS22 in MCS Table 2 are introduced as incremental tests on top of Rel-15 requirements. In that case, most of typical MCS configurations have been covered already.

For 2Tx ATG PUSCH, only one MCS could be enough for incremental test. As we discussed above, there might not be ideal static channel as in simulation for ATG scenario. It would be challenging to support high MCS on high layer transmission even the target SNR in simulation is low, such as 256QAM. As for 16QAM and 64QAM, 16QAM MCS16 is the highest MCS in Table 1 which has been tested by Rel-15. To avoid duplicated 16QAM test as Rel-15 by using simpler AWGN model, 64QAM with low MCS index could be considered.

By checking existing Annex for FRC table, following 64QAM MCS indexes are introduced for demodulation specification (38.104). It would be better not to introduce new code rate to existing specification to reduce effort.

- A.5 64QAM MCS20 R=567/1024. Used for Rel-15 PUSCH 1 layer transmission.

- A.10 64QAM MCS19 R=517/1024. Used for FR2 high speed train PUSCH and UL TA.

- A.11 64QAM MCS17 R=438/1024. Used for Rel-18 FR1 PUSCH with 4Tx.

- A.13 64QAM MCS28 R=948/1024. Used for Rel-18 ATG FR1 PUSCH 1Tx.

It seems MCS17 or MCS20 could be considered for ATG 2Tx PUSCH requirements. We slightly prefer to take MCS20 which used in Rel-15.

Proposal 14	RAN4 could consider 64QAM MCS20 for ATG 2Tx PUSCH demodulation requirements.

Issue 3-1-9: TPMI for incremental test

- Proposals

  - Proposal 1:

    - TPMI index 0

Issue 3-1-10: Others

- Proposals

  - Proposal 1:

    - Type A

    - HARQ retransmission

    - 70% of maximum throughput

  - Proposal 2:

    - Others same as existing Rel-15 BS PUSCH demodulation requirement with 2Tx

In current ATG 1Tx PUSCH demodulation requirements, only PUSCH type A is considered. It would be better to keep same configuration for 2Tx either. All other configurations not discussed above could follow 1Tx ATG PUSCH demodulation requirements either.

Proposal 15	Use following configurations for ATG 2Tx PUSCH demodulation requirements.

- TPMI index 0

- Type A

- HARQ retransmission

- 70% of maximum throughput

# Conclusions

In the previous sections we made the following observations:

Observation 1	No Rel-15 2Tx BS demodulation requirements in other clauses of TS38.104 expect 8.2.1.

Observation 2	The target SNR of MCS16 and MCS20 are OK with 2Tx2Rx under ATG channel matrix in 38.101-4 and low correlation matrix.

Observation 3	The AWGN performance of 4/8Rx could be predictable once 2Rx performance is tested.

Observation 4	The existing ATG 1Tx PUSCH demodulation requirements only have 2Rx antenna configuration.

Observation 5	The impact of X-pol low correlation is same as ULA low correlation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	 Consider the following CA configuration selection rules:

Proposal 2	Consider the following test applicability rules for ATG DL CA;

Proposal 3	Consider only rank 1 for the downlink CA requirements

Proposal 4 	Consider 64QAM MCS22 for ATG downlink CA requirements

Proposal 5	Consider 64QAM for the rank 2 requirements

Proposal 6 	Consider both 2x2 and 2x4 for the antenna configurations

Proposal 7 	Do not consider RI requirements

Proposal 8	Limited new test cases could be considered for ATG 2Tx PUSCH demodulation requirements.

Proposal 9	Take antenna configuration as 2T2R with low correlation.

Proposal 10	Take channel model AWGN+200Hz for FDD, and AWGN+500Hz for TDD in ATG PUSCH demodulation requirements.

Proposal 11	If RAN4 agree to use H= 1j1-j as 2Tx static channel matrix, it would be better to capture it in BS specification. RAN4 to furtherly consider if 1Tx static channel matrix should be also explicitly indicated in BS specification.

Proposal 12	Consider 5MHz for FDD 15kHz SCS and 10MHz for TDD 30kHz SCS for ATG 2Tx PUSCH demodulation requirements.

Proposal 13	Only consider rank 2 and DM-RS port 2 {0, 1} for ATG 2Tx PUSCH demodulation requirements.

Proposal 14	RAN4 could consider 64QAM MCS20 for ATG 2Tx PUSCH demodulation requirements.

Proposal 15	Use following configurations for ATG 2Tx PUSCH demodulation requirements.

	TPMI index 0

	Type A

	HARQ retransmission

	70% of maximum throughput

# References

[1] 	RP-243285: Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom, RAN #106, Madrid, Spain

[2]	R4-2504760 WF on NR_ATG_enh_demodulation requirements, CMCC, RAN4 #114bis, Wuhan, China