# 3GPP TSG-RAN WG4 Meeting #115                              R4-2507144

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	5.25.3

Source: 	CMCC

Title: 			 Discussion on LS on Collision between MUSIM Gap and Measurement Gap

Document for:	Discussion

- Introduction

In last meeting, RAN2 sent LS on collision between MUSIM Gap and Measurement Gap to RAN4 [1], the content are duplicated as following.

- RAN2 LS on Collision between MUSIM Gap and Measurement Gap (R4-2505315/R2-2503163) Overall description On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases. Action To RAN4: RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

This contribution provides discussion on this issue.

- Discussion

RAN2 ask RAN4 on whether UE requirement is needed for the collision between the MUSIM Gap and Measurement Gap for the collowing cases:

Case 1: MUSIM gap without priority and the measurement Gap with priority

Case 2: MUSIM gap with priority and the measurement Gap without priority

Case 3: MUSIM gap without priority and the measurement Gap without priority

In our views, case 2 are considered in RAN4 amd requirements are defined. In high level, the collision can be solved by MGRP. In detail, according to TS38.133 9.1.10.5, for the case that there is aperiodic MUSIM gap, the measurement gap occasion colliding with an aperiodic MUSIM gap shall be dropped. Otherwise, collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps:

-	If ‘keep solution’ in clause 9.1.10.3 is not configured, collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply to these gaps. Any collisions between MUSIM gaps shall be addressed as specified in clauses 9.1.10.3 and 9.1.10.4.

-	Otherwise if ‘keep solution’ in clause 9.1.10.3 is configured, keep solution is used for the remaining collided and non-dropped MUSIM gaps, after resolving the collisions between measurement gaps and MUSIM gaps based on their MGRP following the above mentioned collision rule.

For case 1 and case 3 where periodic MUSIM gaps are not configured priority, we think the collsion can be solved by based on MGRP of the colliding MUSIM gaps. In detail, for each collision between MUSIM gaps, the occasion of the MUSIM gap with longer MGRP shall be kept and the occasion of the MUSIM gap with shorter MGRP shall be dropped. If the colliding MUSIM gaps have the same MGRP, there is no requirements.

Proposal 1: for collision between periodic MUSIM gaps where no priprity configured, it is proposed that the collision are handled based on MGRP of the colliding MUSIM gaps. In detail, for each collision between MUSIM gaps, the occasion of the MUSIM gap with longer MGRP shall be kept and the occasion of the MUSIM gap with shorter MGRP shall be dropped. If the colliding MUSIM gaps have the same MGRP, there is no requirements.

Proposal 2: for LS on collision between MUSIM Gap and measurement Gap. It is proposed to reply RAN2 as following

- If there is aperiodic MUSIM gap, the measurement gap occasion or periodic MUSIM gap occasion colliding with the aperiodic MUSIM gap shall be dropped.

- Otherwise, collisions between MUSIM gaps and measurement gaps where no priorities are configured for MUSIM gaps and/or measurement gaps are handled based on MGRP of the colliding gaps, and the MUSIM gap or measurement gap with longer MGRP shall be kept, the MUSIM gap or measurement gap with shorter MGRP shall be dropped

- If the colliding MUSIM gap and measurement gap have the same MGRP, there is no requirements

- For collisions between MUSIM gaps

- If keep solution is configured, all MUSIM gaps are kept

- If keep solution is not configured, and priority is not configured, collisions between MUSIM gaps are handled based on MGRP of the colliding MUSIM gaps, and the MUSIM gap with longer MGRP shall be kept, the MUSIM gap with shorter MGRP shall be dropped

- If the colliding MUSIM gaps have the same MGRP, there is no requirements

- Conclusion

This contribution provides discussion on RAN2 LS on Collision between MUSIM Gap and Measurement Gap. The proposals are:

Proposal 1: for collision between periodic MUSIM gaps where no priprity configured, it is proposed that the collision are handled based on MGRP of the colliding MUSIM gaps. In detail, for each collision between MUSIM gaps, the occasion of the MUSIM gap with longer MGRP shall be kept and the occasion of the MUSIM gap with shorter MGRP shall be dropped. If the colliding MUSIM gaps have the same MGRP, there is no requirements.

Proposal 2: for LS on collision between MUSIM Gap and measurement Gap, it is proposed to reply RAN2 as following

- If there is aperiodic MUSIM gap, the measurement gap occasion or periodic MUSIM gap occasion colliding with the aperiodic MUSIM gap shall be dropped.

- Otherwise, collisions between MUSIM gaps and measurement gaps where no priorities are configured for MUSIM gaps and/or measurement gaps, are handled based on MGRP of the colliding gaps, and the MUSIM gap or measurement gap with longer MGRP shall be kept, the MUSIM gap or measurement gap with shorter MGRP shall be dropped

- If the colliding MUSIM gap and measurement gap have the same MGRP, there is no requirements

- For collisions between MUSIM gaps

- If keep solution is configured, all MUSIM gaps are kept

- If keep solution is not configured, and priority is not configured, collisions between MUSIM gaps are handled based on MGRP of the colliding MUSIM gaps, and the MUSIM gap with longer MGRP shall be kept, the MUSIM gap with shorter MGRP shall be dropped

- If the colliding MUSIM gaps have the same MGRP, there is no requirements

- Reference

- [1] R4-2505315 (R2-2503163), LS on Collision between MUSIM Gap and Measurement Gap