3GPP TSG-RAN WG4 Meeting #115	R4-2507151

# St. Julian's, Malta, 19th – 23rd May, 2025

Source: 	Huawei, Hisilicon

Title: 	Discussion on introducing 3 Tx and 4 Tx switching

Agenda Item:	5.1.1

Document for:	Discussion

1	Introduction

In the last meeting, Tx switching for Rel-19 TEI made significant progress, and a LS [1] including the following three scenarios was sent to RAN1. RAN4 agreed to support at least scenario 1 in Rel-19 TEI. Whether RAN4 will support scenario 1a and scenario 2 in Rel-19 TEI is pending on RAN1 decision in RAN1#121 meeting.

- Scenario 1. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to 'dualUL’ - Case 1-1: Two antenna ports on Band A and one antenna port on Band B - Case 1-2: One antenna port on Band A and two antenna ports on Band B - Scenario 1a. Switching between the following 2 cases for 3Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’ - Case 2-1: Three antenna ports on Band A and zero antenna port on Band B - Case 2-2: Zero antenna port on Band A and three antenna ports on Band B - Scenario 2. Switching between the following 2 cases for 4Tx UEs configured with uplinkTxSwitchingOption set to ‘switchedUL’ - Case 3-1: Four antenna ports on Band A and zero antenna port on Band B - Case 3-2: Zero antenna port on Band A and Four antenna ports on Band B

This contribution further analyses Scenarios 1a and 2, specifically considering NUL+SUL and finding no impact on RAN1 specifications.

2	Discussions

In the previous meeting's discussion, the primary concern of other companies revolved around the potential impact on RAN1. This directly resulted in the agreement to introduce only Scenario 1, with Scenarios 1a and 2 being deferred pending feedback from RAN1. Referring to our previous paper [2], Scenario 2 has been further subdivided into CA and SUL, and Scenario 1a can also be.

| uplinkTxSwitchingOption set to 'switchedUL' | Example 1: (TDD+SUL) | Example 2: (TDD+TDD) | Example 3: (TDD+FDD) |
| --- | --- | --- | --- |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx |
| Case2 | 0Tx+2/4Tx | 0Tx+2/4Tx | 0Tx+2/4Tx |

Regarding CA, as detailed in subclause 6.1.6.2.0 of TS 38.214 [3] (copied below), a maximum of 2Tx is supported. Consequently, for Scenarios 1a and 2 with more than 2Tx switching between NUL and NUL, RAN1 specification modifications are necessary. Therefore, when Scenarios 1a and 2 involve switching band combinations configured with CA, this necessitates waiting for RAN1 to provide feedback on the acceptability of the RAN1 impact.

| ##### 6.1.6.2.0 Uplink switching with two uplink bands For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. <--texts omitted--> - If uplinkTxSwitching-2T-Mode is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. |  |
| --- | --- |

In contrast, for SUL, subclause 6.1.6.3 of TS 38.214 [3] (copied below) shows that there is no restrictions on the number of TX ports. Thus, Scenarios 1a and 2 involving more than 2Tx switching between SUL and NUL do not necessitate RAN1 specification changes.

Observation 1: Given that supporting scenario 1a and scenario 2 with an SUL band does not necessitate any RAN1 specification modifications, RAN4 can introduce this case.

#### 6.1.6.3 Uplink switching with two uplink bands for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

Building upon the aforementioned observations, we further concentrate our analysis on the RAN4 specification impacts for scenarios 1a and 2 where one band is an SUL band, given its independence from RAN1 specification impacts. In the analysis of SUL band combinations, our typical default is to consider 1Tx or 2Tx. Despite the R18 WID [4]'s explicit prioritization of 4Tx support for TDD bands, we understand the potential for its extension to FDD/SUL bands. Significantly, the final CR [5] does not impose any duplex mode restrictions on 4Tx, implying its possible application to FDD and SUL. Similarly, the ongoing R19 MIMO 3Tx WID [6] also do not include any duplex mode restrictions.

Observation 2：R18 4T WID and R19 MIMO 3Tx WID do not include any duplex mode restrictions

The UE's TX number is generally understood as the number of TXs it can operate concurrently. The R19 discussion regarding a 3TX inter-band CA combination, for example, implies a configuration of one band with 1 TX and another with 2 TXs, requiring adherence to concurrent performance metrics. This definition, however, is not applicable to SUL or uplinkTxSwitchingOption set to 'switchedUL', which do not support concurrent transmission. As a result, for UEs with more than 2 TXs, TX switching when one of the two bands is an SUL band can be decoupled from the TX number, meaning there is no need to explicitly specify the TX number supported by each band in separate subclause of the specification, because SUL is basically carrier selection and indepedent of Tx number being supported for each band.

Observation 3：For uplinkTxSwitchingOption set to 'switchedUL' or SUL, TX switching can be decoupled from the number of TXs, because it is basically carrier selection and indepedent of Tx number being supported for each band.

To avoid redundancy, we propose addressing the following cases within a single subclause of RAN4 spec, similar to the RAN1 specification for SUL, rather than introducing them separately. An example subclause capturing the following cases can be found in Appendix, largely consistent with subclause 6.3C.3.2, except for the removal of the limitation on TX number in each band.

| uplinkTxSwitchingOption set to 'switchedUL' | TDD/FDD+SUL |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx | 3Tx+0Tx | 3Tx+0Tx |
| Case2 | 0Tx+2Tx | 0Tx+3Tx | 0Tx+4Tx | 0Tx+2Tx | 0Tx+3Tx |

Proposal 1: Introduce Tx switching for more than 2Tx when one of the two bands is an SUL band in Rel-19 TEI considering no impact on the RAN1 spec.

- Addressing the following examples within a single subclause of RAN4 spec, similar to the RAN1 spec for SUL, rather than introducing them separately, because SUL is basically carrier selection and indepedent of Tx number being supported for each band.

3	Summary

This contribution extends the analysis to Scenarios 1a and 2, with a specific focus on SUL, and identifies no impact on RAN1 specifications.

Observation 1: Given that supporting scenario 1a and scenario 2 with an SUL band does not necessitate any RAN1 specification modifications, RAN4 can introduce this case without waiting for feedback from RAN1.

Observation 2：R18 4T WID and R19 MIMO 3Tx WID do not include any duplex mode restrictions.

Observation 3：For uplinkTxSwitchingOption set to 'switchedUL' or SUL, TX switching can be decoupled from the TX number, and it just needs to be consistent with the single band's supported TX number.

Proposal 1: Introduce Tx switching for more than 2Tx when one of the two bands is an SUL band in Rel-19 TEI considering no impact on the RAN1 spec.

- Addressing the following examples within a single subclause of RAN4 spec, similar to the RAN1 specification for SUL, rather than introducing them separately, because SUL is basically carrier selection and indepedent of Tx number being supported for each band.

| uplinkTxSwitchingOption set to 'switchedUL' | TDD/FDD+SUL |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx | 3Tx+0Tx | 3Tx+0Tx |
| Case2 | 0Tx+2Tx | 0Tx+3Tx | 0Tx+4Tx | 0Tx+2Tx | 0Tx+3Tx |

4	References

- [1] R4-2505221, LS on UL Tx switching for TEI19, MeidaTek inc. 3GPP TSG-RAN WG4 Meeting #114bis.

- [2] R4-2504180, Discussion on introducing Tx switching for 4Tx FWA, Huawei, Hisilicon. 3GPP TSG-RAN WG4 Meeting #114bis.

- [3] 3GPP TS 38.214: "NR; Physical layer procedures for data"

- [4] RP-233186, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), Huawei, Hisilicon. 3GPP TSG-RAN Meeting #102.

- [5] R4-2321787, draft Big CR to TS 38.101-1: 4Tx requirements, Huawei, Hisilicon. 3GPP TSG-RAN WG4 Meeting #109.

- [6] RP-242394, WID revision: NR MIMO Phase 5, Samsung. 3GPP TSG-RAN Meeting #105.

5	Appendix

6.3C.3.6	Time mask for switching between two uplink carriers with more than two transmit antenna connectors

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10].

The switching periods described in Figure 6.3C.3.6-1a and Figure 6.3C.3.6-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [uplinkTxSwitchingPeriodLocation] [7], and the length of uplink switching period X is less than the value indicated by UE capability [uplinkTxSwitchingPeriod2T2T].

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling [uplinkTxSwitchingPeriodLocation] is ignored by the UE and does not take effect in this case.

Figure 6.3C.3.6-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.6-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The following applies for the uplink switching case specified in clause 6.1.6.3 of [10] when the configuration of the location of the switching period by [uplinkTxSwitchingPeriodLocation] is ignored by the UE:

-	if an uplink switching is triggered for an uplink transmission starting at T0 based on higher layer configuration(s) or DCI(s) received before T0 − Toffset as specified in [10] and the UE is not configured or scheduled with uplink transmissions for a duration of at least the uplink switching gap indicated by [uplinkTxSwitchingPeriod2T2T] on any of the carriers before T0, transient periods of 10 s are located at the end of the last symbol(s) configured or scheduled on the carriers before T0 and at the start of the first symbol(s) configured or scheduled at T0 on the switched-to carrier.

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].