3GPP TSG-RAN WG4 Meeting #115	R4-2507153

Malta, 19th – 23rd May 2025

Agenda item:	7.17.4

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for NR MMSE-IRC BS demodulation.

Document for:	Discussion

- Introduction

We show the simulation results based on WF in RAN4#114bis [1].

- Simulations

PUSCH requirements with 1 interferer cells, Mapping type A, CP-OFDM, MCS table 16

| Tx/Rx | SCS and CBW | Propagation conditions | MMSE-MRC | MMSE-IRC | Gain |
| --- | --- | --- | --- | --- | --- |
| 1T2R | 15 kHz, 5MHz | TDLA30-10 for HetNet | 19.755 | 11.861 | 7.894 |
|  |  | TDLC300-100 for HomNet | 15.964 | 12.716 | 3.248 |
|  | 30 kHz, 100MHz | TDLA30-10 for HetNet | 19.853 | 11.912 | 7.941 |
|  |  | TDLC300-100 for HomNet | 15.963 | 12.702 | 3.261 |

- Conclusion

This contribution shows the simulation results of NR MMSE-IRC PUSCH demodulation.

References

- R4-2504757, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#114bis