TSG-RAN Working Group 4 (Radio) meeting #115	R4-2507156

Malta, EU, 19th to 23rd May 2025

Agenda item:			10.1

Source:	Ericsson

Title:	Discussion on the implementation of shorter form for co-existence and co-location requirements across the specifications

Document for:	Approval

# Introduction

During RAN4 #115 a way forward document was approved in [2] and the proposal was to Maintain tables for co-existence and co-location in each specification but simplify them.

Companies were encouraged to present the possible implementations of these options during this meeting, identifying pros and cons of the above options.

The following aspects need be considered when drafting the CRs:

- All notes deleted from the legacy tables are properly reflected in new table format.

- For conformance testing specifications, special care needs to be taken to correctly reflect frequency-dependant MU/TT values.

- FFS: how to reassure that unnecessary co-location requirements are NOT unintentionally added with this new framework, e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc.

- FFS: how to reassure that unnecessary co-existence requirements are NOT unintentionally added with this new framework.

- FFS: how to consider potential BS-class specific co-existence limits – refer to the BS Evo objective (TFES request).

# Discussion

The target format of co-ex and co-location tables is described below, using TS 38.104 and BS Type 1-C for exemplification:

- Co-existence requirement

Measurement bandwidth for the requirements in table 6.6.5.2.3-1 is TBD.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| Frequency range for co-existence requirement | Operating band to co-exist with | Basic limits (dBm/MHz) | Notes |
| --- | --- | --- | --- |
| Frequency range of downlink operating band | GSM850, CDMA850 or GSM900 | -57 | Note 1 |
|  | DCS1800 or PCS1900 | -47 |  |
|  | Other | -52 |  |
| Frequency range of uplink operating band | GSM850, CDMA850, GSM900, DCS1800 or PCS1900 | -61 | Note 1, Note 2, Note 3, Note 4 |
|  | Other | -49 |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, n29 n67; the co-existence requirements in this table does not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for the excluded frequency range may be covered by local or regional requirements. NOTE 2: For Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies to BS operating in Band n28, starting 4 MHz above the Band n28 downlink operating band, and specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 68 UL operating band, where requirement between 698 MHz and 703 MHz NOTE 3: For a BS operating in Band n29, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band. NOTE 4: For BS operating in Band n67, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with Band 28/ n28 UL operating band or Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz. |  |  |  |

Observation 1: In table 6.6.5.2.3-1 there is a mistake with regards to the measurement bandwidth as 100 kHz was previously assumed for specific cases and we cannot use the same limit in 1 MHz. Changing the measurement bandwidth is outside the scope of the RAN Task. The correct form would be:

Observation 2: In some cases we need to delete 3 tables (e.g for WA, MR and LA)  while introducing a new one with all the information.

Observation 3: In some specification the Out-of-band blocking requirements for co-location need to be compressed as well in one table. This aspect was not mentioned in the WF

Proposal 1: Simplify the Blocking requirements for co-location following the template used in TS 38.104 clause 7.5.3 (Conducted) and 10.6.2 (OTA)

Proposal 2: The text in front of the new co-existence and co-location tables need to be modified to adapt for the changes.

E.g. from 36.104

# Summary

Observation 1: In the new proposed form of table 6.6.5.2.3-1 constained in the WF there is a mistake with regards to the measurement bandwidth as 100 kHz was previously assumed for specific cases and we cannot use the same limit in 1 MHz. Changing the measurement bandwidth is outside the scope of the RAN Task.

Observation 2: In some cases we need to delete 3 tables (e.g for WA, MR and LA)  while introducing a new one with all the information.

Observation 3: In some specification the Out-of-band blocking requirements for co-location need to be compressed as well in one table. This aspect was not mentioned in the WF

Proposal 1: Simplify the Blocking requirements for co-location following the template used in TS 38.104 clause 7.5.3 (Conducted) and 10.6.2 (OTA)

Proposal 2: The text in front of the new co-existence and co-location tables need to be modified to adapt for the changes.

# References

[1] 	RP-243288	Moderator's summary of discussion about R19-RAN4-BSRF, RAN#106

[2] R4-2504741	Way forward for [114bis][331] BSRF_Simplify_CoLo_Coex