3GPP TSG-RAN WG4 Meeting # 115						        R4-2507179

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.5

Source: 	OPPO

Title: 	On OTA aspect

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the OTA aspect is provided in this paper.

- Discussion

During the RAN4#114bis meeting, the agreed WF[1] has been captured as below:

Issue 4-5-3: Test procedure for Tx RF requirements

Agreement:

- Step 1: Conduct the calibration procedure to determine testing antenna gain, CW antenna gain, mismatch between testing/CW antenna to DUT, insertion loss and cable loss, etc.

- Step 2: Place DUT based on UE positioning guidelines.

- Step 3: Device is assumed charging during the measurement according to device declaration on the required energy conditions.

- Step 4: Set the target test frequency and transmit power for signal generator and CW signal. The received power at the antenna of DUT is Pin. Pin is set as the power level of minimum reference sensitivity of device equal to or larger than “sensitivity+[X]dB”

- Step 5: Measure and record the backscattering signal level from DUT and calculate the backscattering power level transmitting from DUT antenna, i.e., Pout, based on the calibration data from step 1

- Step 6: Move to next measurement point and repeat Step 3-5 until complete all the testing point in the measurement grid.

- Step 7: Calculate the ratio of CW power level to D2R signal power (i.e., before and after backscattering) according to the equation: backscattering loss (dB) = Pout (TRP) – Pin (TRP)

- Step 8: FFS on performance metric. FFS on how to specify the performance metric.

The equation to calculate the backscattering loss has considered the device antenna gain at each side. The test will have two middle parameter as the input CW TX power at device antenna and the backscattered power at device antenna.

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

For the input CW TX power at device antenna, from the agreed WF, it is step 4 as to set the target test frequency and transmit power for signal generator and CW signal so that the received power at the antenna of DUT is Pin. From further consideration, it seems the Pin might be better to be set to the calibration procedure. The Pin can be tested with an RX antenna at the device position. After that we replace the RX antenna with device and test the device output power. The two steps can be illustrated as below:

Figure 1 Step a to test the RX power of device antenna

Figure 2 Step b to test the backscattered TX power of device antenna

With that the backscattering loss can be calculated with RX power – TX power of device antenna.

From our understanding, the step a can be a calibration step before the test and hence we only consider the test procedure of the TX output power of the device. In such case, we don’t need to further consider the CW output power.

Observation 1: Two step method can be used to test the backscattering loss.

Observation 2: The RX power of CW can be a calibration step before the test for the device begin.

Proposal 1: To consider the calibration of CW RX power with the 20cm distance of CW and device.

Proposal 2: To use the calibration result of CW RX power minus the TX output power of device as the backscattering loss of device.

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Observation 1: Two step method can be used to test the backscattering loss.

Observation 2: The RX power of CW can be a calibration step before the test for the device begin.

Proposal 1: To consider the calibration of CW RX power with the 20cm distance of CW and device.

Proposal 2: To use the calibration result of CW RX power minus the TX output power of device as the backscattering loss of device.

4 References

[1] R4-2505230	WF on A-IOT device requirements, CMCC