3GPP TSG-RAN WG4 Meeting # 115						        	R4-2507181

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.26.3.2

Source: 	OPPO

Title: 	On RF requirements for device 1

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the system parameter is provided in this paper.

- Discussion

During the RAN4#114bis meeting, the agreed WF has been captured as below:

Issue 3-1-1: Transmit output power

Agreement:

- Performance metric of transmit output power:

  - Backscattering loss

- Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power

  - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss

- For the Input CW Tx power to define requirements:

  - FFS on the whether to use one value or a range of CW Tx power

  - FFS on the value(s) of CW Tx power

- Consider following aspects to determine backscattering loss to define requirements:

  - modulation loss

  - antenna efficiency

  - backscattering circuitry loss

  - impairment margin

  - minimum distance to the reader

  - differential radar cross section in two different modulation states

  - differential reflection coefficient in two different modulation states

  - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself

  - Other factors not excluded

Topic 3-2: Modulation quality

Issue 3-2-1: SFO requirement

Agreement:

- 10% SFO is assumed as the minimum performance for devices.

- FFS on whether to define SFO requirement and how to test the SFO requirement if defined.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Define modulation quality requirements for both BPSK and OOK.

- Consider to define same modulation quality metric for both BPSK and OOK if possible.

- Reference measurement process needs to be discussed

Topic 3-3: Emission requirements

Issue 3-3-1: Emission requirements

Agreement:

- Discuss whether to define EIPR or TRP as performance metric for emission requirements

- Spurious emission: reuse legacy NR UE spurious emission requirement and further discuss the OOB boundary.

- FFS on CW maximum input power level for testing

Topic 3-4: Reference sensitivity

Issue 3-4-1: Reference sensitivity

Agreement:

- Define minimum requirements of REFSENS based on link budget (target is [15m]) and measurement results reported by companies

- Consider to use inventory procedure for REFSENS testing, success rate ([90%]) or miss detection rate ([10%]) as the metric for inventory

- At least to define maximum gain direction sensitivity requirements

- FFS on how to different sensitivity requirements on different orientations

Topic 3-5: Others

Issue 3-5-1: Maximum input power

Agreement:

- Define maximum input level, consider following options:

- Option 1: using [1m] distance assumption from reader to device

- Option 2: using square law appliance range

- Option 3: using dynamic range of device receiver

Issue 3-5-2: Occupied bandwidth

Agreement:

Further discuss whether to define occupied bandwidth

From the above agreement, we need to define the backscattering loss considering also the CW input power. From the EU regulation EN 302208, the CW input power will be 33dBm/200kHz from the CW output antenna connector. Simply use the free space path loss equation to estimate as

PL (dB)=92.45+20log10​(dkm​)+20log10​(fGHz​)

For the distance, use the 20cm distance between CW and the tag as illustrated in regulation test setup. And for band n8 the UL frequency will be 880-915MHz and we use 900MHz to calculation. Hence the PL will be around 17.6dB. The 33dBm/200kHz is E.R.P hence the CW antenna gain is already considered. With that, the input power of the device is around 15.4dBm/200kHz. Also as compared to the RF ID regulation, the output power of the tag is limited as -10dBm E.R.P in 640kHz channel bandwidth which is corresponding to -16dBm/200kHz. With that the backscattering loss of an RF ID tag is around 30dB.

Observation 1: With the regulation, the RF ID tag has around 30dB backscattering loss.

Observation 2: The EU regulation test the maximum E.R.P direction.

Some measurement captured in the thesis [2] is shown as below. It can be seen the return loss of the device is around -30dB which is quite aligned with the above observation 1.

Observation 3: The return loss of a typical RF ID tag is around 30dB.

With the above discussion, it seems that for a dipole antenna, only test a maximum direction is not a good approach since it only consider the energy in one direction which has not reflected the efficiency of the device antenna. In such case we would like to propose to test the backscattering loss with TRP metrics.

Proposal 1: To test the backscattering loss with TRP metrics.

From TRP perspective, we need to consider the antenna efficiency which illustrates the useful energy reflected by the antenna. Usually for a 900MHz FR1 UE antenna, the antenna efficiency is about -9dB considering the TRP requirement for PC3 UE in low bands. For a device, the antenna is specific used for the certain frequency and hence a better antenna efficiency can be reached. Hence -6dB is a quite reasonable number. Furthermore, for the modulation loss, for OOK there will be -3dB modulation loss to be considered and hence there will be in total -9dB backscattering loss for OOK.

Proposal 2: To define backscattering loss as -9dB for OOK and -6dB for BPSK.

For the SFO requirement we have proposed in our companion paper for the system parameter discussion as for the D2R transmission bandwidth and the frequency shift should tolerate 10% frequency shift. In such case, we propose not to further introduce a new requirement for SFO.

Figure 1 illustration of the SFO to be considered in frequency shift and D2R transmission bandwidth

Proposal 3: Not to introduce a new requirement for SFO but together consider with the D2R frequency shift and transmission bandwidth.

For the emission requirement, as currently the backscattering loss is defined based on TRP, we propose to also consider the emission requirement with TRP metric. For the OOB boundary, 400kHz for low band is used considering 320kHz channel bandwidth of the RF tag. Also for NR UE, the OOB boundary has considered the channel bandwidth as the boundary. In such case, we propose also to reuse the D2R channel bandwidth as the OOB boundary.

Proposal 4: To use the D2R channel bandwidth as the OOB boundary.

From testing perspective, the tag has -10dBm E.R.P output power with the in-band as -25dBm E.R.P within the OOB range which result in 15dB suppression. This is the same test setup of radiated power with 20cm distance between CW signal and the device which is around 15.4dBm/200kHz which equals to 2.1dBm/15kHz input signal at the device antenna. For the simulation stage, we have assumed 25dBc for in-band emission consider the device with 15kHz transmission bandwidth within the whole 10MHz NR channel bandwidth. In such case with the -6dB backscattering loss and 25dBc in-band emission suppression, the emission will be around -29dBm/15kHz which equals -30.7dBm/10kHz. This can fulfil the -25dBm/10kHz emission mask requirement. In such case, we propose to reuse -25dBm/10kHz TRP requirement for the emission mask inside the OOB range.

Observation 4: The emission requirement can be fulfilled in the test setup as 20cm between CW and device.

Proposal 5: To use -25dBm/10kHz TRP for the inside OOB range requirement.

Figure 2 EU Regulation emission mask

3	Conclusions

In this paper, we give discussion on the device 1 RF requirement, the observations and proposals are as below:

Observation 1: With the regulation, the RF ID tag has around 30dB backscattering loss.

Observation 2: The EU regulation test the maximum E.R.P direction.

Proposal 1: To test the backscattering loss with TRP metrics.

Proposal 2: To define backscattering loss as -9dB for OOK and -6dB for BPSK.

Proposal 3: Not to introduce a new requirement for SFO but together consider with the D2R frequency shift and transmission bandwidth.

Proposal 4: To use the D2R channel bandwidth as the OOB boundary.

Observation 4: The emission requirement can be fulfilled in the test setup as 20cm between CW and device.

Proposal 5: To use -25dBm/10kHz TRP for the inside OOB range requirement.

4 References

[1] R4-2502858	WF on requirements for A-IoT device, CMCC

[2] Theory and Measurement of Backscattering from RFID Tags, Pavel V. Nikitin and K. V. S. Rao