3GPP TSG-RAN WG4 Meeting # 115						           R4-2507183

Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.2.2

Source: 	OPPO

Title: 	TP to TR 38.787 for intra-band non-contiguous CA

Document for:	Approval

- Introduction

During the last RAN4#114bis meeting, the A-MPR simulation has been discussed with companies provided simulation results. In this TP, we try to capture the simulation results..

- Text Proposals

<Start of change>

6.2.3	UE additional maximum output power reduction

For the A-MPR analysis to comply the US regulation, RAN4 agreed to study and specipy the related PC2 A-MPR requirements with NS_52 after final FCC regulation for the additional emission limits is released in US.

For the A-MPR analysis to comply the EU reguration, RAN4 need to study and specify the PC3/PC2 A-MPR requirements with NS_33 based on the merged the interested companies results.

The required EU regulations for SL CA UE with 10MHz CBW are listed as below

- Additional Spectrum mask: When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.2.3-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

- Additional emission requirements:

Table 6.2.3-2: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

- Additional requirements to protect CEN DSRC:

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

6.2.3.1	MPR for PSSCH/PSCCH transmission

6.2.3.1.2	Simulation result from OPPO

Based on the simulation assumption, below configuration has been used to develop the A-MPR requirement.

Table 6.2.3.1.2-1 A-MPR simulation configuration

| case | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | CBW1 | CBW2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 0 | 10 | 42 | 15 | 10 | 10 |
| 2 | 10 | 18 | 10 | 22 | 15 | 10 | 10 |
| 3 | 10 | 19 | 10 | 22 | 15 | 10 | 10 |
| 4 | 10 | 27 | 10 | 14 | 15 | 10 | 10 |
| 5 | 10 | 33 | 10 | 7 | 15 | 10 | 10 |
| 6 | 10 | 34 | 10 | 7 | 15 | 10 | 10 |
| 7 | 10 | 41 | 10 | 0 | 15 | 10 | 10 |
| 8 | 25 | 0 | 25 | 27 | 15 | 10 | 10 |
| 9 | 25 | 19 | 25 | 8 | 15 | 10 | 10 |
| 10 | 25 | 20 | 25 | 8 | 15 | 10 | 10 |
| 11 | 25 | 26 | 25 | 1 | 15 | 10 | 10 |
| 12 | 30 | 0 | 30 | 22 | 15 | 10 | 10 |
| 13 | 30 | 19 | 30 | 3 | 15 | 10 | 10 |
| 14 | 30 | 19 | 30 | 2 | 15 | 10 | 10 |
| 15 | 30 | 21 | 30 | 1 | 15 | 10 | 10 |
| 16 | 50 | 1 | 50 | 0 | 15 | 10 | 10 |

As the major requirement for the A-MPR is the ASE requirement which needs -30dBm/MHz at the band edge. This will be quite stringent to the UE. Hence, we have simulated the first CC at 5860 channel and 5870 channel to see the difference. For the gap, we have applied the 10MHz. For the upper edge of the band, it should be quite the same as the band lower edge and hence the simulation is omitted.

The results are shown as below:

Table 6.2.3.1.2-2 Simulation result for A-MPR

| case | A-MPR 5860MHz | A-MPR 5870MHz | B |
| --- | --- | --- | --- |
| 1 | 14.3 | 11.3 | 3 |
| 2 | 12.6 | 9.5 | 3 |
| 3 | 13.1 | 10.2 | 3 |
| 4 | 13.2 | 10.4 | 3 |
| 5 | 12.7 | 9.8 | 3 |
| 6 | 12.4 | 9.8 | 3 |
| 7 | 12.3 | 9.8 | 3 |
| 8 | 12.5 | 9.7 | 7.5 |
| 9 | 14.8 | 12.1 | 7.5 |
| 10 | 16.5 | 13.4 | 7.5 |
| 11 | 12.8 | 9.6 | 7.5 |
| 12 | 12.9 | 9.6 | 9 |
| 13 | 11.5 | 8.8 | 9 |
| 14 | 11.5 | 8.8 | 9 |
| 15 | 12.3 | 9.2 | 9 |
| 16 | 13.7 | 10.4 | 15 |

Above Table 6.2.3.1.2-2 is the simulation result for 10+10MHz as NC CA with the first CC at 5860MHz and 5870MHz respectively. Also, PC3 with 2PA 1LO architecture has been used.

<End of Change>