# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507189

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on IoT NTN TDD mode

Agenda item:	7.33.6

Document for:	Approval

Introduction

At RAN 105# meeting, the revised WID on IoT NTN TDD mode [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In this meeting, we will provide the initial analysis on the RAN4 impact on periodic pattern. The work item description can be seen as below:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

Discussion

- In this meeting, we will discuss the scenarios and express our views on potential impact to RAN4 requirements towards the above objective based on RAN1’s discussion.

  - 2.1 General aspects

- In previous meeting, we have reached the agreement on general aspects as below:

## Issue 0: General aspects Agreement (online): For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings: - The target SNR - Applicable coverage (whether the EC needs to be considered) - Applicable Rmax - Applicable DRX cycle - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

- In legacy scenario, we mainly consider enhanced coverage (EC), the repetition times will ensure the measurement accuracy under lower SNR conditions, that is, the serving coverage is enhanced. Using intra-frequency requirements as an example, Tdetect is 26s for normal coverage and 256s for enhanced coverage. If considering the TDD mode, the cell detection time for enhanced coverage will be further extended. From our perspective, enhanced coverage shall not be considered in TDD mode.

- Observation 1: If considering the TDD mode, the cell detection time for enhanced coverage will be further extended.

- Proposal 1: Enhanced coverage (EC) shall not be considered in TDD mode.

  - 2.2 Cell Re-selection

- In the last meeting, cell reselection requirements has not been discussed. The following is the moderator’s recommendation:

Moderator’s Recommendation: For the cell reselection for TDD NB-IoT in NTN, discuss the following details in the next meeting: - Tdetect, Tevaluation and Tmeasure

- For cell re-selection requirements, there are three parameters shall be considered if N=9, they are Tdetect, Tevaluation and Tmeasure. Furthermore, RAN1 has reached the agreement on fixed downlink subframes within D=8:

Agreement The downlink subframes within D=8 are fixed to: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

- Based on the above agreement, we have the figure as below:

Figure 1 D=8 with N=9

- As shown in figure 1, we conclude that the NPSS is available per every N frame, but the NSSS is available per 2N frame. Consequently, NPSS and NSSS have extended N/2N times respectively, UE may need more time to detect/measure/evalute at least one NPSS/NSSS.

- In the last meeting, there is still one remaining issue below:

- For earth moving cell: - Option 2-A: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length - Option 2-B: Re-evaluate the maximum latency in cell detection, measurement, and evaluation required for this scenario, as well as the minimum number of NPSS, NSSS, and NRS samples needed for cell detection and measurement at an SNR of [-1] dB, and enhance Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

- For cell re-selection requirement, it highly relies on the DRX cycle and one issue is raised. We know that the smallest DRX cycle is 320ms and N=9 (90ms), the common multiple of both is 1440ms, that is, DRX cycle aligns perfectly with the period of N=9 once every 1440ms. If UE wakes up every 320ms, there is no downlink signals would be detected during each wake-up time and the power consumption would be increased. Thus, the relaxed requirements shall be used for short DRX cycles.

- We deem that the measurement period shall be extended in order to guarantee the measurement accuracy.  However, if reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long, the initial measurement results may not be useful any more for cell re-selection and other impacts. To our understanding, the trade off between measurement accuracy and measurement delay shall be considered.

- Based on the above analysis, we think option 2-B is more reasonable since the side condition is more higher than legacy, the number of measurement samples shall be re-evaluated, that is, the number of measurement samples shall be reduced.

- Observation 2: The downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0].

- Observation 3: NPSS is available per every N frame, NSSS is available per 2N frames.

- Observation 4: DRX cycle aligns perfectly with the period of N=9 once every 1440ms. If UE wakes up every 320ms, there is no downlink signals would be detected during each wake-up time and the power consumption would be increased.

- Observation 5: If reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long, the initial measurement results may not be useful any more for cell re-selection.

- Proposal 2: For short DRX cycles, the relaxed requirements shall be considered.

- Proposal 3: RAN4 could reduce the measurement samples for cell re-selection to trade off the measurement accuracy and total measurement delay in normal coverage.

  - 2.3 RRC re-establishment

- In the last meeting, RAN4 confirms the general scope for RRC re-establishment RRM requirements, the details shall still be discussed in this meeting. The reached agreements could be seen as below:

Agreement (online): In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN, - No update to TUL_grant - For the following parameters in TUE_re-establish_delay_NB-IoT - No update: - TPRACH_NB-IoT - TSI_NB1-NC [and TSI_NB1-EC] - Consider update, and the details are FFS: - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell

- Since the enhanced coverage shall be considered, RAN4 shall only focus on Tsearch_NB1-NC,i for unknown cell. If target cell is unknown cell, Tsearch_NB1-NC,i = Ksatellite,i *1400 ms. In TDD mode, this parameter shall be extended. There are two components:Ksatellite,i and 1400ms (the maximum duration required for a UE to perform a complete cell search), the former parameter is not related to the TDD mode, only the latter one shall be modified by TDD mode. From our perspective, this parameter shall be scaled by N=9 not 2N, since Tsearch_NB1-NC,i mainly refers to the time of rough synchronization, we shall refer to the periodicity of NPSS not NSSS in TDD mode.

- Observation 6: Tsearch_NB1-NC,i mainly refers to the time of rough synchronization.

- Proposal 4: Tsearch_NB1-NC,i shall be scaled by N=9.

  - 2.4 RRC Connection redirection to Non-Anchor Carrier

- In the last meeting, RAN4 has discussed the RRC connection redirection and the agreements can be seen as below:

Agreement (online): For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN, - No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH - Update the following parameters - Tperiod_DL_bitmap: to be updated based on RAN1 design - TDL-UL switch: to be updated based on RAN1 design

- We find that only Tperiod_DL_bitmap and TDL-UL switch shall be updated based on RAN1 design, where

- Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It can be configured for 10ms or 40ms, which controls the downlink signal “on and off”. If we update this value by considering N=9, the values could be configured as 90ms in TDD mode rather than 10ms or 40ms. From our perspective, the compatibility issues need to be considered, that is, we shall consider whether legacy UEs support the new value or not.

- TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. The same understanding as above.

- Observation 7: Tperiod_DL_bitmap can be configured for 10ms or 40ms, which controls the downlink signal “on and off”. TDL-UL switch is 8 ms.

- Proposal 5: RAN4 could update the value of Tperiod_DL_bitmap and TDL-UL switch based on N=9 (90ms).

- Proposal 6: If new values of Tperiod_DL_bitmap and TDL-UL switch are introduced, the compatibility issues need to be considered.

Conclusion

In this paper we provided our views on RRM requirements on R19 IoT NTN TDD mode enhancement.

- Observation 1: If considering the TDD mode, the cell detection time for enhanced coverage will be further extended.

- Proposal 1: Enhanced coverage (EC) shall not be considered in TDD mode.

- Observation 2: The downlink subframes within D=8 are fixed to [3 4 5 6 7 8 9 0].

- Observation 3: NPSS is available per every N frame, NSSS is available per 2N frames.

- Observation 4: DRX cycle aligns perfectly with the period of N=9 once every 1440ms. If UE wakes up every 320ms, there is no downlink signals would be detected during each wake-up time and the power consumption would be increased.

- Observation 5: If reuse the same sample numbers in legacy NB-IoT, the whole measurement/evaluation period would be very long, the initial measurement results may not be useful any more for cell re-selection.

- Proposal 2: For short DRX cycles, the relaxed requirements shall be considered.

- Proposal 3: RAN4 could reduce the measurement samples for cell re-selection to trade off the measurement accuracy and total measurement delay in normal coverage.

- Observation 6: Tsearch_NB1-NC,i mainly refers to the time of rough synchronization.

- Proposal 4: Tsearch_NB1-NC,i shall be scaled by N=9.

- Observation 7: Tperiod_DL_bitmap can be configured for 10ms or 40ms, which controls the downlink signal “on and off”. TDL-UL switch is 8 ms.

- Proposal 5: RAN4 could update the value of Tperiod_DL_bitmap and TDL-UL switch based on N=9 (90ms).

- Proposal 6: If new values of Tperiod_DL_bitmap and TDL-UL switch are introduced, the compatibility issues need to be considered.

- 4 References

- RP-242415, New WID on introduction of IoT-NTN TDD mode, Iridium Satellite LLC.

- TS36.133

- R4-2502607, WF on RRM requirements for IoT _NTN_TDD, Qualcomm.