# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507190

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Ku bands for NR NTN

Agenda item:	7.11.7

Document for:	Approval

Introduction

At RAN #107 meeting the revised WI “Introduction of Ku bands for NR NTN” [1] was approved.  In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #114 meeting we will specify the scenarios and general view on the topics. In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

Discussion

In the last meeting, RAN4 only discussed the general scope for NR NTN on Ku bands and the uplink transmit timing requirements. In this paper, we will provide the other potential RRM requirements for NR NTN on Ku bands. Furthermore, considering the BDaT session also had the agreements on SCS for downlink SSB signals and uplink data transmission, we will supplement the concrete requirements like uplink timing requirements etc.

  - 2.1 RRM scope for NR NTN on Ku bands

In previous meetings, we specified the general scope for NR NTN on Ku bands including SCS in FR1 and FR2 NTN numerology. In this section, we would like to analyze other issues on general aspect.

Issue 1-2: Applicability update (section 3.6.12)

The way forward could be seen as below:

Option 1: The requirements for FR2-NTN/NTN band above 10GHz except for timing requirements in Clause 7 are applicable to Ku-band NTN with FR1-NTN and FR2-NTN numerologies.

The section 3.6.12 in TS 38.133 can be seen as below:

3.6.12 Applicability of requirements for Satellite Access The requirements for Satellite Access defined in clauses with suffix ‘C’ apply provided that UE indicates nonTerrestrialNetwork and is accessing a cell served by a Satellite Access Node (SAN). The requirements apply provided that serving and all neighbour satellites on the same layer are of same satellite type (LEO or GEO).

From our perspective, this update is not needed since this section is a kind of high level description without any details. For option 1, it can be specified when we defining RRM requirements in spec. Consequently, we observe that it is not necessary to do such update.

Observation 1: Section 3.6.12 is a kind of high level description without any details.

Proposal 1: It is not necessary to update the applicability in section 3.6.12.

  - 2.2 Uplink transmit timing requirement

In previous meetings , RAN4 firstly confirmed four cases from RRM requirements perspective. Based on four cases, we defined the related uplink timing requirements, and there still have other issues shall be confirmed in this meeting.

Issue 1-9: Timing accuracy requirements for case 4 (section 7.1C)

For case 4, we have already confirmed the concrete values on timing accuracy requirements. However, the side conditions have not been agreed, we will provide our analysis on this issue in this meeting. Firstly, we could observe the following options:

- Issue 1-3: UL transmit timing requirement - Agreement: - The same Te_NTN requirement applies to case 4. On top of the side conditions agreed for Rel-18 Ka band, RAN4 to add the following side condition to Te_NTN requirements for Case 4 - Option 1: Capture it as a NOTE on the test case in the core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec in the core corresponding test case. - Option 2: Capture it as a NOTE in core requirement: The interval between adjacent epoch times should not exceed [2.56 or 5.12] sec.

For the uplink timing requirements for case 4, the majority companies agree with the requirements for mobile VSAT served by GSO could be reused. The main difference is that the serving satellite is NGSO for mobile VSAT UE not GSO, thus, we believe that the side condition is needed. Now the main point of contention currently lies in whether the side condition shall be captured in core requirements. To our understanding, we prefer to capture the side condition in the test case. In legacy NTN discussion, we also defined the interval between adjacent epoch time as LEO 2.56s and GEO 10.24s as below:

Table A.3.36.2-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5 GEO: 900 |
| cellSpecificKoffset | slot | LEO: 14 GEO: 258 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | Detailed ephemeris information is provided in TS 38.508-1 [38]. |
| ta-Report |  | Not configured |

The same logic shall be followed as legacy, the side condition shall be in the test case.

Observation 2: In legacy NTN discussion, RAN4 defined the interval between adjacent epoch time as LEO 2.56s and GEO 10.24s, the interval between adjacent epoch time is defined in test case.

For another point, the value can also be reused as legacy which is 2.56s.

Observation 3: The interval between adjacent epoch times of LEO is 2.56s.

Proposal 2: The side condition shall be added in test cases and the value for LEO can follow the legacy value, which is 2.56s .

  - 2.3 Potential RRM requirements

In last meeting, RAN4 has studied several potential RRM requirements and reached the agreement. However, there are still some unsolved requirements on the table. We would like to provide our vies in this section.

Issue 1-4: Inter-satellite cell reselection requirements (section 4.2C)

For inter-satellite cell reselection requirements, only one company raised this issue as following:

Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs in Rel-19.

For this issue, we prefer to not define inter-satellite cell reselection for VSAT UEs in R19 since RAN4 does not define this requirements in Ka band. The company also said that during R18 WI, when the VSAT requirements were first created for spec, there was limited time for discussing this potential scenario, and RAN plenary took the decision to limit the scope of NTN applicability. Thus, we would not like to define such complex scenario as legacy Ka band unless RAN plenary agree to discuss this.

Proposal 3: RAN4 shall not create inter-satellite cell reselection requirements for VSAT UEs on Ku bands in R19.

Issue 1-6: NR SAN Handover (section 6.1C.1.3)

The controversial issue is whether the satellite switching with re-sync shall be considered. The following analysis shall be referred.

- Agreement: - Reuse the same value of T_sat_beam as in Ka band for Ku band - For VSAT UEs operating in FR1-NTN numerology, not specify the RRM requirements for TN-NTN mobility, inter-RAT mobility [or satellite switching with re-sync].

For VSAT UEs operating in FR1- NTN numerology, we have already agreed that TN-NTN mobility and inter-RAT mobility shall not be considered, since VSAT UE is not mainly worked in TN network whatever NR TN or LTE TN. For satellite switching with re-sync, we deem that it shall be considered since there is the requirements for satellite switching with re-sync from NR SAN FR1 cell to NR SAN FR1 cell in legacy.

Proposal 4: RAN4 shall consider satellite switching with re-sync for VSAT UE operating in FR1 NTN numerology.

Issue 1-12: RLM (section 8.1C)

The potential way forward as following:

Option 1: Confirm the assumption that LMax is equal to 8 for FR1-NTN

We are fine to revisit LMax from 4 to 8. In the current spec, we observed the following requirements:

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell | Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2-NTNNote 2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3 GHz is replaced by 1.88 GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |  |

The number of LMax depends on carrier frequency range of PCell, we have already agreed mobile VSAT UEs are served by GSO or NGSO in FR1- NTN numerology, that is, the frequency range of Ku band is not changed and larger than 3GHz. Consequently, the number of LMax shall be only 8 in FR1 for Ku band.

To our understanding, we shall add one note in core requirements for VSAT UE on Ku band: For Ku band NTN, Lmax shall be defined as 8 not 4.

For minimum requirements, to our understanding, the legacy values could be reused. However, we shall distinguish the Ku band NTN supporting FR1 and FR2 numerology.

Observation 4: The number of LMax depends on carrier frequency range of PCell.

Observation 5: The frequency range of Ku band is not changed and larger than 3GHz.

Proposal 5: LMax shall be equal to 8 not 4 for FR1-NTN.

Proposal 6: For minimum requirements, the legacy values could be reused. Furthermore, RAN4 shall distinguish the Ku band NTN supporting FR1 and FR2 numerology.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Observation 1: Section 3.6.12 is a kind of high level description without any details.

Proposal 1: It is not necessary to update the applicability in section 3.6.12.

Observation 2: In legacy NTN discussion, RAN4 defined the interval between adjacent epoch time as LEO 2.56s and GEO 10.24s, the interval between adjacent epoch time is defined in test case.

Observation 3: The interval between adjacent epoch times of LEO is 2.56s.

Proposal 2: The side condition shall be added in test cases and the value for LEO can follow the legacy value, which is 2.56s .

Proposal 3: RAN4 shall not create inter-satellite cell reselection requirements for VSAT UEs on Ku bands in R19.

Proposal 4: RAN4 shall consider satellite switching with re-sync for VSAT UE operating in FR1 NTN numerology.

Observation 4: The number of LMax depends on carrier frequency range of PCell.

Observation 5: The frequency range of Ku band is not changed and larger than 3GHz.

Proposal 5: LMax shall be equal to 8 for FR1-NTN.

Proposal 6: For minimum requirements, the legacy values could be reused. Furthermore, RAN4 shall distinguish the Ku band NTN supporting FR1 and FR2 numerology.

- References

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN.

- R4-2504897, WF on RRM requirements for NR_NTN_Ku_bands, Apple.