# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507192

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LP-WUS

Agenda item:	7.28.5.2

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

General aspects

In previous meetings, the basic scenarios were discussed with a few agreements. In this section, we mainly discuss the LP-WUS monitoring and the MR/LR measurements based on RAN1 and RAN2’s agreements.

Whether to keep Case#2

In previous meetings, RAN4 has already agreed the case#1 (MR Fully offload to LR, only LR) without consideration of configured high priority frequency, that is, only LR performs the serving cell measurement and MR is OFF. In previous meeting, we also agreed that MR and LR are in ON state (MR partially offload to LR, MR+LR).

For case#1, the measurement only performed by LR, that is, the LR measurement result will decide the MR is ON or OFF.

For case#3, MR and LR both perform the serving cell measurement, only MR performs neighbour cell measurement.

There are still case#2 and case#4 (measurements with high priority frequency) and we will firstly analyze the remaining scenarios and then specify the RRM requirements for all cases.

Firstly, we shall focus on the feasibility of Case#2 and the following figure can be seen:

Figure 1 Different cases

Based on the above figure, we find that the MR and LR are ON and both of them perform the serving cell measurement for Case#2. However, the main difference between Case#2 and Case#3 is that there is no neighbour cell measurement for Case#2 but for Case#3. In general, Case#2 and Case#3 depend on the serving cell measurement results. If the service quality of the current cell is good enough, the UE will not do the neighbour cell measurement. However, if measurement result is below the threshold which can enter into Case#1 in figure 1, UE is still in MR+LR state (Case#2). If measurement result is below the legacy neighbour cell measurement triggering threshold, the UE shall perform the neighbour cell measurement (Case#3). We deem that that is not a corner case and Case#2 may be needed. However, we shall have a comprehensive understanding if we decide to study Case#2. To my understanding, whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

We know that if UE stays in MR+LR state, the scaling factor for MR measurement time period is needed. The MR measurement is more infrequent than LR measurement and MR measurement will cost more power. Furthermore, RAN2 has an agreement below:

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

RAN2 has agreed that the entry/exit condition for serving cell measurement relaxation and neighbouring cell measurement are merged, that is, case 2 actually is case 3. Consequently, the Case#2 is not necessary to consider.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Threshold for switch between Case#1,Case#3 and Case#5

For entry and exit conditions based on LR and MR measurement, RAN2 has already reached the following conclusion in previous meetings:

127 meeting: For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. The exit condition is based on the LR measurement results. 127bis meeting: If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

For scenario 1, there are two cases which from Case#5 to Case#3 or Case#1 (5-->3 or 5-->1) and from Case#1 to Case#3 or Case#5 (1-->3 or 1-->5). For the first case, only MR measurement in Case#5 and if MR measurements result can be satisfied, UE can enter into Case#3. For the second case, only LR performs serving cell measurement, and the LR based measurement can be the only criteria to enter into Case#3 or Case#5 which is the exit condition agreed in RAN2, and the threshold can be the legacy neighbour cell triggering since the neighbour cell measurement would be triggered.

For scenario 2, Case#3 shall enter into Case#5 or Case#1 (3-->5 and 3-->1). MR and LR perform the serving cell measurement together in Case#3. If network configures two different thresholds which are for MR based measurement and LR based measurement, the two measurements can compare to the different thresholds. However, RAN2 agreed that at least MR measurement shall be considered, and LR could also be considered. To my understanding, LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature. If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not. This issue shall be clarified by RAN2 and it is RAN2’s decision.

- We deem that the threshold between Case#3 and Case#1 is the whether neighbour cell measurement is triggered. Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement. RAN1 has agreed introduced the LP-RSRP and LP-RSRQ in previous meetings, thus, the new threshold for LP-RSRP and LP-RSRQ could be considered. Also, some companies proposed an offset for LR measurement threshold, that is, we could continue to use the existing threshold with an offset for LP-RSRP and LP-RSRQ. For other switching cases, RAN4 shall wait for RAN2’s agreements and follow RAN2’s conclusion.

Observation 2:

- For scenario 1, there are two cases which from Case#5 to Case#3 and from Case#1 to Case#3.

- For scenario 2, Case#3 shall enter into Case#5 or Case#1.

Observation 3: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Observation 4: LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature.

Proposal 2: Use existing thresholds for SS-RSRP and SS-RSRQ from Case#3 to Case#1, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Proposal 4: If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not is based on RAN2’s decision.

LP-WUR status at legacy case

In previous meetings, we do not discuss the UE behaviour at legacy case (Case#5), the following way forward can be seen as below:

- Proposals - P1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. - P1-1: No LP-WUR measurement and evaluation requirement is applicable when LR is “on” at the legacy state; - Note: in detail, no requirement is applicable on LR for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). - P2: After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined. - P3: LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures

There is a common understanding that the partially offloading and fully offloading measurement will be performed when LR is ON. LR only performs the serving cell measurement. Regarding this issue, we would like to split two parts:

- Whether the LR is ON in legacy case.

- If LR is ON, whether the RRM requirements will be defined.

- For the first, we have reached the agreement as below:

Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- For the second point, RAN2 has the agreement as below:

- NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

- From our perspective, LR only performs serving cell measurement in Case#3 and Case#1. If LR is ON in legacy case, there is no measurement behaviour, only MR performs L3 measurement. Thus, no requirement is applicable when LR is at legacy case. However, RAN2’s agreement shows that NW configures thresholds for both MR and LR measurement, then the LP-WUS monitoring entry condition is met when all the measured results are above the configured thresholds. It means if LR is ON, then the LR would perform serving cell measurement. We know that the channel condition shall be guaranteed if LR is ON and performs serving cell measurement. However, if LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern, that is, we should not consider this case and do not define any RRM requirements.

- Observation 5: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 6: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 5: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

Higher priority frequency layer

When UE stays in center area of the current cell and the service of the cell is good enough, the UE will not perform neighbour cell measurement without configured high priority frequency. However, the UE shall search for inter-frequency and inter-RAT layers of high priority if high priority frequency information is provided by the serving cell based on TS 38.133:

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

- Observation 5: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 (to be confirmed) | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- For Case#4:

UE is in fully LR state and the MR shall be OFF, that is , UE is in the center area of the current cell and the service is good enough. In this case, serving cell has configured the high priority frequency information and UE shall search for high priority frequency layers and perform the neighbour cell measurement. Thus, MR shall be ON from OFF since UE search for the high priority frequency layer at Thigher_priority_search. But only LR performs serving cell measurement. As mentioned before, RAN2 is discussing the neighbour cell measurement relaxation condition and if the condition is the same as legacy, we deem that the legacy relaxed requirement can be as baseline and RAN4 shall only discuss the relaxed scaling factor.

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 6: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Detailed LP-WUR requirements at RRC IDLE/INACTIVE state

Periodicity for LP-WUR RRM requirements

In previous meetings, RAN4 has already discussed the periodicity for LP-WUR RRM requirements. There are still some detailed issues shall be discussed and the previous agreement as follows:

RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. RAN4 114 way forward: - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity. RAN4 114bis meeting: Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

Upper bound

We think the upper bound is need on SSB-based LP-WUR measurement periodicity. Actually, defining the upper bound is beneficial to the service quality for LR measurement. It is a trade-off between power saving and how fast UE can exit LR when the channel condition is worse. To our understanding, in cases where channel condition drastically deteriorates, if there is a rapid exit from the current LP-WUR mode coupled with corresponding adjustments, UEs will be able to respond more effectively to unexpected situations. For instance, during mobility, when a UE transitions from a region with high-quality links to an area with weaker links, it needs to promptly detect this change and implement appropriate measures to maintain communication quality. If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Observation 8: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 7: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

Lower bound

RAN1 has defined the periodicity of LP-SS as below:

- Agreement - Regarding whether to support additional sync signal to LP-SS, support the following for LP-WUS with M=1, M=2: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support LP-SS periodicity of 160ms

We observe that LP-SS periodicity is larger than or equal to 160ms. To our understanding, we prefer not to define the lower bound for LP-SS periodicity in measurement delay requirements. We know that the motivation of defining lower bound is to obtain power saving gain, as the measurement is more frequently if LP-SS periodicity is too small compared to SSB periodicity. However, we do not think that if LP-SS periodicity is at least supported by 160ms may not have much power saving contribution. From network perspective, the lower bound is not necessary.

Proposal 8: Lower bound is not necessary to define.

On LR measurement filtering requirement

In the last meeting, we have agreed as below:

RAN4 114bis meeting: Agreement: The UE shall filter the [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by [LO periodicity /2] for SSB based LR. RAN4 114bis meeting draft summary: P1-1: The UE shall filter the LP-RSRP and LP-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by LP-SS periodicity/2 for LP-SS based LR. (vivo CT Ericsson) P1-2: For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering requirement (CMCC) P3: UE shall filter the LR measurements using at least 2 measurements with enough separation. (Huawei) RAN4 112bis meeting: Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering Agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

There is remaining one issue for LP-RSRP and LP-RSRQ filtering measurements. To our understanding, P1-2 is more suitable. In RAN4 112bis meeting, we have reached one agreement above that measurement delay requirements for LP-WUR based on LP-SS in Idle/Inactive mode shall be defined based on LP-SS periodicity. Since at least two SSB exists in one DRX cycle in legacy, we shall consider the filtering measurement to guarantee the whole measurement accuracy. However, we deem that only one LP-SS is contained in LP-SS periodicity unlike SSBs in one DRX cycle. Consequently, for LP-SSS-RSRP and LP-SSS-RSRQ measurement, no need to define filtering requirements.

Proposal 9: For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering requirement.

On how to define LR evaluation requirements

In previous meetings, we have already defined the structure for LR evaluation requirements as below:

Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

In this meeting, we shall confirm the detailed values on x,y and x1,y1. Actually, we think that x,y shall be specified by simulation results which are the lower bound for meeting accuracy requirements. In order to prevent the ping-pong effect, we confirmed x1>x and y1>y, that is, x1,y1=N*x, M*y (N>1,M>1). To our understanding, when considering the values of N or M, we shall trade-off the measurement accuracy to prevent ping-pong effect and the whole measurement delay. Thus, we propose N=2 and M=2.

Proposal 10: For the values of x,y, they shall be specified by simulation results.

Proposal 11: For x1 and y1, x1,y1=N*x, M*y (N>1,M>1). When defining N and M, we shall trade-off the measurement accuracy to prevent ping-pong effect and the whole measurement delay, where N=2 and M=2.

LP-SSS-RSRP/RSSI Measurement

In the lat meeting, RAN1 has sent the LS to RAN4 and the details could be seen as below:

In RAN1, the following agreements were made on the RRM measurement metrics based on SSS for OFDM-based LP-WUR: Agreement (RAN1#120) Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement (RAN1#120bis) For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. From RAN1 perspective, it was agreed that SMTC window is not applicable for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. RAN1 respectfully asks RAN4 to confirm it.

From our perspective, we deem that the SMTC could not be configured for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell. There are two points of reasons:

- SMTC is configured for neighbouring cell measurement for MR not for LR. We are discussing the OFDM-based LR, thus, SMTC is not proper for LR measurement.

- SMTC is configured for neighbouring cell measurement for MR not for LR. To reduce the complexity of UE measurements, particularly during neighbouring cell SSB measurements, the UE needs to achieve synchronization. After introducing SMTC, the UE only needs to perform blind detection of SSBs within the SMTC period. This eliminates the need for blind SSB detection over extended time intervals. We are discussing the LR serving cell measurement since LR could only perform serving cell measurement. For neighbouring cell measurement, RAN2 agreed that only MR could perform.

- Observation 9: SMTC is configured for neighbouring cell measurement for MR not for LR.

- Observation 10: RAN2 agreed that only MR performs neighbouring cell measurement and LR only performs serving cell measurement.

- Proposal 12: For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

MR wake-up delay

In legacy, when DRX is configured and there is no traffic transmission, the MR (main receiver) enters a sleep state, only waking up at corresponding moments (DRX ON) to perform paging monitoring. If the specific conditions for entering LR coverage are satisfied, LR allows the UE MR to enter a deep sleep state prior to monitoring the LP-WUS. When the UE receives SIB message, the UE initiates monitoring of LP-WUS, subsequently triggering the awakening of the UE MR to monitor PDCCH, this process aims to further mitigate power consumption:

Figure 3 UE behavior upon detecting LP-WUS with paging indication in IDLE/INACTIVE state

Based on the above procedure, it is crucial that UE shall not miss paging when entry/exit conditions for LP-WUS monitoring are met. RAN4 shall study and define the related limitation on timing from UE receiving SIB message to waking up MR to monitor legacy PO when the entry condition is met. It also can be seen that the time window LP-WUS occasions (LOs) are defined for LP-WUS monitoring, each LO has one or more LP-WUS occasions.

From my perspective, the following shall be considered so that the UE will not miss the paging:

Figure 4 Time delay for paging monitoring

MR wake-up time: UE detects the LP-WUS and decodes it, then LP-WUS wakes up MR to monitor the legacy PO.

When the entry condition for LP-WUS monitoring is met, the time delay shall contain MR wake-up time TMR_wake-up . This time will decide whether LP-WUS can timely wake up MR and ensure that it does not miss the paging monitoring.

There are two scenarios shall be considered for defining the related wake-up delay:

- Scenario 1: Deep sleep.

- Scenario 2: Ultra deep sleep.

For Scenario 1, to our understanding, ramp up time, AGC, time sync shall be contained in wake up delay during this procedure. There are two kinds of LR shall be also considered when defining the wake up delay as follows:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- The measurement is performed by LR before MR awake and LR has the rough timing sync, which can inform MR when LR triggers MR awake. Thus, only 1 SSB could be used to adjust timing and frequency offset for OFDM based receiver, 1~2 SSB could be used to adjust timing and frequency offset for OOK based receiver, since OFDM based receiver has more accurate timing sync performance than OOK based LR. Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, so we think maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- For Scenario 2, the wake up delay can be seen as below:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Based on the above analysis, we deem that 3SSBs are enough for defining wake up delay, the main difference in Scenario 1 and Scenario 2 is only the ramp up time. Consequently, we think only 3SSBs could be used for AGC, timing sync and others.

- Observation 11:

- For deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- For ultra deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Observation 12: Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- Proposal 13: 3SSBs could be used for AGC, timing sync and others, and 5SSBs are not needed.

LP-WUR at connected mode

We mainly focus on the measurement requirements in the above. For the connected mode, in the latest WID, we can see that no RAN4’s involvement is required:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

It is obviously the CONNECTED state is not in the scope of the WID. RAN4 shall not define the measurement requirements at connected mode until RAN plenary add this scenario in revised WID.

Proposal 14: RAN4 shall not define the measurement requirements at CONNECTED state.

Conclusion

In this paper we provided our views on LPWUS.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Observation 2:

- For scenario 1, there are two cases which from Case#5 to Case#3 and from Case#1 to Case#3.

- For scenario 2, Case#3 shall enter into Case#5 or Case#1.

Observation 3: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Observation 4: LR measurement based switching is an optional feature and MR measurement based switching is a mandatory feature.

Proposal 2: Use existing thresholds for SS-RSRP and SS-RSRQ from Case#3 to Case#1, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 3: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Proposal 4: If network configures the threshold for LR based measurement, one issue will be raised. Whether UE shall satisfy the both thresholds like LP-WUS monitoring or not is based on RAN2’s decision.

- Observation 5: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 6: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 5: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 6: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Observation 8: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 7: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

Proposal 8: Lower bound is not necessary to define.

Proposal 9: For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering requirement.

Proposal 10: For the values of x,y, they shall be specified by simulation results.

Proposal 11: For x1 and y1, x1,y1=N*x, M*y (N>1,M>1). When defining N and M, we shall trade-off the measurement accuracy to prevent ping-pong effect and the whole measurement delay, where N=2 and M=2.

- Observation 9: SMTC is configured for neighbouring cell measurement for MR not for LR.

- Observation 10: RAN2 agreed that only MR performs neighbouring cell measurement and LR only performs serving cell measurement.

- Proposal 12: For LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable.

- Observation 11:

- For deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (around 20ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (20ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- For ultra deep sleep state:

- OFDM based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1 SSB) timing sync

- OOK based LR wakes up MR to perform PDCCH monitoring:

- Ramp-up time (400ms, 800ms) + (1~2 SSB) AGC + (1~2 SSB) timing sync

- Observation 12: Compared to ultra deep sleep, very coarse synchronization information are involved in deep sleep state, where “very coarse” synchronization information refers to a clock possibly running at low power, maybe 1~2 SSB for timing sync for OOK based LR in deep sleep state.

- Proposal 13: 3SSBs could be used for AGC, timing sync and others, and 5SSBs are not needed.

Proposal 14: RAN4 shall not define the measurement requirements at CONNECTED state.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869

[3] R4-2503116