# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507193

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for LP-WUS

Agenda item:	7.28.6

Document for:	Approval

Introduction

RRM core requirements for Rel-19 LP-WUS are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on LP-WUS.

Discussion

- For LP-WUS, according to previous agreements discussed in core part, some new test cases will be added, which will also be defined in clause A.6.

- Proposal 1: For LP-WUS, some new test cases will be added, which will also be defined in clause A.6.

- In core part discussion, RAN4 has already studied some core requirements in Idle/Inactive mode including MR measurement requirements, LR measurement requirements and switching among different states.

- For MR measurements requirements, we deem that the legacy test cases could be as baseline since the measurement behaviour has no big difference between legacy and measurement in LP-WUS feature.

- For LR measurement requirements, different cases shall be identified. In case1, MR enter into ultra deep sleep state and only LR performs serving cell measurements, only considering high priority frequency the cell re-selection will be occurred. However, if high priority frequency is considered, the test case for high priority frequency cell re-selection shall be considered. In case 3, MR performs relaxed serving cell measurement and neighbouring cell measurement, LR performs serving cell measurements. For this case, the test cases for cell re-selection shall be considered. The specific number of test cases could be discussed in this meeting.

- Proposal 2: In case 1, if high priority frequency layer is configured, the test cases shall be considered; otherwise, no test cases will be confirmed.

- Proposal 3: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting.

- Proposal 4: The legacy test cases for MR measurement shall be as baseline.

Conclusion

In this paper we provided our views on RRM requirements on R19 LP-WUS:

- Proposal 1: For LP-WUS, some new test cases will be added, which will also be defined in clause A.6.

- Proposal 2: In case 1, if high priority frequency layer is configured, the test cases shall be considered; otherwise, no test cases will be confirmed.

- Proposal 3: In case 3, the test cases for cell reselection shall be considered and the specific number of test cases could be discussed in this meeting.

- Proposal 4: The legacy test cases for MR measurement shall be as baseline.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133