# 3GPP TSG-RAN WG4 Meeting #115                                  R4-2507196

# St.Julian’s, Malta, 19th – 23th May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements MUSIM

Agenda item:	5.25.3

Document for:	Approval

Introduction

In the last meeting, RAN2 sent LS to RAN4 and the details could be seen as below:

On the collision between the MUSIM Gap and Measurement Gap (for the UE supporting musim-GapPriorityPreference capability) in R18, RAN2 identified that the network can configure the MUSIM gap without priority and the measurement Gap with priority together and vice versa. In addition, it is also supported from RAN2 specification that the network can configure the MUSIM gap without priority and the measurement Gap without priority together. RAN2 respectfully ask RAN4 on whether UE requirement is needed for the above collision cases.

In this meeting, we would like to provide our views on this issue.

Discussion

Some companies just said that MUSIM gaps shall always have priority levels since the following agreement:

Agreements: Network A assigns priority levels to all configured periodic MUSIM gaps even if UE does not indicate preferred priority for one or some periodic MUSIM gaps

We have two different understanding towards the above agreement:

- NW A will always configure the priority level to all configured periodic MUSIM gaps.

- NW A determines whether to configure the MUSIM gap priority or not, but if configured, NW A shall configure the priority for all of MUSIM gaps.

- For measurement gap, it contains two kinds of gap which are Type-1 measurement gap and Typ-2 measurement gap. For Typ-1 measurement gap, it doesn’t have any priority level. For Type-2 measurement gap, it could be understood as concurrent gap which has the priority gap. Among these, one special case is if single gap is configured via Type-2 signaling, this single gap has no priority level.

- The above understanding is the basic scenario from RAN4 perspective, now the following cases shall be clarified to RAN2:

- Table 1 MUSIM colliding with Type-1 measurement gap and Type-2 measurement gap

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Type-2 measurement gap priority |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Yes |
| - 2 | - Yes | - / | - No, special case |
| - 3 | - No | - / | - Yes |
| - 4 | - No | - No | - No, special case |

Observation 1:

- NW A will always configure the priority level to all configured periodic MUSIM gaps.

- NW A determines whether to configure the MUSIM gap priority or not, but if configured, NW A shall configure the priority for all of MUSIM gaps.

- Observation 2: For measurement gap, it contains two kinds of gap which are Type-1 measurement gap and Typ-2 measurement gap. For Typ-1 measurement gap, it doesn’t have any priority level. For Type-2 measurement gap, it could be understood as concurrent gap which has the priority gap. Among these, one special case is if single gap is configured via Type-2 signaling, this single gap has no priority level.

- Proposal 1: The following cases shall be considered:

- Table 1 MUSIM colliding with Type-1 measurement gap and Type-2 measurement gap

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Type-2 measurement gap priority |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Yes |
| - 2 | - Yes | - / | - No, special case |
| - 3 | - No | - / | - Yes |
| - 4 | - No | - No | - No, special case |

- MUSIM gap collide with Type-1 measurement gap

- For case 1, we have already reached an agreement as below:

Agreement: - P2: Collision is handled based on the MGRP of the collided gaps - P2-1: In a collision, the gap occasion with longer MGRP will be kept when any measurement gaps in the collision gaps is not assigned a priority; and the gap occasion with shorter MGRP will be dropped. - P2-2: No requirements apply if any of the two gaps in a collision have the same MGRP.

- For this scenario, type-1 has no assigned priority and NW A doesn’t configure the priority level to all configured MUSIM gaps. Consequently, when MUSIM gaps colliding with Type-1 measurement gaps without assigned priority level, legacy method which is based on MGRP could be used.

- For case 4, the main difference between case 1 and case 4 for Typ-1 measurement gap is that NW A doesn’t configure the priority level in case 4. However, the same collision processing for case 4 shall still use the above method, which is based on MGRP.

- Table 2 The collision processing for MUSIM gaps colliding with Type-1 measurement gaps

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Based on MGRP |
| - 2 | - Yes | - / | - / |
| - 3 | - No | - / | - / |
| - 4 | - No | - No | - Based on MGRP |

- Proposal 2: When MUSIM gaps colliding with Type-1 measurement gaps, the following collision processing shall be considered:

- Table 2 The collision processing for MUSIM gaps colliding with Type-1 measurement gaps

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Based on MGRP |
| - 2 | - Yes | - / | - / |
| - 3 | - No | - / | - / |
| - 4 | - No | - No | - Based on MGRP |

- MUSIM gap collide with Type-2 measurement gap

- For case 1 (colliding with type-2 measurement gap), the following agreement has been reached:

Agreement: - Collision between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. Agreement: - Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. “Keep solution” is used for the remaining non-dropped MUSIM gaps.

- We observed that when MUSIM gaps colliding with type-2 measurement gap, the priority based collision processing is used. To our understanding, the same method could be used for case 3.

- For case 2 and case 4 (colliding with type-2 measurement gap), the special case for type-2 measurement gap shall be considered. We have explained that the type-2 measurement gap will not be assigned priority level as type-1 measurement gap if only single gap is configured via type-2 signaling. Consequently, the collision processing shall be same as type-1 measurement gap, which is based on MGRP.

- Table 3 The collision processing for MUSIM gaps colliding with Type-2 measurement gaps

| - Case | - MUSIM gap priority | - Type-2 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - Yes | - Based on priority |
| - 2 | - Yes | - No, special case | - Based on MGRP |
| - 3 | - No | - Yes | - Based on priority |
| - 4 | - No | - No, special case | - Based on MGRP |

- Proposal 3: When MUSIM gaps colliding with Type-2 measurement gaps, the following collision processing shall be considered:

- Table 3 The collision processing for MUSIM gaps colliding with Type-2 measurement gaps

| - Case | - MUSIM gap priority | - Type-2 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - Yes | - Based on priority |
| - 2 | - Yes | - No, special case | - Based on MGRP |
| - 3 | - No | - Yes | - Based on priority |
| - 4 | - No | - No, special case | - Based on MGRP |

Conclusion

In this paper we provided our views on RRM requirements on R18 Multi-SIM.

Observation 1:

- NW A will always configure the priority level to all configured periodic MUSIM gaps.

- NW A determines whether to configure the MUSIM gap priority or not, but if configured, NW A shall configure the priority for all of MUSIM gaps.

- Observation 2: For measurement gap, it contains two kinds of gap which are Type-1 measurement gap and Typ-2 measurement gap. For Typ-1 measurement gap, it doesn’t have any priority level. For Type-2 measurement gap, it could be understood as concurrent gap which has the priority gap. Among these, one special case is if single gap is configured via Type-2 signaling, this single gap has no priority level.

- Proposal 1: The following cases shall be considered:

- Table 1 MUSIM colliding with Type-1 measurement gap and Type-2 measurement gap

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Type-2 measurement gap priority |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Yes |
| - 2 | - Yes | - / | - No, special case |
| - 3 | - No | - / | - Yes |
| - 4 | - No | - No | - No, special case |

- Proposal 2: When MUSIM gaps colliding with Type-1 measurement gaps, the following collision processing shall be considered:

- Table 2 The collision processing for MUSIM gaps colliding with Type-1 measurement gaps

| - Case | - MUSIM gap priority | - Typ-1 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - No | - Based on MGRP |
| - 2 | - Yes | - / | - / |
| - 3 | - No | - / | - / |
| - 4 | - No | - No | - Based on MGRP |

- Proposal 3: When MUSIM gaps colliding with Type-2 measurement gaps, the following collision processing shall be considered:

- Table 3 The collision processing for MUSIM gaps colliding with Type-2 measurement gaps

| - Case | - MUSIM gap priority | - Type-2 measurement gap priority | - Collision processing |
| --- | --- | --- | --- |
| - 1 | - Yes | - Yes | - Based on priority |
| - 2 | - Yes | - No, special case | - Based on MGRP |
| - 3 | - No | - Yes | - Based on priority |
| - 4 | - No | - No, special case | - Based on MGRP |

- 4References

[1] R2-2503163, LS on Collision between MUSIM Gap and Measurement Gap.