3GPP TSG-RAN WG4 Meeting #115	R4-2507221

Saint Julian’s, Malta, May 19th –May 23rd, 2025

Source:	Nokia

Title:	Discussion on Fast SCell with EMR core part

Agenda item:	7.18.4

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

## Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR ### Issue 2-1-1: Whether to define new UE capability Further discuss this issue. ### Issue 2-1-2: Update requirements of unknown PUCCH SCell activation based on EMR Directly treat the CR (R4-2503864) online. No agreement is needed.

Discussion

Whether to define new UE capability

### Issue 2-1-1: Whether to define new UE capability

In the last meeting it was not concluded whether there is a need for new UE capability or not. There are already two capabilities defined for this feature and the main benefits of those capabilities are coming from supporting this fast SCell activation with EMR. Hence, the rel-18 capabilities are enough.

No need to define a new capability for Rel-19 feature. Use existing capabilities from rel-18 work item.

PUCCH SCell activation

### Issue 2-1-2: Update requirements of unknown PUCCH SCell activation based on EMR

In the last meeting companies were bringing up PUCCH SCell activation scenario. We are ok to support this but there are some clarifications we would like to discuss. The changes proposed in our CR R4-2507221 is made on top of R4-2503864.

If the target SCell is known and following conditions are met: - UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or, - UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or, - UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode, the pathloss reference signal is known for unknown PUCCH SCell during activation if the following conditions are met during the period between the last transmission of the RS resource used for EMR report and the completion of PUCCH SCell activation, where the RS resource is the target pathloss reference signal or QCLed (with Type D) to the target pathloss reference signal. - The target pathloss reference signal determination is based on the EMR reporting in 4.7.3 and 5.8.3. - The target pathloss reference signal remains detectable during the PUCCH SCell activation period - SNR of the target pathloss reference signal≥-3dB - The associated SSBs reported in EMR report with the target pathloss reference signal remain detectable during the PUCCH SCell activation period - SNR of the associated SSB reported in EMR report ≥-3dB

From the reporting, we think that it should be clear that EMR report is used in this particular case.

Clarify in PUCCH based SCell activation that EMR report is used

Another issues to discuss is whether the time condition of 1280ms is sufficient for EMR reporting and whether it can be guaranteed. We are ok also remove the 1280ms time condition from this case.

Discuss whether 1280ms time condition on Scell activation command to RS is useful in PUCCH SCell activation.

RAN4 should also briefly discuss whether the CSI-RS based PL-RS is supported with EMR or whether PL-RS is assumed to be SSB based.

Discuss whether CSI-RS based PL-RS is supported with PUCCH SCell activation.

Conclusion

The following Observations and Proposals were made:

Proposal 1: No need to define a new capability for Rel-19 feature. Use existing capabilities from rel-18 work item.

Proposal 2: Clarify in PUCCH based SCell activation that EMR report is used

Proposal 3: Discuss whether 1280ms time condition on Scell activation command to RS is useful in PUCCH SCell activation.

Proposal 4: Discuss whether CSI-RS based PL-RS is supported with PUCCH SCell activation.