3GPP TSG-RAN Working Group 4 Meeting #115	 R4-2507226

St Julien, Malta, 19th – 23rd May 2025

Source:	Ericsson

Title:	A-MPR regions for NS_05/NS_05U

Agenda item:	5.26.1

Document for:	Approval

# Background

From last meeting, it was an agreement to evaluate to split the agreed Region A2 into two new regions [1]. In this contribution we consider the A-MPR regions, presenting A-MPR results, we make a proposal of that Region A2 is split into Region A2 and Region A4.

Agreement: - Encourage companies to evaluate the feasibility of the following: - Split the agreed Region A2 into two new regions

The current agreement for the A-MPR regions for NS_05U in Table 1 and for NS_05 in Table 2.

Table 1: A-MPR regions for NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBend |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 2.52MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | ≤ 1.35*RBstart + 2.52MHz/12/scs ≥ 2.34MHz/12/scs | ≥ 8.1MHz/12/scs |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ 1.2*RBstart + 3.6MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs | ≤ 1.2*RBstart + 3.6MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 12.24MHz/12/scs |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 1.45*RBstart + 2.16MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs | ≤ 1.45*RBstart + 2.16MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 12.24MHz/12/scs |
| 20 | < 1945.7 | ≥ 1.5*RBstart + 4.14MHz/12/scs | ≥ 0 | ≤ 5.76MHz/12/scs ≤ RBstart*1.5 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs | ≤ 1.5*RBstart + 4.14MHz/12/scs ≥ 2.88MHz/12/scs | ≥ 16.2MHz/12/scs |

Table 2: A-MPR regions for NS_05 (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35* RBstart + 4.32MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart *1.2 + 4.68MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart *1.5 + 4.14MHz/12/scs; | ≥ 0 | ≤ 5.76MHz/12/scs ≤ RBstart *1.5 + 4.14MHz/12/scs | ≤ 4.68MHz/12/scs |

# Discussion: A-MPR for NS_05 and NS_05U

Proposed A-MPR values will be presented in this contribution. Nevertheless, we make the following observation, where the intermodulation of the 3rd order may cause the spectrum requirement to not be fulfilled without additional power reduction.

Observation 1: for RB allocations starting in the Region A2 with ≤ 4.68 MHz/12/SCS at the lower carrier edge, the required A-MPR is reduced when the RBstart is such that the IMD3 falls in the -25 dBm/MHz range below 1906.6 MHz as compared to that needed when the IMD3 falls in the -30 dBm/MHz range.

## Improve Coverage by splitting Region A2

The current agreement, the region A2 (for Bandwidth = 20MHz and SCS = 15kHz) is at RBstart ≤ 26 (4.68MHz), which implies a required PRB offset of 28 PRB from the lower carrier edge to avoid A-MPR. Based on observation 1, our proposed solution would be to split Region A2 into two regions. Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A4 to be between 10 (1.80MHz) < RBstart ≤ 26 (4.68MHz).

Proposal 1: Split the agreed Region A2 into two new regions: Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A4 to be between 10 (1.80MHz) < RBstart ≤ 26 (4.68 MHz).

This would enable to place the common PUCCH in region A4 with less backoff than in current Region A2 (and the new proposed Region A2) which will contain a higher Backoff to meet the requirements.

## Simulation requirements and assumptions

The DECT additional Spectrum Emission Mask requirements:

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

The following assumptions for the simulations have been applied for the last 10 years:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -28dBc

- Image: -28dB

- CIM3: -60dBc

- CIM5: -70dBc

We would like to highlight an observation of improved simulation assumption and reiterate our argument, an IQ image rejection exceeding 34dB is needed to meet EVM requirements for 256QAM and some of the UL non-contiguous CA emissions requirements are impossible to meet unless an LO leakage better that 28dBc is achieved.

In the last meeting, it was pointed out, the carrier leakage = -34dBc was too much improvement. We have run a one DFT-s-OFDM QPSK, Bandwidth = 20MHz simulation case with the assumption of Carrier Leakage = -30dBc and IQ Image rejection = -34dB.

This improvement of simulation assumption of would have reduced A-MPR needed in Region A2 and Region A3 and the size of the now agreed Regions.

- Carrier Leakage: -30dBc

- IQ Image rejection: -34dB

Figure 1: Simulation results of DFT-s-OFDM QPSK 20MHz 15kHz 106RBs fc = 1930MHz for PC3 in Band n1 NS_05U with Carrier Leakage = -30dBc and IQ Image rejection = -34dB would shrink the Region A2’s size as well as reduce the A-MPR Backoff value, which would improve coverage for the UE.

Observation 2: It can be observed that improved simulation assumptions (Carrier Leakage = -30dBc and IQ Image rejection = -34dB) would shrink the Region A2’s size as well as reduce the A-MPR Backoff value, which would improve coverage for the UE.

Nevertheless, based on the simulation assumption to be kept as Carrier Leakage = -28dBc and IQ Image = -28dB, we have simulated the Spectrum Emission Mask requirements to protect the DECT technology. Below are our simulation results.

## Simulation Results

Figure 2: Simulation results for DFT-s-OFDM QPSK CBW = 20MHz SCS = 15kHz fc = 1930MHz. Here is our split of Region A2 to Region A2 and Region A4 for CBW = 20MHz.

More simulation results are in the Annex below.

Table 3 shows the modification we would like to see to the regions, where Region A4 is proposed.

Table 3: A-MPR regions for NS_05

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBend |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 2.52MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ 1.2*RBstart + 3.6MHz/12/scs | ≥ 0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs | N/A | N/A |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 1.45*RBstart + 2.16MHz/12/scs | ≥ 0 | ≤ 2.16MHz/12/scs | ≤ 1.62MHz/12/scs | ≤ 2.16MHz/12/scs | > 1.62MHz/12/scs ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ 1.5*RBstart + 4.14MHz/12/scs | ≥ 0 | ≤ 5.76MHz/12/scs ≤ RBstart*1.5 + 4.14MHz/12/scs | ≤ 1.8MHz/12/scs | ≤ 5.76MHz/12/scs | > 1.8MHz/12/scs ≤ 4.68MHz/12/scs |

Our proposal of Regions for NS_05U.

Table 4: A-MPR regions for NS_05U

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  | Region A3 |  | Region A4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart | LCRB | RBend | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 2.52MHz /12/scs | ≥ 0 | ≤ 2 .16MHz /12/scs | ≤ 1.8MHz /12/scs | ≤ 1.35*RBstart + 2.52MHz /12/scs ≥ 2.34MHz /12/scs | ≥ 8.1MHz /12/scs | N/A | N/A |
| 15 | 1927.5 ≤ fc <1932.5 | ≥ 1.2*RBstart + 3.6MHz /12/scs | ≥ 0 | ≤ 3.96MHz /12/scs | ≤ 3.6MHz /12/scs | ≤ 1.2*RBstart + 3.6MHz /12/scs ≥ 2.88MHz /12/scs | ≥ 12.24MHz /12/scs | N/A | N/A |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 1.45*RBstart + 2.16MHz /12/scs | ≥ 0 | ≤ 2.16MHz /12/scs | ≤ 1.62MHz /12/scs | ≤ 1.45*RBstart + 2.16MHz /12/scs ≥ 2.88MHz /12/scs | ≥ 12.24MHz /12/scs | ≤ 2.16MHz /12/scs | > 1.62MHz /12/scs ≤ 2.16MHz /12/scs |
| 20 | < 1945.7 | ≥ 1.5*RBstart + 4.14MHz /12/scs | ≥ 0 | ≤ 5.76MHz /12/scs ≤ RBstart*1.5 + 4.14MHz/12/scs | ≤ 1.8MHz /12/scs | ≤ 1.5*RBstart + 4.14MHz /12/scs ≥ 2.88MHz /12/scs | ≥ 16.2MHz /12/scs | ≤ 5.76MHz /12/scs | > 1.8MHz /12/scs ≤ 4.68MHz /12/scs |

Table 5: A-MPR for NS_05U

| Modulation/ Waveform |  | 5M | 10MHz @1925MHz |  |  |  | 15MHz @1927.5MHz |  |  |  | 15MHz @1932.5 |  |  |  | 20MHz @1930 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A1 | A2 | A3 | A4 | A1 | A2 | A3 | A4 | A1 | A2 | A3 | A4 | A1 | A2 | A3 | A4 |
| DFT-s-OFDM | Pi/2 BPSK | N/A | ≤ 3.5 | ≤ 2.0 | ≤ 2.0 | N/A | ≤ 4.0 | ≤ 2.5 | ≤ 2.0 | N/A | ≤ 2.0 | ≤ 2.5 | ≤ 1.5 | ≤ 1.0 | ≤ 4.0 | ≤ 3.5 | ≤ 2.0 | ≤ 2.0 |
|  | QPSK |  | ≤ 3.5 | ≤ 2.0 | ≤ 2.0 | N/A | ≤ 4.0 | ≤ 2.5 | ≤ 2.0 | N/A | ≤ 2.5 | ≤ 2.5 | ≤ 1.5 | ≤ 1.0 | ≤ 4.0 | ≤ 4.0 | ≤ 2.0 | ≤ 2.0 |
|  | 16QAM |  | ≤ 3.5 | ≤ 2.5 | ≤ 2.5 | N/A | ≤ 4.0 | ≤ 3.0 | ≤ 2.5 | N/A | ≤ 3.0 | ≤ 3.0 | ≤ 2.5 | ≤ 1.0 | ≤ 4.0 | ≤ 4.0 | ≤ 2.0 | ≤ 2.0 |
|  | 64QAM |  | ≤ 4.0 | ≤ 3.0 | ≤ 2.5 | N/A | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 | N/A | ≤ 3.0 | ≤ 3.0 | ≤ 2.5 | ≤ 1.0 | ≤ 4.0 | ≤ 4.5 | ≤ 2.0 | ≤ 2.5 |
|  | 256QAM |  | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ |
| CP-OFDM | QPSK |  | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ |
|  | 16QAM |  | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ |
|  | 64QAM |  | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ |
|  | 256QAM |  | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | N/A | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ | ≤ |

As suspected, the approved Region A2 is sub-optimal for bandwidth = 20MHz. One thing is clear and that is by splitting the Region A2 in two Regions, where we call the new regions Region A2 and Region A4, coverage for Region A4’s allocations are improved. Although we had hopped of avoiding A-MPR common PUCCH completely, it is possible to reduce the required PRB offset to 10 PRB using the new Region split. The Split can be done both for NS_05U and for NS_05. It is most beneficial for Bandwidth = 20MHz

Proposal 2: Split Region A2 both for NS_05U and for NS_05. In this paper, the new regions are referred to as Region A2 and Region A4 in both NS tables.

For NS_05U, there are a few RB allocations which will be affected by the NS_100 when adjacent UTRA is next to the band. Therefore, the table for A-MPR for NS_100 shall be invoked for all outer allocations when the A-MPR would have otherwise been smaller.

Proposal 3: Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) require some minimum backoff for outer RB allocation when the NS_05U is signalled.

Table 6: Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2)

# Optionality in earlier releases

It has been agreed to introduce the updated emission limits and associated A-MPR from Rel-19 [3]. The modified MPR behaviour can be used for allowing UEs of earlier releases indicate support of the reduced A-MPR and make the network aware of this, at least for UEs compliant with Rel-16 and later.

Rel-19 UEs shall be compliant with the Rel-19 version of the 38.101-1. In other words, it is mandatory for Rel-19 UEs to be compliant with the Rel-19 and the bit shall be set:

whereas UEs compliant with “this version of the specification” earlier than Rel-19, may set the bit to indicate support of the lower A-MPR

in accordance with the original intention of the modified MPR behaviour to maintain backwards compatibility. This means that the bitmap for n1 would always be included by UEs compliant with Rel-19 or later.

Proposal 4: For indication of modified MPR behaviour for NS_05/NS_05U, UEs compliant with Rel-19 or later shall set the relevant bit to “1” and thus always include the bitmap for band n1 such that backwards compatibility for modified MPR indication is maintained.

A version of the specification should not specify/define behaviour of a UE compliant with a version of another release. The modified MPR behaviour, or the release independence for that matter, should not do this either.

# Conclusion

In this contribution, we shared our views for updated Band n1 requirements, which will lead to change of Network signalling value NS_05 and NS_05U. Observations and Proposals are summarized below:

Observation 1: for RB allocations starting in the Region A2 with ≤ 4.68 MHz/12/SCS at the lower carrier edge, the required A-MPR is reduced when the RBstart is such that the IMD3 falls in the -25 dBm/MHz range below 1906.6 MHz as compared to that needed when the IMD3 falls in the -30 dBm/MHz range.

Observation 2: It can be observed that improved simulation assumptions (Carrier Leakage = -30dBc and IQ Image rejection = -34dB) would shrink the Region A2’s size as well as reduce the A-MPR Backoff value, which would improve coverage for the UE.

Proposal 1: Split the agreed Region A2 into two new regions: Region A2 to end at RBstart ≤ 10 (1.80MHz) and Region A4 to be between 10 (1.80MHz) < RBstart ≤ 26 (4.68 MHz).

Proposal 2: Split Region A2 both for NS_05U and for NS_05. In this paper, the new regions are referred to as Region A2 and Region A4 in both NS tables.

Proposal 3: Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) require some minimum backoff for outer RB allocation when the NS_05U is signalled.

Proposal 4: For indication of modified MPR behaviour for NS_05/NS_05U, UEs compliant with Rel-19 or later shall set the relevant bit to “1” and thus always include the bitmap for band n1 such that backwards compatibility for modified MPR indication is maintained.

# References

[1] R4-2505222, “WF on PHS protection update”, KDDI

[2] R4-2500231, “(NR_newRAT-Core) Further study on NR A-MPR reevaluation scenarios on n1 for Japan”, KDDI

[3] R4-2420481, “WF on PHS protection update”, KDDI, OPPO

# Annex