3GPP TSG-RAN WG4 Meeting #115                                                                    R4-2507235

Malta, 19th – 23rd May, 2025

Agenda item:         7.22.2

Title:	Comprehensive Simulation Analysis on Dataset Alignment, Model Interoperability, and Fine-tuning Impacts

WI/SI:	FS_NR_AIML_air_Ph2

Release:	Rel-19

Source:	Korea Testing Laboratory

Document for:	Discussion

# 1. Introduction

This contribution provides detailed simulation results and technical analyses focused on key factors influencing CSI compression performance, specifically addressing dataset alignment, interoperability among encoder-decoder pairs, and benefits of two-side fine-tuning.

Comprehensive simulations demonstrate that dataset alignment significantly impacts SGCS performance and interoperability. Fully aligned mixed datasets consistently yield robust and stable encoder-decoder performance, whereas utilizing company-specific datasets frequently leads to notable performance degradation due to inherent statistical variations and differences in dataset distributions. Additionally, some originally provided models exhibited significant performance issues, likely resulting from incompleteness or insufficient initial training.

This contribution explicitly analyzes key factors influencing CSI compression, including the importance of dataset alignment, interoperability issues among encoder-decoder pairs, and the effectiveness of two-side fine-tuning. Considering practical deployment constraints, alternative solutions currently agreed by RAN1, namely Direction A (offline dataset and parameter exchange) and Direction B (encoder parameter sharing), are introduced as realistic options.

2. Discussion

2.1 Dataset Alignment and Technical Impact

The ongoing standardization discussions within RAN4 for Option 3 Track 2 and Option 4 aim to establish a standardized interface between network-side (NW) and user equipment (UE)-side AI/ML CSI compression models. This standardization is intended to ensure maximum inter-vendor interoperability, allowing vendors to independently implement CSI compression models while maintaining robust and stable performance. The simulation results presented in this contribution explicitly highlight dataset alignment as an essential technical requirement for achieving these RAN4 standardization objectives. By emphasizing the importance of dataset alignment, this contribution aims to provide clear technical evidence to inform and further advance the ongoing RAN4 discussions on AI/ML-based CSI compression.

The provided simulation results clearly underscore the significant impact of dataset alignment on SGCS performance and interoperability across different encoder-decoder pairs. When encoder-decoder pairs are trained and tested using fully aligned mixed datasets, robust and consistent performance is consistently achieved. Conversely, employing company-specific datasets frequently leads to notable performance degradation, primarily caused by statistical variations and inherent differences in data distribution between individual datasets. This negatively affects model generalization capabilities and interoperability, posing practical challenges for real-world deployments.

Furthermore, the substantial performance degradation observed in some originally provided models suggests incomplete or insufficient initial training procedures, reinforcing the necessity of high-quality initial model training in conjunction with dataset alignment. As demonstrated by the two-side fine-tuning results, simultaneous fine-tuning of encoder and decoder models substantially improves SGCS values, highlighting the critical technical importance of dataset alignment. However, practical limitations under current Rel-19 interface specifications preclude simultaneous fine-tuning in actual field deployments. Thus, two-side fine-tuning serves primarily as a technical reference emphasizing dataset alignment and model interoperability, rather than as a direct implementation recommendation for the Rel-19 standard.

To address practical limitations identified with two-sided fine-tuning, RAN1 discussions have proposed alternative methodologies, specifically Direction A (offline engineering with encoder parameters and datasets exchange) and Direction B (encoder parameter exchange for direct UE-side inference). Direction A includes exchanging datasets ({target CSI, CSI feedback}) or encoder parameters along with target CSI information, enabling the UE to perform offline training and optimization independently. Direction B involves sharing NW-side encoder parameters directly with the UE for on-device inference without additional offline engineering. Both approaches provide practical and feasible alternatives to two-side fine-tuning, offering pathways toward enhanced interoperability and improved performance while aligning with current RAN1 agreements and specifications.

2.2 Interoperability among Encoder-Decoder Pairs

The simulation outcomes clearly highlight interoperability as a crucial technical requirement. Cross-testing of encoder-decoder pairs trained on fully aligned mixed datasets consistently yielded stable and robust SGCS performance. However, significant interoperability issues and performance instabilities arose when testing encoders trained exclusively with company-specific datasets. This deterioration originates primarily from dataset misalignment and is exacerbated by the previously identified insufficient initial model training.

Given these observations, ensuring dataset alignment across vendors is essential not only for performance enhancement but also as a fundamental technical precondition to achieving practical interoperability in multi-vendor network environments, as consistently emphasized by ongoing RAN4 and RAN1 discussions.

2.3 Benefits and Technical Implications of Two-side Fine-tuning

Simulation results demonstrate that two-side fine-tuning effectively mitigates performance degradation caused by dataset misalignment and initial training deficiencies. By concurrently optimizing both encoder and decoder with a well-aligned shared dataset, substantial improvements in model robustness and consistency were achieved.

However, recognizing current Rel-19 scope limitations for practical deployments, simultaneous fine-tuning approaches are impractical in live operational environments. Nevertheless, these simulation results underscore the essential role of joint optimization methodologies and dataset alignment for interoperability. Therefore, future standardization efforts, particularly those discussed within RAN1, should consider joint optimization approaches for reference benchmarking or offline engineering practices to ensure encoder-decoder compatibility and optimal performance.

In practical Rel-19 deployments, alternative approaches currently under discussion by RAN1—such as Direction A (dataset and encoder parameter exchange for offline training) and Direction B (NW-side encoder parameter sharing for direct on-device inference)—can effectively substitute two-side fine-tuning. Direction A allows vendors to exchange training datasets and model parameters, thereby facilitating independent offline optimization. Direction B enables efficient on-device inference using shared encoder parameters, minimizing additional training overhead and complexity. These alternatives align with ongoing RAN1 agreements and represent viable pathways to addressing the interoperability challenges identified in this study.

[RAN1 #118] Agreement For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary. Agreement Continue the study of following directions - on-device operation and UE side offline engineering - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues - [Issue 3] Is there an overhead concern, and if so, how to address it? - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs? - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it? - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

3. Result for comparison of testing on two-sided fine-tuning model and original provided model

- Result for comparison of testing on two-side fine-tuning model and original provided model

- Dataset: Mixed dataset from ‘MIXEDR4_114dsei00.zip’ (total 500,000 samples)

  - Retraining dataset: First 70% (samples 1 to 350,000)

  - Test dataset: Last 10% (samples 450,000 to 500,000)

- Evaluation Results Provided:

  - Testing on each company’s original provided encoder-decoder pair (quantized track2).

  - Testing on two-side retrained encoder-decoder pairs.

- Two-side fine-tuning Methodology:

  - Input retraining dataset to company’s original encoder.

  - Pass encoder output to company’s original decoder and calculate SGCS.

  - Loss function: 1 – SGCS

  - Key parameters:

    - Initial learning rate: 0.0001

    - Training epochs: 50

    - Batch size: 256

  - The same methodology is uniformly applied to all evaluated companies.

Table 1. SGCS performance comparison: original vs. two-side fine-tuning models

| Category | Reference | Reference SGCS value | Testing on given model | Testing on two-side fine-tuning model |
| --- | --- | --- | --- | --- |
| VIVO | R4-2418748 | 0.732 | 0.0447 | 0.69 |
| Ericsson | R4-2416314 | 0.6977 | 0.0058 | 0.57 |
| Samsung | R4-2411636 | 0.76 | 0.5374 | 0.569 |
| MediaTek | R4-2411711 | 0.722 | 0.5189 | 0.556 |
| CATT | R4-2411343 | 0.75 | 0.5517 | 0.551 |

In particular, notably low SGCS performance values were observed in the testing of the original provided models from Ericsson (0.0058) and VIVO (0.0447). Such significant performance degradation is likely attributable to either incomplete or insufficient initial training of these provided models, or substantial deviations of the datasets and model initialization conditions from the RAN4 recommended baseline conditions using the mixed dataset. This highlights the critical need to ensure adequate initial model training and strict adherence to aligned dataset conditions as defined by RAN4 guidelines.

Observation 1: The significantly lower SGCS values observed from the provided original models are primarily attributed to incomplete initial training or inadequate generalization resulting from dataset misalignment. This clearly emphasizes the necessity of ensuring high-quality initial datasets and proper training procedures to achieve effective interoperability among vendor models.

Observation 2: Applying two-side fine-tuning substantially improved the SGCS values across all evaluated models, demonstrating that joint optimization of encoder-decoder pairs significantly mitigates interoperability and dataset alignment issues. However, due to practical constraints under the current Rel-19 specifications, these results should be regarded strictly as a technical reference rather than a direct implementation recommendation.

Observation 3: Considering practical constraints for field implementation, future standardization activities within RAN4 and RAN1 should prioritize viable alternatives to two-side fine-tuning, specifically Direction A (offline dataset and parameter exchange) or Direction B (encoder parameter sharing). It is essential that upcoming standardization clearly defines practical interfaces enabling individual vendors to independently optimize their models while maintaining interoperability across different encoder-decoder pairs.

4. Conclusion

In this contribution, we provided our viewpoints.

Observation 1: The significantly lower SGCS values observed from the provided original models are primarily attributed to incomplete initial training or inadequate generalization resulting from dataset misalignment. This clearly emphasizes the necessity of ensuring high-quality initial datasets and proper training procedures to achieve effective interoperability among vendor models.

Observation 2: Applying two-side fine-tuning substantially improved the SGCS values across all evaluated models, demonstrating that joint optimization of encoder-decoder pairs significantly mitigates interoperability and dataset alignment issues. However, due to practical constraints under the current Rel-19 specifications, these results should be regarded strictly as a technical reference rather than a direct implementation recommendation.

Observation 3: Considering practical constraints for field implementation, future standardization activities within RAN4 and RAN1 should prioritize viable alternatives to two-side fine-tuning, specifically Direction A (offline dataset and parameter exchange) or Direction B (encoder parameter sharing). It is essential that upcoming standardization clearly defines practical interfaces enabling individual vendors to independently optimize their models while maintaining interoperability across different encoder-decoder pairs.

5. Reference

- 3GPP TSG RAN WG4 Meeting #115, "WF on AI/ML air interface study," TDoc R4-2505107, April 2025.

6. Contact Information

| Name | Affiliation | Email |
| --- | --- | --- |
| Sang-Woon Jeon | Hanyang University | sangwoonjeon@hanyang.ac.kr |
| Kangyu Gao | Hanyang University | gaokangyu@hanyang.ac.kr |
| Zhao-Kun Shao | Hanyang University | shaozhaokun@hanyang.ac.kr |
| Jae-suk Lee | Korea Testing Laboratory | jslee@ktl.re.kr |
| HG Song Raphael | Korea Testing Laboratory | hgsong@ktl.re.kr |