3GPP TSG-RAN WG4 Meeting #115	R4-257247

St Julian’s, Malta, May 19th –May 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 with updates to OTA spatial emission requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is draft CR to TS 38.141-2 with updates to Expected EIRP mask. This draft CRs is based on endosred during RAN4#114bis meeting draftCR R4-2504765. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 3.3: introduction “Above the Horizon” abbreviation Clause 4.1.1: replace “EEIRP” to “AtH” Clause 4.9.3.2: editorial correction Clause 6.9.4.2: editorial correciton, additon of text on mechanical tilt from TR 38.908. Clause 6.9.5.1: clarifications added Clause Annex N.2.1: editorial correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will include errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1.1, 4.9.3.2, 6.9.4.2, 6.9.5.1, Annex N.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

ATG	Air-to-Ground

AtH	Above the Horizon

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CATR	Compact Antenna Test Range

CPE	Common Phase Error

CP-OFDM	Cyclic Prefix-OFDM

CLTA	Co-Location Test Antenna

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

EUT	Equipment Under Test

EIRP	Equivalent Isotropic Radiated Power

EEIRP	Expected EIRP

EIS	Equivalent Isotropic Sensitivity

FBW	Fractional Bandwidth

FSS	Fixed Satellite Service

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

LNA	Low Noise Amplifier

MR	Medium Range

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

OCC	Orthogonal Covering Code

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

PT-RS	Phase Tracking Reference Signal

PWS	Plane Wave Synthesizer

RB	Resource Block

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RS	Reference Signal

RV	Redundancy Version

RX	Receiver

RoAoA	Range of Angles of Arrival

SCS	Sub-Carrier Spacing

SSB	Synchronization Signal Block

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDD	Time Division Duplex

TDL	Tapped Delay Line

TRP	Total Radiated Power

TT	Test Tolerance

UCI	Uplink Control Information

ZF	Zero Forcing

# 4	General radiated test conditions and declarations

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in TS 38.141-2 (the present document), i.e. to all radiated tests defined in FR1 for BS type 1-H, BS type 1-O and radiated tests defined in FR2 for BS type 2-O. The frequency ranges FR1 and FR2 are defined in clause 5.1 of TS 38.104 [2].

The minimum requirements are given in TS 38.104 [2]. Test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

The requirements are classified according to spatial characteristics as shown in table 4.1.1-1 and table 4.1.1-2.

Table 4.1.1-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FR1 | FR2 | conformance directions |
| Radiated transmit power |  | Directional | OTA peak directions set | OTA peak directions set | 5 |
| OTA BS output power |  | TRP | See annex I |  |  |
| OTA output power dynamics |  | Directional | OTA peak directions set | OTA peak directions set | 1 |
| OTA transmitter OFF power |  | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA transient period |  | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  |  | Directional | N/A | OTA peak directions set (Note 2) | 1 |
| OTA modulation quality |  | Directional | OTA coverage range | OTA coverage range | 5 |
| OTA frequency error |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA time alignment error |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA occupied bandwidth |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA ACLR |  | TRP | N/A | N/A | See annex I |
| OTA operating band unwanted emission |  | TRP | N/A | N/A | See annex I |
| OTA transmitter spurious emission | General requirement | TRP | N/A | N/A | See annex I |
|  | Protection of the BS receiver of own or different BS | Co-location | See clause 4.12 | N/A | See clause 4.12 |
|  | Additional spurious emissions | TRP | N/A | N/A | See annex I |
|  | Co-location with other base stations | Co-location | See clause 4.12 | N/A | See clause 4.12 |
| OTA transmitter intermodulation |  | Co-location | See clause 4.12 | N/A | See clause 4.12 |
| OTA spatial emission |  | AtH | See clause 6.9.5 | N/A | See clause 4.9.3 |
| NOTE 1: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. NOTE 2: For FR2, RF Core requirements are defined on TRP levels. Conformance requirements are verified by EIRP measurements in the reference direction. |  |  |  |  |  |

<Next of the change>

<Next of the change>

### 4.9.3	Test beam direction set

#### 4.9.3.1    General

Conformance testing of OTA spatial emission requirement is based on a test beam direction set.

#### 4.9.3.2    Test beam direction set for protection of FSS UL

Test beam direction set is utilized for conformance testing of requirements in clause 6.9.5.1 for BS type 1-H and BS type 1-O operating in band n104. The test beam directions are defined in table 4.9.3.2-1.

Table 4.9.3.2-1: Test beam direction set for protection of FSS UL

| Beam index | Elevation angle | Azimuth angle |
| --- | --- | --- |
| 1 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 |
| 2 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 |
| 3 | Θmin+2.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 |
| 4 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 |
| 5 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 |
| 6 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 |
| 7 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 |
| 8 | Θmin+7.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 |
| 9 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 |
| 10 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 |
| 11 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 |
| 12 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 |
| 13 | Θmin+12.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 |
| 14 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+2.5(Φmax-Φmin)/25 |
| 15 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 |
| 16 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 |
| 17 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 |
| 18 | Θmin+17.5(Θmax-Θmin)/25 | Φmin+22.5(Φmax-Φmin)/25 |
| 19 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+7.5(Φmax-Φmin)/25 |
| 20 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+12.5(Φmax-Φmin)/25 |
| 21 | Θmin+22.5(Θmax-Θmin)/25 | Φmin+17.5(Φmax-Φmin)/25 |
| NOTE 1: Θmin, Θmax, Φmax, Φmin is defined in the declaration D.10 with the associated declaration D.64. |  |  |

Figure 4.9.3.2-1 Test beam set for conformance testing (to be added)

----------next change---------

  - 6.9	OTA spatial emission

### 6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.9.2	Minimum requirement

The minimum requirement for BS type 1-H and BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

### 6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

### 6.9.4	Method of test

#### 6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level with the corresponding BW.

In the test range, the EUT is mounted at the positioner with mechanical down-tilt set to zero.

#### 6.9.4.2	Procedure

When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

The test range shall be calibrated according to calibration method described in TR 37.941, clause 8.3.

1. Place the BS at the positioner, such that blocking effect in the vertical domain is minimized.

2. Align the manufacturer declared coordinate system orientation (D.2) of the BS with the coordinate system used by the test system.

3. The measurement device characteristics shall be: Detection mode set to True RMS.

4. Set the BS to transmit on both polarizations according to applicable test configuration in clause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1 [3], clause 4.9.2.

5. Orient the positioner (and the BS) to the angle for measurement point (θn,φm) for m=1..M and n=1.. N. Spatial sampling grid is defined in Annex N.3.

6. Configure test beam(s) and test equipment for measuring test beam k for k=1..K, as defined in clause 4.9.3.2.

7. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

8. Repeat step 6 and 7 for all K test beams.

9. Repeat step 5, 6, 7 and 8 for all M and N positioner angles.

10. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for bins i=1…7 as described in Annex N.

In the test procedure above the order of Step 8 and Step 9 may be exchangeable.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the testing shall be repeated for the following two instances:

-	Minimum mechanical down tilt and the reduced electrical steering range

-	Maximum electrical steering range and increased mechanical down tilt

### 6.9.5	Test requirements

#### 6.9.5.1	Requirement for BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, the power measured EEIRP shall not exceed the limits specified in table 6.9.5.1-1 for mechanical down-tilt angle equal to zero.

Note. OTA spatial emissions in table 6.9.5.1-1 are defined with respect to the horizon with angles above the horizon positive and below the horizon negative The reference BS coordinate system defined in sub-clause 4.14 system is defined with respect to the BS enclosure with angles above the boresight negative and below the boresight positive. If the BS is aligned with the horizon (mechanical tilt is zero) then the coordinate systems θ sign is inverted.

If mechanical tilt is applied then coordinates are translated as described in Annex N2.

In case the EEIRP measurement is repeated for non-zero mechanical tilt and mechanical tilt of the BS is arranged at the positioner with respect to horizon, then θ sign is inverted and other coordinate translation in Annex N2 is not needed.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<<30 | 16 |
| 6 | 30≤<<60 | 15 |
| 7 | 60≤<<90 | 15 |
| Note 1: The requirement shall apply to all supported mechanical tilts. |  |  |

Figure 6.9.5.1-1: Definitions of θHLi and θHHi angles.

<Next change>

# Annex N (normative):

# Expected EIRP calculation and spatial sampling grid

# N.1	General

The Expected EIRP calculation is based on two averaging processes:

- Averaging over the test beam directions in clause.

- Averaging over horizontal and vertical angles within specified vertical angles bins

The required sampling resolution for the EIRP measurement per test beam direction is described in Annex N.3

# N.2	Averaging processes

This clause describes the details of the calculation of the EEIRP from the measured EIRP samples.

    - N.2.1 Averaging over beamforming directions

The first averaging process involves averaging over different test beam directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθ,φ=1Kk=1KEIRPk(θ,φ)

The power measured is the sum of the EIRP in both polarisations.

The power measured is the sum of the EIRP in both polarisations. The EIRPθ,φ is the average EIRPk(θ,φ) over the K number of test beam directions for the angle (θ,φ).

For the case where non-zero mechanical tilt is considered the reference coordinate system in clause 4.14 in this document needs to be converted to the global coordinate system used by the test requirement (Figure 6.9.5.1-1). The angles in the reference coordinate system can be translated to the global coordinate system as:

, where θ is the vertical angle starting from antenna aperture plane and φ is the horizontal angle defined for reference coordinate system in clause 4.14.

θH is the vertical angle and φHis the horizontal angle in the global coordinate system, defined with respect to the horizon in figure 6.9.5.1-1.

θMT is the mechanical down-tilt angle.

    - N.2.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This EEIRP for a spherical strip is therefore:

EEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn

, where

θHLi , θHHi	is the elevation angular range bounded by θHLi and θHHi in degrees, e.g. for bin 1, this will be 0⁰ and 5⁰ respectively.

θn	is the corresponding angle of the nth midpoint between the elevation intervals, referenced from the horizon. Midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°.

φm	is the corresponding angle of the mth midpoint between the azimuth intervals

nL	is the lowest elevation index of sampling points within the θHLi , θHHi range

nH	is the highest elevation index of sampling points within the θHLi , θHHi range

M	is the number of azimuth points within the (-180°, 180°) range

N	is the number of elevation points within the θHLi , θHHi range

Note: All angles are in degrees.

    - N.3 Spatial sampling grid

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD

, where  is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=w2+h2

, where w is the array antenna width, and h is the array antenna length.

<End of draft CR>